SA WG2 Meeting #170	S2-2507408

25 – 29 Aug, 2025, Goteborg, SE

Source:	Apple

Title:	[WT#1.4, Voice] Native Voice in 6G

Document for:	Agreement

Agenda Item:	20.6.1.4

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution provides updates to the scoping of WT#1.4 for support of voice and emergency services in 6G.

# 1   Justifications

The 6G SA2 SID in SP-250806 [x2] approved at TSG SA #108 contains the following work task corresponding to the support of voice and emergency services:

1.4.	Study whether and how to support and/or enhance the essential/regulatory services (i.e. voice, Messaging, location services, Emergency services, MPS, Mission Critical services, PWS) in 6G.

Voice has been supported since the beginning of mobile networks and can be considered one of the most fundamental services. As new generations were defined, eventually any generation contained voice as an essential feature.

Voice services have incorporated many special features over the years. Looking back in history, both 4G and 5G systems offer support for voice through IMS. However, there are several differences between VoLTE and VoNR. For example, LTE had TTI bundling and NR supported TB repetition. 5G NR also has a few special features typically associated with voice, such as UE configurable RLF timers, RoHC, short RLC/PDCP SNs, SPS, RAN Assisted Codec Rate Adaptation (recommended bit rate), delay budget reporting, ECN, etc. The presence of such differences had made it difficult to offer VoNR right from the start in 5G. This was not only because some of the associated sub-features are optional but because interoperability and testing must be ensured. At the same time, VoLTE had to be matched by VoNR to guarantee the same level of voice quality. Additionally, the support of voice over the 5G access network needed extensions in the CN e.g. in NAS, session and IMS signalling for voice call establishment, and emergency service had to be supported over NR/5GC. The CN also had to provide SBI services with equivalent functionality to the Diameter Rx and Cx/Sh reference points, extensions to PCF and BSF, and the N26 interface was impacted.

Along these lines and to provide voice support in initial 5G deployments, 5G includes a RAT fallback (from NR to LTE) and an EPS fallback option (from 5GC to EPC) which is used when VoNR cannot be supported. However, voice fallback has also proven complex to provide. Since voice is a very essential feature, the sourcing company prefers voice (e.g. Vo6G) support from day 1, including emergency sessions and location information for emergency voice.

From a user experience standpoint, the quality of Vo6G must be comparable to VoNR. To support Voice over 6G from Day1 demands voice specific KPIs, for example, with regards to power consumption (DRX), cell edge performance (coverage enhancements), RTT (UL audio scheduling alignment), and more. In other words, there is some RAN impact as 6G is going to have a new air interface.

In summary, Vo6G should be supported in the first 6G release. Reusing IMS-based voice service and corresponding QoS mechanism looks the most reasonable way forward for 6G. With voice support in Day-1 the quality of Vo6G must be comparable to VoNR. The definition of a conditional fallback procedure (e.g. to VoNR) may need to be studied, especially in early deployment phases in case 6G cannot provide sufficient coverage or stable user experience. Ideally the fallback option should be lightweight and applicable under special conditions only.

**** First Change ****

# Annex A.X. WT#1.4 Scope for Voice and Emergency Services

WT#1: Define the overall 6G architecture as collection of capabilities and high-level functionalities considering the following sub-work tasks and other work tasks to support 6G access network:

1.4.	Study how to support voice and emergency services in 6G, including the following aspects:

- How to enable native voice service for 6G

- How to enable emergency voice service and location service for emergency voice service, including establishment of an emergency voice session for UEs in limited service state.

- How to enable IMS-based voice service over 6G and update IMS specifications to support 6G.

- Whether a fallback mechanism to 5GS is needed and if so, how to simplify procedures and under which conditions to support it.

NOTE X:	It is assumed that interworking and service continuity aspects need to be coordinated with WT#2.

NOTE Y:	Coordination with RAN and SA4 will be required.

NOTE Z: The existing IMS architecture is used as a starting point for the study.

**** End of Changes ****