3GPP TSG-WG SA2 Meeting #170	S2-2507411

Goteborg, SE, 25th Aug – 29th Aug, 2025

Source:	HONOR

Title:	New KI for WT#1 Study the support of AIoT services in Topology 2

Document for:	Approval

Agenda Item:	20.5.1

Work Item / Release:	FS_AmbientIoT_Ph2_ARC / Rel-20

Abstract: KI for WT1 on study the support of AIoT services in Topology 2.

1. Introduction

This contribution proposed the Key Issues related to the WT#1 described in SP-250834：

WT#1: Study the support of AIoT services in Topology 2, including: - Study and investigate how to support AIoT services under the RRC-based option for UE Reader connectivity. NOTE 3: Rel-19 TR solution and interim conclusion should be used as basis for rel-20 work. Solution submitted to rel-20 need to align with rel-19 TR interim conclusion. No rel-19 AIoT Device impact from this WT is expected.

According to the SP-250834 approved in SA#108, the Rel-19 TR solutions and interim conclusions are the basis for Rel-20 study. And also, Rel-19 remaining editor’s notes for topology 2, excepting the user-plane architecture related, should be resolved in Rel-20 study phase.

Proposal 1: In the Rel-20 study of WT#1, SA WG2 should draft key issues based on Rel-19 interim conclusions and Rel-19 remaining editor’s notes about topology 2, excepting the user-plane architecture related.

In the TR 23.700-13 v2.0.0, the RRC-based architecture for Topology 2 is shown in clause 8.1.3.3:

Figure 8.1.3.3-1: Protocol Stack for the RRC option

Besides, the following editor’s notes for Topology 2 are shown in the TR 23.700-13 v2.0.0:

- Aggregation related

Editor's note:	For RRC based solution of topology 2, whether the aggregation can be performed by the RAN is FFS and coordination with RAN WGs is needed.

To resolve this editor’s note, it is proposed to study whether the NG-RAN and/or UE reader performs the aggregation based on the assistance information provided by the AIOTF.

- UE authorization related

Editor’s note:	Additional subscription information, e.g. validity information, for the UE Reader is FFS.

Editor’s note:	Whether and how to enable authorization to the UE is FFS.

Editor's note:	How to authorize the UE Reader will be determined later.

In Rel-19, there is no conclusion on whether and how to enable authorization to the UE, and what additional subscription information should be for the UE reader. The UE authorization aspect needs further study.

- AIoT Service procedure related

Editor's note:	How addressing works for UL for Option B is FFS.

Editor's note:	Further details how the RRC option works are FFS.

To resolve the above two editor’s notes, it is necessary to study the AIoT service (i.e., inventory, command) procedure, UE reader selection, and NG-RAN node selection to support AIoT services under the RRC-based option for UE Reader connectivity.

Proposal 2: The above issues should be captured in the KI for WT#1.

2. Text Proposal

It is proposed to capture the following changes in TR 23.700-30.

# * * * * First change * * * *

5.X	Key Issue #1: Study the support of AIoT services in Topology 2

This key issue will study the system architecture enhancement to the support of AIoT services in Topology 2.

NOTE 1:	Rel-19 TR solution and interim conclusion should be used as basis for rel-20 work. Solutions submitted to this key issue need to align with rel-19 TR interim conclusion. No rel-19 AIoT Device impact from this key issue is expected.

The following issues will be studied:

-	Study and investigate how to support AIoT services under the RRC-based option for UE Reader connectivity.

-	Study the procedure to support AIoT services under the RRC-based option for UE Reader connectivity.

-	Study how the AIOTF performs the NG-RAN node selection for DL and how the CN (e.g., AIOTF, AMF) and/or the NG-RAN perform the UE reader selection.

NOTE 2: This issue needs coordination with RAN WGs.

-	Study whether the NG-RAN and/or UE reader performs the aggregation based on the assistance information provided by the AIOTF.

NOTE 3: This issue needs coordination with RAN WGs

- Study how to enable/revoke authorization to a UE reader and identify what the additional UE subscription information is need.

# * * * * End of changes * * * *