SA WG2 Meeting #170	S2-2507415

25 – 29 Aug, 2025, Goteborg , SE

Source:	T-Mobile USA, Nokia

Title:	Work Task for Voice in 6G

Document for:	Agreement

Agenda Item:	20.6.1.4

Work Item / Release:	FS_6G_ARC / R20

Abstract of the contribution: This contribution is intended to provide a breakdown of the Work Task for voice and emergency voice service support in 6G Systems and proposes related Key Issues.

# Discussion

SA plenary approved a study Item Description (SID) to study 6G architecture in SP-250806, the purpose of which it stated, “This study aims to define a system architecture for 6G mobile networks for improvement of existing services and support of new services, to meet the 6G system requirements as defined by 3GPP SA1 and TSG RAN.”  The objectives were further divided into Work Tasks (WT) of which WT#1.4 which stated:

1.4. Study whether and how to support and/or enhance the essential/regulatory services (i.e. voice, Messaging, location services, Emergency services, MPS, Mission Critical services, PWS) in 6G.

This paper focuses on the voice services aspects (including emergency voice) of this WT and aims to define Key Issues and any related Architectural Assumptions and Requirements in the study documentation.

In addition to WT 1.4 described above the SID also defined WT#2 related to migration and interworking, as voice services are expected to interwork and support migration the following is assumed to apply to voice services:

“WT#2: Study migration and interworking, including

-	How to support migration to 6GS

-	How to support interworking with 5GS

-	Whether and how to support interworking with EPS

-	How to support interworking between 6GS and 4G/5G NTN/satellite access that use EPS/5GS.

NOTE 2:	Interworking with 2G/3G are not considered in this study. Interworking between 6GS and 5GS is assumed to work even if the UE has previously registered in 2G, 3G or 4G.

NOTE 3:  The detailed migration study scope will be coordinated and aligned with RAN.”

As well as WTs the objectives of the study included further guiding principles, of which the following may be applicable to voice services:

“The study will work towards goals endorsed at TSG#107(Mar 2025) to "create lean and streamlined standards for 6G, e.g. by dimensioning an appropriate set of functionalities, minimizing the adoption of multiple options for the same functionality, avoiding excessive configurations, etc. Any exception to the above shall be well justified."”

AND

“The conclusion for each work task should:

a.	Identify functionalities, NFs etc. that use 5GC NFs as basis and any enhancements.

b.	Identify functionalities, NFs etc. that need further study and that may be redesigned

c.	Identify new functionalities, e.g. NFs to be added for supporting new features”

## Observations:

Based on the above text from objectives of the 6G study the following observations are made:

1.	Voice and Emergency Services (of which Emergency Voice is a sub-set), are essential services that should be supported in the 6G systems.

2.	Voice services should be supported while a network migrates to 6GS – this implies seamless service continuity from and to the 6G system

3.	Voice Services on the 6GS should support interworking with 5GS, and investigations into supporting interworking with EPC should also be considered

4.	NTN Voice Services on the 6GS should support interworking with 4G/5G NTN/satellite access that use EPS/5GS

5.	Defining voice service support in 6GS should avoid excessive configurations and minimizing the adoption of multiple options

## Aspects for 6G Study

Based on the above observations it is proposed that the following principles be adopted:

a)	IMS be used as the basis for 6G voice re-using the same principles for 4G and 5G IMS voice services.

b)	To minimize options, only one 6G Voice solution will be defined – since voice is an essential service of 6G this implies that voice should defined without reliance on any prior network system (i.e. that voice be “native” to 6G).

c)	NTN Voice Services on the 6GS should be supported.

d)	Mobility and Interworking aspects of voice should be covered as aspects of WT2.

e)	Changes to support 6G Voice may not be entirely contained to IMS, and may require enhancements to 5GC NFs, as well as any new NFs defined for 6GS (and in regard to EPC interworking enhancements to EPC may be necessary – but should be minimized).

f)	Since in many regulatory environments Voice service is only permissible where Emergency Voice can be used by a subscriber, then, Emergency Voice in 6GS should be defined as an essential service using the principles of non-emergency voice where applicable.

# Proposal

Based on the above, it is proposed to use the following text to describe the Work Task and Key Issues for 6G voice and 6G emergency voice.

*** First Change ***

# Annex A.X. WT#1.4

The following work tasks are to investigate the essential and regulatory services for 6G:

## A.X	Work Task for Voice Services in 6G:

This work task will investigate how to define voice services for the 6G system – the following aspects will be studied:

- How the 6GS indicates to the UE that it supports voice services.

- Whether and how to update the IMS specification to accommodate any changes to the core network to support a 6G IPCAN.

- Study the necessary capabilities in the 6GS required to support IMS based voice over the 6G access network.

- How to support voice services via 6G NTN access.

NOTE: It is assumed that service continuity for voice and emergency voice is addressed in a dedicated KI for WT 2.

- Whether and How to simplify the core procedures to reduce signalling load and/or increase network efficiency.

## A.Y	Work Task for Emergency Voice Services in 6G:

This key issue will investigate how to define emergency voice services for the 6G system:

- How the 6GS indicates to the UE that it supports emergency voice services.

- How to re-use the capabilities for voice service also for emergency voice services, including interactions with the 6G IPCAN.

- Study the necessary capabilities in the 6GS required to support IMS based emergency voice over the 6G access network.

- How to support emergency voice services via 6G NTN access.

- Whether and how to simplify the core procedures to increase probability of completing an emergency call, reduce signalling load, and/or increase network efficiency for emergency voice service.

Additionally, this key issue will consider:

- How to establish and terminate the PDU session (or equivalent) access for the emergency voice service.

- How to identify emergency services in the access network, core network NFs, and IMS nodes that the PDU session (or equivalent) is being used for emergency voice service; and how the required priority for an emergency call is supplied by each entity in the call path.

- How to support UE’s in Limited-Service state can establish emergency session.

Editor’s Note:	It is FFS if other key issues for other essential services will be defined.

*** Second Change ***

# 5.X. Key Issue #X: Voice Services for 6G

This key issue will investigate how to define voice services for the 6G system – the following aspects will be studied:

- How the 6GS indicates to the UE that it supports voice services.

- Whether and how to update the IMS specification to accommodate any changes to the core network to support a 6G IPCAN.

- Study the necessary capabilities in the 6GS required to support IMS based voice over the 6G access network.

- How to support voice services via 6G NTN access.

NOTE: It is assumed that service continuity for voice and emergency voice is addressed in a dedicated KI for WT 2.

- Whether and How to simplify the core procedures to reduce signalling load and/or increase network efficiency.

# 5.Y. Key Issue #Y: Emergency Voice Services for 6G

This key issue will investigate how to define emergency voice services for the 6G system:

- How the 6GS indicates to the UE that it supports emergency voice services.

- How to re-use the capabilities for voice service also for emergency voice services, including interactions with the 6G IPCAN.

- Study the necessary capabilities in the 6GS required to support IMS based emergency voice over the 6G access network.

- How to support emergency voice services via 6G NTN access.

- Whether and how to simplify the core procedures to increase probability of completing an emergency call, reduce signalling load, and/or increase network efficiency for emergency voice service.

Additionally, this key issue will consider:

- How to establish and terminate the PDU session (or equivalent) access for the emergency voice service.

- How to identify emergency services in the access network, core network NFs, and IMS nodes that the PDU session (or equivalent) is being used for emergency voice service; and how the required priority for an emergency call is supplied by each entity in the call path.

- How to support UE’s in Limited-Service state can establish emergency session.

*** End of Changes ***