SA WG2 Meeting #170	S2-2507422

25 – 29 Aug, 2025, Goteborg , SE

Source:	T-Mobile USA (Rapporteur), Nokia

Title:	Document Key Issue for capability exposure in TR 23.700-56

Document for:	Agreement

Agenda Item:	20.7.1

Work Item / Release:	FS_NG_RTC_Ph3_ARC / R20

Abstract of the contribution: This contribution is intended to provide a Key Issue relating to the enhancement of exposure capabilities for the Study on system architecture for next generation real time communication services Phase 3

# Proposal

Based on the objectives of the SID in document SP-25085, it is proposed to use the following text for a Key Issue of TR 23.700-56.

*** First Change ***

## 5.2	Key Issue #2: Enhancements to capability exposure framework for eMMTel services

This Key Issue will investigate how to enhance IMS capability exposure framework to expose IMS capability for eMMTel services (voice/video) and related events besides Data Channel, supporting IMS session establishment, modification and release to enable eMMTel services that may require enhancing audio/video services with DC capabilities.

Solutions should describe how to enhance Rel-19 IMS event exposure framework and event definition to support IMS DC events supported by DCSF (e.g. application downloading event) and non-DC events, including subscriber specific and non-subscriber specific supported by IMS AS, (e.g. subscriber registration event, or event of all UEs calling a particular customer support number and that have DC capabilities for customization of customer support to UE capabilities).

NOTE 1: 	This Key Issue will analyze relevant use cases, the gap between OMA and 3GPP APIs and specify exposure functionality missing from OMA APIs.

NOTE 2:	This Key Issue does not intend to transfer the responsibility of OMA API(s) to 3GPP.This Key Issue will study:

-	Identification of new events and/or capabilities to be exposed, in addition to those already listed in Table AD.2.5.3-1 of TS 23.228 V19.3.0 [2].

-	Assessment of whether existing OMA APIs support the exposure of the newly identified events and/or capabilities.

*** End of Changes ***