3GPP SA WG2 Meeting #170	S2-2507427

# Goteborg, Sweden, 25th – 29th Aug 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.682 | CR | xxxx | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [Draft] Support of S&F data storage quota and data retention period |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, Sateliot, Novamint |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI20_SAT_S&F-ARC |  |  |  |  |  | Date: |  |  | 2025-08-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This draft CR proposes to support mechanism to provision S&F data storage quota and data retention period in the network by AF |  |  |  |  |  |  |  |  |
| Summary of change: |  | Support mechanism to provision S&F data storage quota and data retention period in the network by AF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Support of S&F data storage quota and data retention period is not available. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.6.1.11, 5.6.4.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*************************** Start of Changes ***************************

#### 5.6.1.11	Specific Parameters for Store and Forward Satellite operation information

This monitoring event allows the SCS/AS to be notified of Store and Forward Satellite operation information, identified by EPS according to clause 4.13.9 of TS 23.401 [7].

1.	The SCS/AS sets Monitoring Type to "Store and Forward Satellite operation information" prior to sending Monitoring Request to the SCEF as in step 1 of clause 5.6.1.1. The SCS/AS includes Store and Forward Satellite operation information including S&F data retention period per UE, S&F data storage quota per UE, S&F data retention period per satellite and S&F data storage quota per satellites in the Monitoring Event configuration request to the SCEF.

2.	The SCEF executes step 2 of clause 5.6.1.1.

3.	The SCEF executes step 3 of clause 5.6.1.1.

4. The HSS executes step 4 of clause 5.6.1.1. In addition, it checks whether the S&F data retention period; S&F data storage quota, S&F data retention period per satellite and S&F data storage quota per satellite are acceptable based on operator policy, if acceptable then the HSS sets the S&F data retention period per UE; S&F data storage quota per UE, S&F data retention period per satellite and S&F data storage quota per satellite, if it is provided. If any of the values are not acceptable, the HSS rejects the request by executing step 8, and provides a Cause value indicating the reason for the failure condition to the SCEF.

5.	The HSS executes step 5 of clause 5.6.1.1.

6.	The MME executes step 6 of clause 5.6.1.1 and starts watching for Store and Forward Satellite operation information events.

7-9.	Steps 7-9 of clause 5.6.1.1 are executed.

*************************** Next Changes ****************************

#### 5.6.4.4	Specific Parameters for Monitoring Event: Store and Forward Satellite operation information

This monitoring event allows the SCS/AS to provision Store and Forward Satellite operation information including S&F data retention period per UE, S&F data storage quota per UE, S&F data retention period per satellite and S&F data storage quota per satellite into SCEF.

1.	The SCS/AS sets Monitoring Type to “Store and Forward Satellite operation information" in the Monitoring Request to the SCEF sent as in step 1 of 5.6.4.1. The SCS/AS includes Store and Forward Satellite operation information including S&F data retention period per UE, S&F data storage quota per UE, S&F data retention period per satellite and S&F data storage quota per satellites in the Monitoring Event configuration request to the SCEF.

2.	The SCEF executes step 2 of 5.6.4.1.

3.	The SCEF triggers PCRF initiated IP-CAN session modification procedure, including the UE IP address and the Store and Forward Satellite operation information received from the SCS/AS. The PCRF provisions Store and Forward Satellite operation information received in the PCEF.

4.	The SCEF executes step 4 of 5.6.4.1..

*************************** End of Changes ****************************