SA WG2 Meeting #170	S2-2507428

25 – 29 Aug, 2025, Goteborg , SE

Source:	Xiaomi

Title:	[WT#4, ISAC] Supporting Integration of Sensing and Communication (ISAC)

Document for:	Agreement

Agenda Item:	20.6.4

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: this paper is aiming to provide justification and clarification for 6G ISAC as well as further Work Task and potential key issues.

1. Justifications

SA2 Study on Architecture for 6G System was approved in SP-250806, which included the WT#4 “integration of sensing and communication” as following:

WT#4: Study the integration of Sensing and Communication over 3GPP access, considering the sensing modes to be supported and other sources of sensing data.

NOTE 5:	The detailed scope of this WT#4 will be determined considering the scope and work of R20 FS_Sensing_ARC.

Gap Analysis: What are the limitations or gaps in existing architecture (from Rel-15 to Rel-20) that this proposal aims to resolve?

For the status of 5G-A ISAC:

- In Rel-19, SA1 has studied the use cases and requirements of 5G system to support for integrated sensing and communication service. The scope supports all sensing modes, (e.g., UE – gNB bistatic sensing, gNB bistatic/monostatic sensing, UE – UE bistatic/monostatic), and also support non-3GPP sensing (seen as TR22.837, TS 22.137).

- In Rel-20, after TSG#108, the scope of Rel-20 SA2 5G-A Sensing was determined and further aligned with RAN to support for only gNB based sensing. All other requirements/modes are not supported by Rel-20 SA2 5G-A Sensing. This study is still going (seen as TR 23.700-14).

Observation 1: Rel-20 5G-A, only gNB based sensing is supported, All other sensing modes/requirements are not supported.

For status of 6G ISAC

- SA1 is still working on the new use cases and requirements for 6G ISAC. Up to August 2025, 20 agreed new use cases are captured in clause 7 of TR22.870. Meanwhile, the 6G system shall be able to support 5G system requirements defined in TS 22.137 “integrated sensing and communication”. It means all other requirements/modes not supported by Rel-20 5G-A Sensing will be supported in 6G.

- SA2 now is also starting to study 6G system architecture (approved in SP-250806), which includes WT#4 supporting integrated sensing and communication (ISAC). It supposes to support all the sensing modes as well as non-3GPP sensing.

Observation 2: Rel-20 6G, the legacy requirements from SA1 5GA ISAC, and new requirements from SA1 6G ISAC are both supported in 6G ISAC.

For relationship between 5G-A and 6G ISAC

- 6G ISAC supports all sensing modes, including legacy requirements from SA1 5G-A ISAC and new potential requirements from SA1 6G ISAC.

- The 6G ISAC scope needs to consider the work/scope of Rel-20 5G-A sensing, e.g., 6G ISAC scope and study will consider the progress/outcome of SA1 6G ISAC and 5G-A Sensing,

Proposal 01: 6G ISAC can start to discuss the work task and potential key issues in parallel with 5G-A sensing. The 5G-A sensing work can be considered as possible inputs for 6G ISAC study.

Technical Impact: How would this WT/KI influence:

- WT1.1 Overall system architecture,e.g., control signalling to support sensing service procedures,

- WT1.2 Related to user plane architecture, QoS, policy, and network exposure framework, etc. to provide QoS/Policy management for sensing service, sensing results exposure, etc.

- WT#5 data framework to support sensing data collection, transportation, sensing result calculation, and sensing result exposure, sensing data storageetc.

**** First Change ****

Annex A.X. WT#X Scope: Supporting integration of Sensing and Communication

Editor's Note: Describe the technical scope of the proposed Work Task. If applicable, suggest logical subdivision of this WT into smaller sub-WT. This clause is part of the TR Annex.

WT#X: Study the integration of Sensing and Communication over 3GPP access, considering the sensing modes to be supported and other sources of sensing data.

- Architecture and function(s) design to support sensing service.

- Service authorization, revocation and configuration parameters provisioning. This includes investigating mechanisms for authorization and revocation sensing service requests from the service consumer (e.g., UE, 6G NF, 3rd party AF, etc.); It also includes configuration parameters provisioning to sensing entity(ies).

- Discovery and selection of sensing entity(ies) to support Sensing service. This is performed based on service requirements triggered by the service request and capability of the sensing entity(ies).

- Sensing data and the associated information collection and transport mechanisms for result calculation.

- Mechanisms for providing sensing associated information and result (including sensing result and contextual information) exposure, for one time, periodic or event based reporting.

- Supporting sensing service continuity (due to mobility of sensing target or/and sensing entity).

- Supporting integration of non-3GPP sensing.

NOTE Y:	The detailed scope of this WT will be determined considering the scope and work of R20 FS_Sensing_ARC.

**** Second Change ****

5.X. Key Issue #X: Key Issue Title

Editor's Note: This clause defines the potential scope of KI(s) and is part of the TR.

5.X.1	Key Issue #X.1: System Architecture to Support Sensing

This key issue will address the general system architecture and function(s) design required to provide end-to-end support for Sensing service as well as the interactions between NF(s). Potential privacy requirements if identified will be considered for this key issue.

NOTE X:	Solutions for other key issues may also impact general architecture aspects covered by this key issue.

5.X.2	Key Issue #X.2: Authorization, Revocation and configuration provisioning for Sensing service

The key issue is to address the authorization and revocation for a particular Sensing Service, specifically:

For the authorization of the Sensing Service request, and the revocation of an on-going Sensing Service, the study needs to address the following aspects:

-	How and which entity performs Sensing Service request authorization, revocation of an on-going sensing service, if required, and the criteria for the Sensing Service request authorization, and revocation of an on-going sensing service based on the requirements from TS 22.137 [2];

For the authorization and revocation for the Sensing Entities, the study needs to address the following aspects:

-	Whether, when and which entity performs the authorization and revocation of the Sensing Entity(/Entities);

For configuration parameters provisioning to the Sensing Entity(ies), the study includes:

-	How and what parameters need to be exchanged between the Sensing Entities and the Core Network.

NOTE:	the parameters need to be aligned with RAN WGs.

5.X.3	Key Issue #X.3: Sensing Entity and Sensing Function Discovery and (Re-)Selection

This key issue aims to study how to perform the Sensing Entity(ies) and Sensing Function(s) discovery and (re-)selection, in particular:

For discovery and (re-)selection of Sensing Entity(ies), the study includes:

- Which entity(ies) and how performs the discovery and (re-)selection of the network function(s) involving the sensing service, e.g., Sensing Entity(ies), Sensing fucntion;

- What the information and/or criteria are taken into account for the discovery and (re-)selection of the network function(s) involving the sensing service;

5.X.4	Key Issue #X.4: Sensing Data Collection and sensing result exposure

Integration of sensing and communication requires 3GPP network to collect and transfer sensing data and the associated information to a Sensing Function.

This Key Issue aims to study:

-	How to collect sensing data (and the associated information, if any) for Sensing result generation?

-	Which Sensing Function performs Sensing result generation?

-	Whether and what the above associated information is required for Sensing result generation

NOTE:	Implications to RAN or RAN dependent aspects will be coordinated with RAN WGs.

5.X.5	Key Issue #X.5: Sensing Result Exposure

This key issue addresses the exposure of Sensing results to support Sensing Services. The aspects to be studied in this key issue include:

-	Study what types of Sensing results with or without the Sensing contextual information defined in TS 22.137 [2] could be exposed from the Core Network to the Sensing Service Consumer.

-	How the Core Network exposes Sensing results to the Sensing Service Consumer (e.g. one time, periodic or event trigger).

5.X.6	Key Issue #X.6: Supporting sensing service continuity

This Key Issue will study how to support sensing service continuity, including:

- How to support sensing service continuity, due to sensing target mobility, or/and sensing entity mobility.

5.X.7	Key Issue #X.7: Supporting integration of non-3GPP sensing.

This Key Issue will study how to support integration of non-3GPP sensing, including:

- How to discovery/selection of non-3GPP sensing entity

- How to collect and transfer the sensing data from non-3GPP sensing entity(ies)

- how to handle or integrate the non-3GPP sensing data for result calculation

**** End of Changes ****