3GPP SA WG2 Meeting #170	S2-2507431

# Goteborg, Sweden, 25th – 29th Aug 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6421 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on UCMF instance |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | RACS, TEI19 |  |  |  |  |  | Date: |  |  | 2025-08-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | It is not clear if it is possible to deploy more than one UCMF instance in the network. For e.g. if we check below section it states: ### 6.3.15 UCMF Discovery and Selection The AMF, MME, NEF, AF, SCEF, SCS/AS may utilize the NRF to discover UCMF instance(s) unless UCMF information is available by other means, e.g. locally configured in UCMF services consumers. Consider: AMF-1 is connected to UCMF-1 AMF-2 is connected to UCMF-2 UCMF-1 assigns a URC-ID-1 for a given UE radio capability. UE moves from AMF-1 to AMF-2 and URC-ID-1 is received by AMF-2. When AMF-2 contacts UCMF-2 how to maintain uniqueness of the URC ID assigned between UCMF-1 and UCMF-2 is not clear in the specification. Similarly UCR-ID can be passed between AMF(s), AMF-MME, NG-RAN node(s) during different procedures. If more than one UCMF is deployed maintaining uniqueness of the URC ID seems to be difficult with current protocol. Thus we propose to clarify one of the below based on discussion during the meeting: - a single instance of UCMF is deployed; or - if more than one instance of UCMF is deployed then they all resolve/assign exactly same URC ID for a given URC. i.e. the algorithm which is based on implementation is replicated in all the UCMF instances of the network. In this release of specification source company preference is to clarify this aspect without any new protocol impacts. If there is a desire to deploy more than one UCMF instance with independent algorithms to assign/resolve the URC ID then in our understanding it will need a new solution that should be handled with a new work item. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that there is either a - single UCMF instance in network; or - multiple UCMF instances but have exactly same output for all the UCMF services. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not clear for the implementers how the uniqueness of URC ID is maintained in the network if more than one UCMF instance is deployed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.21 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*************************** Start of Changes ***************************

### 6.2.21	UE radio Capability Management Function (UCMF)

The UCMF is used for storage of dictionary entries corresponding to either PLMN-assigned or Manufacturer-assigned UE Radio Capability IDs. An AMF may subscribe with the UCMF to obtain from the UCMF new values of UE Radio Capability ID that the UCMF assigns for the purpose of caching them locally.

Provisioning of Manufacturer-assigned UE Radio Capability ID entries in the UCMF is performed from an AF that interacts with the UCMF either directly or via the NEF (or via Network Management) using a procedure defined in TS 23.502 [3]. A UCMF that serves both EPS and 5GS shall require provisioning the UE Radio Capability ID with the TS 36.331 [51] format or TS 38.331 [28] format or both the formats of the UE radio capabilities.

The UCMF also assigns the PLMN-assigned UE Radio Capability ID values.

Each PLMN-assigned UE Radio Capability ID is also associated to the TAC of the UE model(s) that it is related to. When an AMF requests the UCMF to assign a UE Radio Capability ID for a set of UE radio capabilities, it indicates the TAC of the UE that the UE Radio Capability information is related to.

The UCMF stores a Version ID value for the PLMN assigned UE Radio Capability IDs so it is included in the PLMN assigned UE Radio Capability IDs it assigns. This shall be configured in the UCMF.

The UCMF may be provisioned with a dictionary of Manufacturer-assigned UE Radio Capability IDs which include a "Vendor ID" that applies to the Manufacturers of these UE and a list of TACs for which the PLMN has obtained-Manufacturer-assigned UE Radio Capability IDs.

A PLMN-assigned UE Radio Capability IDs is kept in the UCMF storage as long as it is associated with at least a TAC value. When a TAC value is related to a UE model that is earmarked for operation based on Manufacturer assigned UE Radio Capability IDs, this TAC value is disassociated in the UCMF from any PLMN assigned UE Radio Capability IDs.

For the case that the PLMN is configured to store PLMN assigned IDs in the Manufacturer Assigned operation requested list defined in clause 4.4.1a, the UCMF does not remove from storage any PLMN assigned UE Radio Capability ID no longer used and rather quarantines it to avoid any future reassignment.

A UCMF dictionary entry shall include also the related UE Radio Capability for Paging for each RAT.

/// Option-1:

A single UCMF instance is deployed in the network.

/// Option-2:

If more than one UCMF instance is deployed in the network, all the UCMF instance(s) are expected to assign or resolve the UE Radio Capability ID with the same values. i.e. all the UCMF instance(s) together behave as a single instance of UCMF.

*************************** End of Changes ****************************