SA WG2 Meeting #170	S2-2507433

25 – 29 Aug, 2025, Goteborg , SE

Source:	Xiaomi

Title:	update solution#3 High-level procedure and architecture for sensing conditions

Document for:	Agreement

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC/Rel-20

Abstract of the contribution: this paper proposes to include the sensing assistance information in the sensing service request from AF, and also SF will consider the sensing assistance information when calculating the sensing result. When SF exposes the sensing result, the “sensing contextual information” also may be included.

1. Discussion

In TS 22.137, Sensing assistance information is defined to support the derivation of a sensing result from 3rd party AF. And sensing contextual information is defined for Sensing Function to expose additional information (e.g., map information, time of capture, area information, etc.) besides sensing result to sensing service consumer.

Sensing assistance information: information that is provided to the 5G system from a trusted third-party and can be used to support the derivation of a sensing result. This information does not contain 3GPP sensing data.

NOTE 1:	Examples of sensing assistance information are map information, area information, a UE ID attached to or in the proximity of the sensing target, UE position information, UE velocity information etc.

Sensing contextual information: information that is exposed with the sensing results by 5G system to a trusted third-party which provides context to the conditions under which the sensing results were derived. This information does not contain 3GPP sensing data.

NOTE 2:	Examples includes map information, area information, time of capture, UE location and ID. This contextual information can be required in scenarios where the sensing result is to be combined with data from other sources outside the 5GS.

In key issue#4, the associated information is also required during sensing result calculation. And in key issue#5, sensing contextual information may be also included when exposing the sensing result.

However, in solution#3, the steps in clause 6.3.2.1 General procedure could reflect these two kinds of information.

Proposal: adding the sensing assistance information in step 1 and step 13. And adding the sensing contextual information in step 14.

**** First Change ****

6.3	Solution #3: High-level procedure and architecture for sensing

6.3.0	High level principles

The solution follows the following principles:

-	Provide a general framework for sensing that captures all the operations related to the sensing service and can be used as a common baseline:

-	Intentionally keep the general procedure at high level so that details can be described in solution to specific Key Issues.

-	Introduce the Sensing Function in the architecture for managing the sensing related functionalities, e.g. requesting to sensing entities to perform sensing measurement, receiving the 3GPP Sensing Data from sensing entities, determining the Sensing Result. Details see clause 6.3.1.2.

-	The Sensing Function may be split into Sensing Control Function and Sensing Processing Function as follows, and the details are specified in clause 6.3.1.3:

-	The Sensing Control Function refers the logical function which is involved to support Sensing Service and responsible for providing the related configuration information.

-	The Sensing Processing Function responsible for receiving the Sensing Data and for calculating the Sensing Result.

6.3.1	Description

6.3.1.1	Terms used within this solution

For the purposes of the present solution the following definition applies:

Sensing Control Function: Indicating the logical function which is involved to support Sensing Service and responsible for providing the related configuration information.

Sensing Processing Function: responsible for receiving the 3GPP Sensing Data and for performing the elaboration for determining the Sensing Result.

6.3.1.2	Solution description

This solution aims to provide the general procedure to support the sensing functionality.

The main principles of the procedure are:

-	the AF acting as the Sensing Service Consumer initiates the procedure for sensing by sending the sensing service subscription request to the Core Network.

-	The Network Exposure Function determines an appropriate Sensing Function according to the request from the AF.

-	Upon receiving a request for subscription to sensing service from the AF, the Sensing Function proceeds as follows:

-	Discovers the Sensing Entity/Entities and provides the configuration parameters to them;

-	May establish the path for the reporting of the 3GPP sensing data from the Sensing Entity/Entities. The reporting of 3GPP Sensing Data (as defined in TS 22.137 [2]) is performed from Sensing Entities to Sensing Function.

-	The 3GPP Sensing Data is transmitted to the Sensing Function, which further processes the data, obtains the Sensing Result (as defined in TS 22.137 [2]) and exposes the Sensing Result to the AF.

The following Figure 6.3.1.2-1 provides the reference architecture baseline for sensing.

Figure 6.3.1.2-1: Reference Architecture

NOTE:	Whether the control signalling to Sensing Entity/Entities is sent via AMF or directly is addressed by the other solutions.

Editor's note:	Whether the Controlling and Processing Function is one NF or separate into two NFs and whether and how new interface is defined between them is FFS.

6.3.2	Procedure

6.3.2.1	General procedure

Figure 6.3.2.1-1: Procedure of the sensing general framework

1.	The Sensing Service Consumer (i.e. AF) requests the NEF to obtain the Sensing result from the network via Sensing Service Request including optional sensing assistance information (e.g., map information, area information, etc.) as defined in clause 3.

NOTE 1:	Detailed information of sensing service requirement, sensing service type and corresponding information is addressed by solutions to Key Issue #2 and/or Key Issue #5.

2.	Upon receiving request from AF, the NEF authorizes the AF for usage of the Sensing Service.

3.	The NEF may discover and select the candidate Sensing Function(s).

NOTE 2:	Details regarding NEF authorising the AF is addressed by solutions to Key Issue 2.

4.	The NEF sends the request to the Sensing Function, where the request message contains information received in step 1.

5.	The Sensing Function may perform further authorisation for the sensing request based on the parameters provided by the AF.

NOTE 3:	Details regarding Sensing Function authorising the AF's request is addressed by solutions to Key Issue #2.

NOTE 4:	the Sensing Function or NEF may respond in step 6 and 7 indicating the result of sensing service subscription (e.g. for the failure case).

NOTE 5:	Details regarding Sensing Function discovery/selection is addressed by solutions to Key Issue #3.

6.	If the Sensing Function performs authorisation, it may indicate whether the request has been authorised or rejected via Nsf_SensingServiceExposure_Subscribe Response.

7.	The NEF notifies the AF whether the subscription has been authorised or rejected via Sensing Service Response.

8.	The Sensing Function performs the discovery and selection for the Sensing Entity, based on e.g. the sensing service requirement, and sensing capability supported by the Sensing Entity.

NOTE 6:	The details regarding the discovery and selection of the Sensing Entity are addressed by the solution to Key Issue #3.

9.	The Sensing Function determines the information for the subsequent Sensing measurement and sends the sensing service request message including the configuration parameters to the Sensing Entity.

NOTE 7:	Details regarding Sensing Function sending the configuration parameters, including whether those configuration parameters are directly sent to the Sensing Entities or not, is addressed by solution to Key Issue 1 and Key Issue 6.

10.	Upon receiving the sensing service request message from the Sensing Function, the Sensing Entity may trigger the establishment of the path for 3GPP sensing data reporting.

11.	The Sensing Entity performs the Sensing measurement and obtains 3GPP sensing data.

12.	The 3GPP sensing data is transmitted to the Sensing Function.

NOTE 8:	Details regarding the 3GPP sensing data and the reporting is addressed by the solution to Key Issue 4.

13.	After receiving the 3GPP sensing data, the Sensing Function determines the Sensing result. Optionally Sensing Function may additionally request the Sensing assistance information from the AF to derive the sensing result.

14.	The Sensing Function sends the Sensing result to the NEF via Nsf_SensingServiceExposure_Notify. Optionally the sensing contextual information (e.g., time of capture, area information, etc.) is also sent to the AF with Sensing result if needed.

15.	The NEF provides the Sensing result to the AF via Sensing Service Notify.

NOTE 9:	The steps 11-15 may be performed one-time or a periodically or based on event trigger depending on the sensing service request from AF.

NOTE 10:	Details regarding exposing the Sensing Result are addressed by the solution to Key Issue #5.

NOTE 11:	In addition to support the Sensing Result exposure via control plane, whether the Sensing Result can also be exposed via data connection for sensing operation is addressed by the solution to Key Issue #5.

NOTE 12:	Details regarding the procedure between the Sensing Function and the Sensing Entity from step 9 to 12 will be addressed by the associated solutions.

Editor’s note: Mechanism for sensing should enable the operator to charge for the Sensing Service, which is under the remit of SA WG5.

6.3.2.2	Control and Processing Split of the Sensing Function

Editor's note:		Whether the Controlling and Processing Function is one NF or separate into two NFs and whether and how a new interfaces is defined between them is FFS.

To enable flexible scaling of Sensing Functions and/or localization deployment, the Sensing Function in clause 6.3.2.1 is further split into Sensing Control Function (SCF) and the Sensing Processing Function (SPF). The Sensing Control Function can be located in a central area which can control multiple distributed Sensing Processing Functions.

Figure 6.3.2.2-1: Procedures regarding Sensing Control Function and Sensing Processing Function

NOTE:	Interactions of Sensing Control Function and Sensing Processing Function can be addressed by the other solutions.

Figure 6.3.2.2-1 presents the interactions between SCF and SPF. After SCF received a sensing request in step 8 in Figure 6.3.2.1-1, the SCF sends a Sensing Processing Configuration Request to SPF to request the SPF for the following execution in step 3. The SCF provides information to Sensing Entity in order that in step 10 of figure 6.3.2.1-1 the data connection for sensing operation will be established towards the suitable SPF (more details will be address in specific solutions). After SPF determined sensing result, it sends a Sensing Processing Result Report to SCF. The Report may be performed one-time or a periodically or based on event trigger depending by step 2.

6.3.3	Impacts on Services, Entities and Interfaces

NEF:

-	Selection of Sensing Function.

-	Interaction with AF and Sensing Function for sensing authorization, invocation and revocation.

-	Provision the Sensing result retrieved from Sensing Function to AF.

Sensing Function:

For the Sensing Control Function:

-	Interaction with NEF for exposure of Sensing Results and reception of sensing requests.

-	Interaction with sensing entities for requesting to perform sensing measurement.

-	Interaction with Sensing Process Function.

For the Sensing Process Function:

-	Reception of 3GPP Sensing Data.

-	Determine the Sensing Result based on the 3GPP Sensing data.

-	Interaction with Sensing Control Function.

AF:

-	Interaction with 5GC for requesting sensing service to obtain the Sensing result(s).

-	Acquisition Sensing result(s) from 5GC.

**** End of Changes ****