SA WG2 Meeting #170	S2-2507440

25 – 29 Aug, 2025, Goteborg, SE

Source:	CableLabs, Charter Communications

Title:	[WT#1.3] Work task #1.3 Non-3GPP access

Document for:	Approval

Agenda Item:	20.6.1.3

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution:  Proposes scope of work task 1.3 on non-3GPP access. The proposed work task scope in the Annex is considered as the Key Issue on 6G non-3GPP access.

# 1. Justifications

Sub Work Task 1.3 scope as agreed in 6G System Study item SP-250806:

WT#1: Define the overall 6G architecture as collection of capabilities and high level functionalities considering the following sub work tasks and other work tasks to support 6G access network:

NOTE  1:	The duplication of functionality in RAN and CN will be avoided, while maintaining the existing RAN and CN functionality split.

…

WT#1.3: Study whether and how to support and/or enhance different non-3GPP access (e.g. Wi-Fi, wireline) in 6G and support multi-access data connections between 3GPP access and non-3GPP access.

Architectural Motivation: Integrating non-3GPP accesses with the 6G core is essential because it allows the 6G core network to manage and provide services through diverse connectivity options, creating a more seamless, reliable, and versatile network. Some compelling reasons for this integration include:

- Unified user experience: Users today expect seamless connectivity regardless of the underlying technology. By integrating Wi-Fi, fixed broadband, and other non-3GPP access networks with the 6G core, operators can provide a consistent service experience. Users maintain the same identity, authentication, and service profile whether they're connected via cellular towers, home Wi-Fi, or enterprise networks.

- Network capacity and coverage enhancement: Cellular networks alone cannot economically provide ubiquitous high-capacity coverage. Wi-Fi networks handle enormous amounts of data traffic in dense indoor environments like offices, airports, and homes. Fixed broadband connections offer high-bandwidth access in residential and business locations. Integration allows operators to leverage this existing infrastructure as extensions of their cellular network.

- Service continuity and mobility: Modern applications and services require uninterrupted connectivity as users move between different network environments. Integration enables seamless handovers between cellular and Wi-Fi without dropping calls or sessions, consistent quality of service policies across all access types, and single billing and service management regardless of access method.

Therefore, integrating diverse types of non-3GPP accesses with the 6G core transforms cellular networks from standalone systems into comprehensive connectivity platforms providing significant benefits for both users and network operators.

The key principles to be considered in integrating non-3GPP accesses with 6G core include:

- Standardized interfaces between different types of non-3GPP access networks and the core network using consistent protocols and procedures

- Capability to connect UE to the 6G core through any access network (e.g., Wi-Fi) independently with no reliance on other access network (e.g., 3GPP).

- Unified authentication and authorization of UE regardless of the underlying access technology.

- Seamless handover and service continuity between accesses regardless of the underlying access technology

- In case multiple accesses are available, the ability to select the best access for a specific service, to switch traffic from one access to the other in case one becomes unavailable/unacceptable, and to utilize multiple accesses for load balancing

- Support of both seamless and non-seamless offload use cases to address different operators’ needs

Gap Analysis: While 5G made significant strides in supporting different access types with the core network, there is still considerable room for improvement or re-architect how non-3GPP access is integrated in 6G:

- Simplifying and generalizing the way different non-3GPP accesses (untrusted WLAN, trusted WLAN, wireline) connect with the core network to eliminate the complexity of managing multiple integration methods.

- Inter-access handovers in the scenarios of mobility between different access types were not adequately addressed in 5G and can be further improved on in 6G.

- Unlike in 5G, ensure Non-Seamless WLAN Offload (NSWO) functionality will be part of the first 6G release system architecture.

Technical Impact: Support of non-3GPP access in 6G system architecture.

**** First Change (All New Text) ****

# 2. Annex A.X. WT#1.3 Non-3GPP Scope

Study below aspects on integration of non-3GPP access in 6G System architecture:

- Study how to support and/or enhance non-3GPP access networks (WLAN, wireline) with 6G Core network.

- Study how to support and/or enhance non-seamless WLAN offload (NSWO).

- Study how to support and/or enhance mobility and traffic flow control across multiple access networks.

NOTE 1: 6G will use a simplified common access architecture to support different non-3GPP access networks (WLAN, wireline).

NOTE 2: Security aspects will be addressed by SA3.

**** Second Change (All New Text) ****

# 3. Key Issue #X: Non-3GPP access

Study below aspects on integration of non-3GPP access in 6G System architecture:

- Study how to support and/or enhance non-3GPP access networks (WLAN, wireline) with 6G core network using a common access architecture.

- Study how to support and/or enhance non-seamless WLAN offload (NSWO).

- Study how to support and/or enhance mobility and traffic flow control across multiple access networks.

NOTE 1: A common access architecture for non-3GPP access networks implies a common architectural and procedural framework applicable to WLAN and wireline access.

NOTE 2: Security aspects will be addressed by SA3.