3GPP TSG-SA WG2 Meeting #170	S2-2507448

25 - 29 August 2025, Goteborg, Sweden

Agenda item:	20.4.2

Source:	NTT DOCOMO

Title:	Discussion on Robustness of Energy Consumption Estimation

WI/Release:	FS_EnergySys_Ph2/Rel-20

Document for:	Discussion and Decision

# 1	Discussion

Whereas there is no universally agreed definition of robustness, the common sense of the term in computer science is the capability of a system (e.g., algorithm, model, etc.) to provide results that do not change much when data or system parameters are slightly imperfect.

Adopting this concept to the energy consumption (EC) estimations provided by EIF would be interpreted as the capability of the estimation model/formula to provide results that do not change significantly when parameters change. More specifically, the robustness of the EIF’s EC estimation formula would be interpreted as follows:

It is expected that the EIF provides (almost) the same estimation of EC for the same amount of data transferred by the same UE.

If this requirement is not satisfied, the consumer (e.g., AF) may receive significantly different estimations of EC for the same amount of transferred data, which would be misleading information for it.

The following table provides the estimated EC and corresponding error for the same data volume (DV(UE)) transferred by the UE under different UPF loads (DV(UPF)), based on the following assumptions:

- The idle EC of UPF is 300 units of energy

- Actual/real EC for handling 1 unit of data is 0.0001unit of energy

- Volume of transmitted data by the UE is DV(UE) = 1000 units, so the actual EC(UE) is 0.1 unit of energy

Table 1. EC(UE) estimation by EIF using the total EC of UPF as the input for estimation

| DV(UPF) | Actual EC (UPF) | Estimated EC(UE) = ECUPF×DV(UE)DV(UPF) | Error% = ECUE-0.010.01×100 |
| --- | --- | --- | --- |
| 10000 (~10 UE) | 301 | 30.1 | 30000% |
| 100000 (~100 UE) | 310 | 3.1 | 3000% |
| 1000000 (~1000 UE) | 400 | 0.4 | 300% |
| 10000000 (~10000 UE) | 1300 | 0.13 | 30% |

As it is shown, the estimated EC by the EIF for the same amount of data could be quite different under different loads of UPF and event the error can be significant for a large number of UEs.

Assuming that there is no measurement error (which not a realistic condition), the error would be zero as shown in the following table.

Table 2. EC(UE) estimation by EIF using the EC(UPF) excluding the idle EC

| DV(UPF) | Actual EC (UPF) | Estimated EC(UE) = ECUPF-300×DV(UE)DV(UPF) | Error% = ECUE-0.010.01×100 |
| --- | --- | --- | --- |
| 10000 (~10 UE) | 301 | 0.1 | 0 |
| 100000 (~100 UE) | 310 | 0.1 | 0 |
| 1000000 (~1000 UE) | 400 | 0.1 | 0 |
| 10000000 (~10000 UE) | 1300 | 0.1 | 0 |

However, in practice, there are some errors in the measurements. The following table shows the estimation of EC(UE) in the case that there are +3% measurement error in EC(UPF) and -2% measurement error in EC(Idle UPF).

Table 3. EC(UE) estimation by EIF using the EC(UPF) excluding the idle EC considering measurement errors

| DV(UPF) | Measured EC (UPF) | Estimated EC(UE) = ECUPF-294×DV(UE)DV(UPF) | Error% = ECUE-0.010.01×100 |
| --- | --- | --- | --- |
| 10000 (~10 UE) | 310.03 | 1.603 | 1503 |
| 100000 (~100 UE) | 319.3 | 0.253 | 153 |
| 1000000 (~1000 UE) | 412 | 0.118 | 18 |
| 10000000 (~10000 UE) | 1339 | 0.1045 | 4.5 |

Comparing Table 1 (where it is assumed no measurement error) and Table 3 (with measurement errors) shows that excluding the idle EC from the total EC improves the robustness of the results event in the case of measurement errors (while it does not eliminate the issue).

# 2	Conclusions

It is proposed to introduce another alternative EC estimation formula, where the idle EC is excluded from the total EC, in the Annex T of TS 23.501 and leave it to implementation and operator policy to use the appropriate estimation formula by EIF.