SA WG2 Meeting #170	S2-2507449

25 – 29 Aug, 2025, Goteborg, SE

Source:	Apple

Title:	[WT#1.2, SBA Framework] SBA enhancements in 6G

Document for:	Agreement

Agenda Item:	20.6.1.2

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution provides technical reasoning, gap and impact analysis behind WT#1.2 for SBA Framework, as well as related updates to the scoping of the WT.

# 1   Justifications

The 6G SA2 SID in SP-250806 [x2] approved at TSG SA #108 contains the following sub–work task corresponding to the SBA framework:

1.2.	Study whether and how to support and/or enhance the following aspects in 6G: the SBA framework, network slicing, network sharing, user plane architecture, QoS framework, policy framework, network exposure framework, architecture for specific scenarios e.g. fixed wireless access, localized service access.

## Architectural Motivation

It is broadly assumed that the SBA framework defined in 5G can be reused for the 6G system. SBA offers several advantages such as extensibility to add new functionality in a simple manner, loose coupling between services, and reusability. However, 3GPP needs to evaluate if the functionality and system features inherent to that framework is still sufficient to fulfil the requirements for NF communication in 6G.

## Gap Analysis

Compared to traditional point-to-point signalling with its carefully crafted procedures between rather fixed termination points, SBA is known to cause an increase in signalling traffic within the Core Network. Furthermore, the proliferation of NFs has been observed in the 5G era. It is anticipated that the number of NFs will continue to increase in the 6G landscape, coupled with the prevailing trend of decomposing functional components into smaller, self-contained services. More granular services offer smaller, more specific functionalities, necessitating multiple interactions to complete intricate tasks. This approach enhances flexibility and adaptability but requires more signalling message exchange. Consequently, it is probable that the signaling load will increase further. Another aspect is that a high number of NF interdependencies translates into a need to reduce latency. Stateless NFs, lacking persistent storage, frequently perform fetch and update operations, leading to additional latency. A database instance may also store data from one or multiple NFs within a chain of messages exchanges, resulting in latency for each step. Both of these aspects, signalling load and reduced delays, represent performance requirements that SBA will have to fulfil in the future.

Observation 1: SBA involves a lot of signalling traffic and requires reasonable TN latency. The number of NFs and related interactions between NFs is expected to grow further.

In 5G SBA, NFs expose RESTful APIs based on HTTP/2. Communication between NFs can use the Request/Response model or the Subscribe/Notify model for 5G Services. Despite its advanced features HTTP/2 presents several challenges. Above all, the protocol continues to experience head-of-line (HOL) blocking at the transport layer due to its dependence on TCP, especially when the transport connection is subject to packet loss. This is when TCP's head of line blocking will not allow releasing received data to higher layer, even if the data is completely received for independent HTTP/2 requests or responses. Connection setup also requires some time, as clients can start sending HTTP requests after 1 RTT (without using TLS) or 2 RTT (with TLS). Additionally, the reliance on TLS may introduce extra latency, affecting performance. For 6G, it would be desirable to limit HOL blocking and reduce connection setup delays.

Observation 2: For 6G, it would be desirable to limit HOL blocking and reduce connection setup delays.

The service-based interface uses the HTTP/2 protocol with JSON as the data serialization scheme. JSON is a textual format where serialization/deserialization comes at the cost of high computational load and energy consumption. Compared to P2P interfaces, the size of signaling messages exchanged over SBA also tends to be larger. Apart from protocols headers and communication patterns, the encoding and serialization method affects the size of the messages. Binary formats have shown significantly better encoding/decoding performance and shorter message size. Moreover, the computational cost of serialization, parsing and handling of redundant information also needs to be considered. 5G studies made by CT4 recommended considering a more efficient alternative to JSON in further 3GPP releases. Although the detailed evaluations are in the remit of CT4, SA2 can consider the general aspects and suitability as the SBA framework gets evaluated. Along with an evaluation of SBA transport, as mentioned above, a study to explore alternative lightweight serialization/encoding schemes may be beneficial.

Observation 3: JSON serialization/deserialization comes at the cost of high computational load and the message payload is large. An exploration of alternative (lightweight) serialization/encoding schemes may be beneficial.

Rel-16 introduced the SCP to help enable dynamic scaling and management of communication and services in the 5G network which could be considered somewhat similar to the STP in 2G/3G or the DSC/DRA in Diameter. Even though the SCP includes functions for indirect communication with/without delegated NF discovery, the process of querying the NRF is still a cumbersome operation (with lots of parameter information being provided) where the NRF remains a critical resource in the 3GPP system. Since these aspects are central to the operation of SBA, potential optimizations and options to enhance the resiliency of the system can be included to the study.

Observation 4: Efficient NF discovery and/or grouping of NFs may enhance the resiliency of the NF system.

In relation to the interworking architecture, 4G-5G introduced the N26 interface, which is a P2P interface between MME and AMF. As 5G adopted SBA and assuming 6G is going to be based on SBA also, 3GPP can study adopting a service-based interface between 5G and 6G networks. From SA2 perspective, architectural implications, e.g., to describe NF service and related functional aspects, can be studied under WT#2 (in general) or as part of WT#1.2 under SBA framework (for more specific interface aspects).

In the current 3GPP system N4 is a P2P interface between UPF and SMF. 5G studied UPF event exposure and defined Nupf services to expose UPF related information. In continuation of this trend, 6G may study incremental enhancements for better scalability and flexibility, including whether and how to evolve N4 into a service-based interface, e.g., to enable direct communication of some information without routing through AMF or SMF.

Observation 5: In conjunction with the overall architecture, the study may evaluate if additional service-based interfaces need to be defined.

## Technical Impact

Overall, the technical impact this WT may have on the 3GPP system may be incremental. There is a potential to optimize SBA transport while reusing general principles from 5G. In addition, several areas need to be analysed, which may influence the NF system in general and the interworking between different NFs in particular. Also, API parameter enhancements related with updates to SBA transport protocols may be expected. Furthermore, any new SBIs will need to be evaluated considering how it maps into the overall architecture what are the Pros and Cons.

## Conclusions

In conclusion, this contribution discusses the scope of the SA2 6G SID for WT#1.2 in Rel-20. We have the following observations:

Observation 1: SBA involves a lot of signalling traffic and requires reasonable TN latency. The number of NFs and related interactions between NFs is expected to grow further.

Observation 2: For 6G, it would be desirable to limit HOL blocking and reduce connection setup delays.

Observation 3: JSON serialization/deserialization comes at the cost of high computational load and the message payload is large. An exploration of alternative (lightweight) serialization/encoding schemes may be beneficial.

Observation 4: Efficient NF discovery and/or grouping of NFs may enhance the resiliency of the NF system.

Observation 5: In conjunction with the overall architecture, the study may evaluate if additional service-based interfaces need to be defined.

Based on the observations and analysis above, we propose an update of the WT scope in the following section.

**** First Change ****

# Annex A.X. WT#1.2 Scope for SBA Framework

WT#1: Define the overall 6G architecture as collection of capabilities and high-level functionalities considering the following sub-work tasks and other work tasks to support 6G access network:

1.2.	Study how the 6G system supports the SBA framework, including the following aspects:

- Study and evaluate whether enhancements are needed to support SBA in 6G, such as extensions or potential optimizations based on the framework defined in 5G. This may include:

  - Whether and how to provide higher flexibility and better performance for NF communication, e.g., to limit head of line blocking, reduce connection establishment delay, etc.

  - Whether alternative serialization formats for encoding structured data over HTTP would be desirable, e.g. to reduce processing time and save energy, reduce payload, etc.

- Study whether and how to support backward compatibility, co-existence and interworking of different types of NFs (such as legacy NFs, evolved NFs, new NFs) in 6GS with 5G NFs in 5GS

- Study whether and how to aid efficient discovery of NFs, limit/reduce NF signalling, NF interdependencies, information exposure between NFs, etc.

- Study whether it is suitable to extend selected 5G P2P interfaces, such as N4 or N26, as service-based interfaces.

NOTE X:	It is assumed to coordinate/split the work with CT4.

NOTE Y:	N26-like interfaces have a dependency on interworking assumptions in WT#2.

**** End of Changes ****