3GPP TSG-SA WG2 Meeting #170	S2-2507496

Gothenburg, Sweden, 25 – 29 August 2025							(revision of S2-2506550)

Source:	Ericsson, Apple?, Huawei?, HiSilicon?, Samsung?, Qualcomm Incorporated?, Nokia?, Google?

Title:	Key Issue WT#3: Support of SMS delivery to emergency centre via NAS

Document for:	Approval

Agenda Item:	20.8.1

Work Item / Release:	FS_SMS2EC_ARC / Rel-20

Abstract of the contribution: This contribution proposes one key issue for WTs in agree SID.

# 1.	Discussion

The agreed SID for the support for SMF to emergency centre in SP-250837 has following aspects, copied below.

# 4 Objective The study item aims at investigating how to specify architectural and procedural enhancements to support Short Message Service to Emergency Response Centre in both EPS and 5GS, based on the requirements specified in TS 22.101. The study includes the following work tasks: - WT-1: How to identify a Short Message to Emergency Response Center based on the use of emergency numbers. Address aspects when serving PLMN does not support this feature, but UE supports it. - WT-2: Whether and how to identify the type of emergency service (police, ambulance, fire brigade, etc.) using the same identification mechanism as for emergency calls. - WT-3: How to route a Short Message to Emergency Response Center (i.e. PSAP) serving the UE’s location, according to the local regulations (in roaming case, the regulations applicable to the UE’s location). NOTE 1: Depending on the type of emergency service, Short Messages can be routed to different emergency response centres. NOTE 2: This study assumes that Short Messages initiated by the Emergency Response Center in response to the Short Message to the Emergency Response Center are treated as regular Short Messages. For EPS and 5GS, 3GPP access (including TN and NTN) is in scope. UE in limited service state will not be in scope of this study item. The architectures for SMS over NAS (see TS 23.272, TS 23.401, TS 23.501, TS 23.502) and SMS over IP (see TS 23.204), and the architectural principles of Emergency Services (see TS 23.167) will be used as a basis. SMS over IP can be applied for both 3GPP access and non-3GPP access (either over EPS or 5GS).

It is proposed to separate the key issues for NAS based SMS delivery and IP (including IMS) based SMS delivery based on WT#3 in the SID.

This paper also consider the merge from following documents.

| S2-2507068 | 23.700-65: New Key Issue for WT#3: Routing of Short Message to Emergency Response Centre | Apple |
| --- | --- | --- |
| S2-2507131 | 23.700-65: 23.700-65 KIs for Work Task 3: SMS2EC routing | Huawei, HiSilicon |
| S2-2507203 | 23.700-65: Key issue for WT#3: Support of routing of SMS to EC based on UE location | Samsung |
| S2-2506498 | 23.700-65: New KI: WT3 Routing of Short Message to Emergency Response Center | Qualcomm Incorporated |
| S2-2506434 | 23.700-65: Key Issues for Short Message Service to Emergency Response Centre | Nokia |
| S2-2506550 | 23.700-65: Key Issue WT#3: Support of SMS delivery to emergency centre via NAS | Ericsson |
| S2-2506882 | 23.700-65: New Key Issue of SMS over NAS to Emergency Response Center | Google |

# 2.	 Proposal

It is proposed to update TR 23.700-65 with below changes:

* * * Start of change * * * *

## 5.x	Key Issue #X: Support of SMS delivery to emergency centre via NAS

### 5.x.1	Description

To support the SMS delivery to emergency centre via NAS based on local regulations, following aspects are included:

-	Investigate the gaps to route the SMS via NAS procedures for the support of SMS delivery to local emergency centre, e.g. UE location determination.

NOTE 1: Identification of SMS delivery to emergency centre is based on the conclusion of other KIs.

-	Provide solutions for the identified gaps to support the routing of the SMS delivery to local emergency centre via SMS service centre of the serving PLMN.

-	How to support emergency centre callback for sending short messages using regular SMS delivery service.

-	The key issue covers SMS delivery via NAS both in EPS and 5GS, including the support of SBI-based SMS interfaces.

NOTE 2: The solutions for EPS and 5GS are based on same principles.

-	The key issue covers both roaming and non-roaming scenarios.

* * * End of change * * * *