3GPP TSG-SA WG2 Meeting #170	S2-2507629

Goteborg , SE, Aug 25 – 29, 2025

Source:	MediaTek Inc., vivo, OPPO

Title:	KI#2: New Sol: CST reduction by pre-configuring SDP parameters

Document for:	Approval

Agenda Item:	20.1.1

Work Item / Release:	FS_5GSAT_Ph4-ARC / Rel-20

Abstract of the contribution: The contribution proposes a solution for call setup time reduction by pre-configuring SDP parameters.

Introduction

This solution aims to reduce the call setup time by introducing the “pre-configuration procedure” for SDP parameters in SIP Registration Procedure,by pre-call SIP message flow or by using the pre-defined SDP parameters.

Discussion

By introducing the “pre-configuration procedure” for SDP parameters, the SDP negotiation can be done “earlier”, i.e., negotiate during SIP Registration Procedure or negotiate by pre-call SIP message flow or by using the pre-defined SDP parameters.

With this procedure, number of messages and message size of later Call Setup can be reduced. This can optimize the overall CST (Call Setup Time). Once this procedure is completed, the SDP parameters would be refered in all the following calls, we may regard this “early” SDP negotiation procedure as an SDP pre-configuration procedure.

Additionally, compression method for SIP messages can be applied to further reduced the message size, such as "SigComp", "UDC". Then the compression efficiency of SIP message is around 2:1 (basic value), thus 0.5*SIP message size is the estimated SIP size after compression.

Proposal

A solution is proposed for KI#2 for incorporation in the FS_5GSAT_Ph4-ARC TR23.700-19.

* * * * Start of Change (all new texts) * * * *

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 |
| X |  | X |  |  |

* * * Next Change (all new texts) * * * *

6.X	Solution #X: CST reduction by pre-configuring SDP parameters

6.X.1	High level principles

This solution addresses Key Issue #2: "IMS Enhancements for GEO NB-IoT NTN Access".

This solution aims to reduce the Call Setup Time by introducing SDP pre-configuration procedure (e.g., during SIP registration procedure) to early configure SDP parameters for later SIP calls. There can be several options to achieve early configure SDP parameters for later SIP calls:

Option 1: Pre-configuration during SIP registration.

Option 2: Pre-configuration by pre-call SIP message flow.

Option 3: Using pre-define SDP parameters.

6.X.2	Description

This solution aims to reduce the Call Setup Time by introducing SDP pre-configuration procedure (e.g., during SIP registration procedure or by pre-call SIP messages) to early configure SDP parameters for later IMS calls. Since the network resource is limited when using GEO satellite access, it’s considered that there might be only few options for SDP parameters (such as for codec capability, parameters for media transfer…) and the SDP parameters can be simple and semi-fixed within the same IMS registration period.

6.X.3	Procedures

6.X.3.1	Option 1: Pre-configuration during SIP registration

Figure 6.X.3.1-1: Registration Procedure with “SDP pre-config” feature

Figure 6.X.3.1-1 is a signaling diagram for UE using GEO satellite access to successfully activate the “SDP pre-negotiation” mechanism, which includes the following steps:

0.	The UE is ATTACHED to a PLMN over GEO via NB-IoT NTN connecting to EPC and has established IMS PDN Connection. The UE acquires the IP address of the UE and the P-CSCF during the IMS PDN Connection establishment procedure.

Editor’s Note: How to ensure that the P-CSCF selected by the UE supports SDP pre-configuration is FFS.

1.	The UE sends a SIP REGISTER message, which includes a “SDP pre-config” feature tag to indicate that the UE support “SDP pre-configuration”, to the IMS CN.

2.	The P-CSCF supports “SDP pre-configuration” and determines to use the "SDP pre-configuration" mechanism and responds with a 401 Unauthorized message that also includes the"SDP pre-config" feature tag.

3.	The UE sends a (encrypted) REGISTER message, which includes an XML body containing the requested SDP parameters of UE side (e.g., IP address, port, and voice codec), to the P-CSCF.

4.	The P-CSCF selects the final SDP parameters, saves it in the P-CSCF and responds with a 200 OK message that also includes an XML body containing the pre-configured SDP parameters of P-CSCF side

5.	The pre-configured SDP parameters is saved in both the UE and P-CSCF sides, and the pre-configured SDP parameters determined during previous steps (i.e., during SIP Registration Procedure) will be used in all the following IMS calls during the IMS registration lifecycle

Figure 6.X.3.1-1a: Example of SDP negotiation during SIP registration procedure

Figure 6.X.3.1-1a is a more detailed SDP negotiation example during SIP registration procedure.:

6.X.3.2	Option 2: Pre-configuration with pre-call SIP message flow

Figure 6.X.3.2-1: Pre-configuration Procedure with pre-call SIP message flow

Figure 6.X.3.2-1 is a signaling diagram for UE using GEO satellite access to successfully activate the “SDP pre-configuration” mechanism with pre-call SIP message flow. , which includes the following steps:

0.	The UE is ATTACHED to a PLMN over GEO via NB-IoT NTN connecting to EPC and has established IMS PDN Connection and registered to IMS. The UE acuiqres the IP address of the UE and the P-CSCF during the IMS PDN Connection establishment procedure. The UE during the SIP registration procedure indicates its support of "SDP pre-config" and the IMS Server responds to accept to use the "SDP pre-config".

1.	The UE sends an SIP OPTION/INFO/INVITE message, which includes a “SDP pre-config” feature tag to indicate that this SIP message is aim to do the “SDP pre-configuration” and a SDP offer containing the requested SDP parameters of UE side (e.g., IP address, port, and voice codec), to the P-CSCF.

Editor’s note: Which SIP message to use for pre-configuration is FFS.

2.	The P-CSCF supports “SDP pre-configuration” and selects the final SDP parameters, saves it in the P-CSCF and responds with a 200 OK message that includes SDP answer containing the pre-configured SDP parameters of P-CSCF side. When the pre-configuration is done by SIP INVITE message, the UE reponds an ACK.

3.	The pre-configured SDP parameters is saved in both the UE and P-CSCF sides, and the pre-configured SDP parameters determined during previous steps will be used in all the following IMS calls during the IMS registration lifecycle.

NOTE 1:	SIP message pre-configuration can be extended to support adding and modification of pre-configured media configurations.

Figure 6.X.3.2-1a: Example of SDP offer/answer in pre-call SIP message flow

Figure 6.X.3.2-1a is a more detailed SDP offer/answer example in pre-call SIP message flow.

6.X.3.3	Option 3: Using pre-define SDP parameters

For the UE using GEO satellite access, the SDP parameters are pre-defined. The following call initiation can based on the pre-defined SDP. That is, the UE can sent SIP INVITE without the SDP and the P-CSCF can help to restore the SDP for GEO satellite access.

6.X.3.4	Call Set Up Procedure after SDP pre-configuration procedure

Figure 6.X.3.4-1: MO Call Setup Procedure with SDP pre-configuration applied

Figure 6.X.3.4-1 is a signaling diagram of a UE using GEO satellite access makes an MO IMS call, which includes the following steps:

0.	The UE successfully finished the IMS registration procedure and SDP parameters are pre-configured by Option 1(see Figure 6.X.3.1-1), Option 2(see Figure 6.X.3.2-1) or Oprion 3.

1.	The UE using GEO access initiates an IMS call and send an SIP INVITE without SDP or with the pre-configured SDP.

2~3.	The P-CSCF restores the pre-configured SDP parameters to trigger call initiation and forward the INVITE with SDP offer to IMS CN.

4~5.	IMS CN sends 100 Trying and 183 Session Progress with SDP answer to the P-CSCF.

6.	Optionally, the Dedicated EPS Besrer Context for voice is activated.

NOTE 1:	Step 6 is optional and based on operator’s policy.

7~9.	The P-CSCF sends PRACK to the IMS CN and receives the SIP 200 OK for PRACK. P-CSCF sends UPDATE to the IMS CN and the IMS CN replies 200 OK for UPDATE. The P-CSCF receives 180 Ringing from IMS CN. The P-CSCF then sends 180 Ringing to the UE.

10~11.	IMS CN sends SIP 200 OK for INVITE to the P-CSCF. The P-CSCF sends SIP 200 OK to the UE with the pre-configured SDP or without SDP. The P-CSCF receives ACK from the UE and sends it to the IMS CN.

12.	The media paths between UE and IMS CN have been established.

Figure 6.X.3.4-2: MT Call Setup Procedure with SDP pre-configuration applied

Figure 6.X.3.4-2 is a signaling diagram of a UE using GEO satellite access accepts a MT IMS call, which includes the following steps:

0.	The UE successfully finished the IMS registration procedure and SDP parameters are pre-configured by Option 1(see Figure 6.X.3.1-1), Option 2(see Figure 6.X.3.2-1) or Oprion 3.

1.	IMS CN sends an INVITE with SDP to the P-CSCF.

2.	The P-CSCF can removes the SDP in the INVITE since the SDP parameters have been pre-configured or keep the SDP.

3.	The P-CSCF sends 100 Trying to the IMS CN.

4~5.	The P-CSCF restores the pre-configured SDP parameters and sends 183 Session Progress with SDP answer to the IMS CN.

5.	Optionally, the Dedicated EPS Besrer Context for voice is activated.

NOTE 1:	Step 5 is optional and based on operator’s policy.

6~7.	IMS CN sends PRACK to the P-CSCF and receives the SIP 200 OK for PRACK. P-CSCF receives UPDATE from the IMS CN and the P-CSCF replies 200 OK for UPDATE.

8.	The UE sends 180 Ringing to the P-CSCF. P-CSCF sends it to the IMS CN.

9~10.	The UE answers the call and sends SIP 200 OK for INVITE to the P-CSCF. The P-CSCF sends SIP 200 OK to the IMS CN. The P-CSCF receives ACK from the IMS CN and sends it to the UE.

11.	The media paths between UE and IMS CN have been established.

NOTE 2:	This solution focus on the SDP pre-configuration flow. For the support from IMS network for transcoding and B2BUA service when either MO or MT UE is connecting via GEO is needed but not discussed in this solution and is assumed to be solved by other solutions.

Editor’s Note: It’s FFS whether this solution compliant to RFC.

6.X.4	Message size estimation of existing flow and SDP Pre-configuration flow

This clause describes comparison on the estimated message size of Option 1, Option 2 and existing flow.

Table 6.X.4-1 provides the estimated size increment of additional tag and XML body in SIP REGISTER for SDP Pre-configuration during IMS registration in Option 1.

| Message | Delta size for SDP Pre-configuration(byte) |
| --- | --- |
| REGISTER | 20 |
| 401 Unauthorized | 20 |
| REGISTER | 295 |
| 200 OK | 235 |
| Total | 570 |

Table 6.X.4-1: Estimated delta size for IMS registration flow with SDP pre-configuration for Option 1

Table 6.X.4-2 provides the estimated message size of SDP Pre-configuration flow with  pre-call SIP messages in Option 2. Take SIP INFO flow for example:

| Message | Estimated SIP message size (header+SDP) (byte) |
| --- | --- |
| INFO/OPTION/INVITE | 500 |
| 200 OK | 255 |
| Total | 755 |

Table 6.X.4-2: Estimated size for SDP pre-configuration with dedicated SIP messages in Option 2

Table 6.X.4-3 provides the estimated message size for existing flow and the message size applied SDP Pre-configuration flow optimization. By applying SDP pre-configuration and the flow without pre-condition, the total message size for call set up can be reduced by 84% which is from 12678 bytes to 2046 bytes. Additionally, message compression can be applied simultaneously, such as "UDC" or "SigComp", to further reduce the CST.

| Message | Existing Flow (byte) | With SDP Pre-configuration Flow (byte) |
| --- | --- | --- |
| INVITE | 2,441 | 1121 |
| 100 Trying | 379 | NO need |
| 183 | 1,601 | NO need |
| PRACK | 1,120 | NO need |
| 200 OK (PRACK) | 530 | NO need |
| UPDATE | 1,929 | NO need |
| 200 OK (UPDATE) | 1,494 | NO need |
| 180 Ringing | 925 | 925 |
| Total | 12,678 | 2046 |

Table 6.X.4-3: Estimated message size for Call Setup Procedure with SDP Pre-configuration flow

Editor’s Note: It’s FFS whether to use other compression method, e.g., SigComp, UDC (Uplink Data Compression)…etc. to further reduce CST.

Editor’s Note: The number used for calculation is FFS.

6.X.5	Impacts on services, entities and interfaces

Editor’s Note: Any further impacts are FFS..

Option 1:

UE:

-	Support SDP pre-configuration procedure in IMS Registration flow

P-CSCF:

-	Support SDP pre-configuration procedure in IMS Registration flow

-	Support storing pre-configured SDP

Option 2:

UE:

-	Support SDP pre-configuration procedure after registered to IMS NW.

-	Support SDP pre-configuration using SIP INVITE/INFO/OPTION

P-CSCF:

-	Support SDP pre-configuration procedure by SIP INVITE/INFO/OPTION

-	Support storing pre-configured SDP

Option 3:

UE:

-	Support storing pre-defined SDP parameters for GEO satellite access

P-CSCF:

-	Support storing pre-defined SDP parameters for GEO satellite access

* * * * End of Change * * * *