3GPP TSG-SA WG2 Meeting #170	S2-2507666

Goteborg, Sweden, Aug 25 – 29, 2025                                                  (was S2-2506613)

Source:	Nokia

Title:	KI#2 new solution – Dynamic service adjustments based on Energy Behaviour Analytics

Document for:	Approval

Agenda Item:	20.4.1

Work Item / Release:	FS_EnergySys_Ph2 / Rel-20

Abstract of the contribution: The contribution proposes a new solution for KI#2 considering the energy consumption inputs from NWDAF.

1. Discussion

The proposed solution aims to implement dynamic service adjustments based on the learning outcomes from Energy Behaviour output in UE analytics of NWDAF.

2. Proposal

It is proposed to agree to have the following text in 3GPP TR 23.700-67.

# * * * * First change * * * *

# 6	Solutions

## 6.0	Mapping of Solutions to Key Issues

Editor's note:	The number of columns of Key Issues will be updated according to clause 5.

Table 6.0-1: Mapping of Solutions to Key Issues

| Solution# | 1 | 2 | 3 |
| --- | --- | --- | --- |
| X |  | x |  |

## 6.X	Solution #X: Dynamic service adjustments based on Energy Behaviour Analytics

### 6.X.0	High-level solution principles

PCF considers the following factors to determine the UEs and PDU Sessions for carrying out service adjustments.

- NWDAF considers inputs related to EC from EIF while generating the existing UE related analytics described in TS 23.288. These outputs are consumed by the PCF for policy decisions for implementing service adjustments.

### 6.X.1	Description

NWDAF collecting inputs from UE communication and mobility analytics along with EC information from EIF and flagging the UEs into corresponding Energy Behaviour categories in the existing UE analytics. Classifying the UEs into different Energy Behaviour categories enables to understand how the UEs are behaving in context to Energy Consumption and accordingly implement policy and charging decisions.

PCF receives existing analytics of the UE from NWDAF which also included the Energy considerations and categories. Operator configures differentiated polices at the PCF based on the analytics information received; and accordingly, the PCF carries out the service adjustments which may result in changes in the AM, UE or SM policies for the UE and/or PDU Sessions.

Editor's note:	Whether and how to consider Energy information in communication with CHF pertaining to Service Adjustments is FFS.

### 6.X.2	Procedures

#### 6.X.2.0	General

Existing procedures are re-used to implement the service adjustments. However, PCF uses the Energy Behaviour Analysis for policy decisions.

#### 6.X.2.1	NWDAF -  Update of existing Analytics

NWDAF gets the inputs from 5GC NF(s) including EIF, and updates the existing analytics in UE Communication, mobility analytics and provides the Energy Behaviour as outcome in UE related analytics.

| Information | Description |
| --- | --- |
| EIF output | Energy exposure information from EIF |
| UE communication analytics | As per 23.288 |
| UE mobility analytics | As per 23.288 |

Energy Behaviour Inputs

Based on the inputs received, the NWDAF maps the UE to Energy Behaviour category (for e.g., Green, Orange, Red or High, Low, etc.). These values may indicate as very healthy if “Green”, average if “Orange” and very high consumer if “Red”.

The analytics outcome provided by the NWDAF include the Energy Behaviour in the outcome of the existing UE analytics as defined in 23.288.

### 6.X.3	Impacts on services, entities and interfaces

PCF:

-	Performs policy control considering Energy Behaviour outputs from the existing UE analytics as the inputs for policy decisions.

NWDAF:

-	NWDAF receives the Energy Information from the EIF.

-	NWDAF implements Energy Behaviour output and classifies the UEs into Energy Behaviour categories in the existing UE Analytics listed above.

# * * * End of changes * * * *