SA WG2 Meeting #170	S2-2507407

25 - 29 August, 2025, Goteborg, Sweden

Source:	Orange

Title:	KI#2 new solution: Conditional establishment of PDU Session based on energy criteria

Document for:	Approval

Agenda Item:	20.4.1

Study Item / Release:	FS_EnergySys_Ph2 / Rel-20

Abstract of the contribution: This pCR proposes a new solution for KI#2 of the Rel-20 FS_EnergySys_Ph2 study.

# Proposal

It is proposed to add the following candidate solution to TR 23.700-67.

*** Start of change ***

## 6.X	Solution #X: Conditional establishment of PDU Session based on energy criteria

### 6.X.0	High-level solution principles

This solution proposes a mechanism to conditionally establish data sessions based on energy-related criteria:

-	The solution enables postponing non-urgent data transfers to times when network energy conditions are more favorable (e.g., higher renewable energy availability, better energy efficiency).

-	The UE may include energy-related criteria in its PDU Session establishment request to indicate conditions under which the session should be established.

-	The SMF evaluates these energy criteria and either establishes the session immediately, rejects it, or delays establishment until the criteria are met.

-	The UE may specify a maximum delay tolerance for session establishment, allowing the network to optimize energy usage while respecting time constraints.

### 6.X.1	Description

This solution addresses Key Issue #2, specifically supporting service adjustments for the UE to enable energy saving by taking energy-related information into account to determine traffic handling.

This solution introduces a mechanism that allows UEs to specify energy-related criteria that should be satisfied before establishing a PDU Session. This approach is particularly beneficial for non-urgent data transfers such as:

-	Software updates,

-	Media content downloads for offline viewing,

-	Email attachments or large files in messaging applications,

-	Sensor data collection and uploads,

-	Backup operations.

When a UE needs to establish a PDU Session for such non-urgent data transfers, the UE includes one or more energy criteria in its PDU Session establishment request. These criteria may include:

-	a minimum energy efficiency threshold (e.g., bits/joule) at the serving gNB,

-	a maximum carbon intensity threshold (e.g., gCO2/kWh) at the serving gNB.

Additionally, the UE may specify a maximum delay tolerance for the session establishment, indicating how long it is willing to wait for favourable energy conditions before the session must be established regardless of energy criteria.

Upon receiving such a request, the SMF evaluates the current energy conditions by consulting the EIF. If the criteria are met, the SMF establishes the session immediately. If not, the SMF may either:

-	reject the request if no delay tolerance is specified, or

-	predict when the energy criteria will be met (within the specified delay tolerance) and inform the UE of this predicted time.

In the latter case, the UE should then send a new PDU Session establishment request at the predicted time, allowing the network to establish the session under more favourable energy conditions.

### 6.X.2	Procedure

Figure 6.X.2-1: Conditional establishment of PDU Session based on energy criteria

1.	The UE determines that it needs to establish a PDU Session for non-urgent data transfer (e.g., downloading content for offline viewing, uploading collected data, etc.).

2.	The UE sends a PDU Session Establishment Request to the AMF, including energy criteria and (optionally) maximum delay tolerance.

3.	The AMF forwards the request to the SMF in a Nsmf_PDUSession_CreateSMContext Request.

4-5. The SMF queries the EIF for current energy conditions.

6.	The SMF determines whether the current energy conditions meet the specified criteria. If the energy criteria are met, the procedure continues at step 12.

7.	If the energy criteria are not met:

-	If no delay tolerance was specified, the SMF rejects the PDU Session establishment request with a cause value indicating "Energy criteria not met" and the procedure ends.

-	If a delay tolerance was specified, the SMF predicts whether and when the energy criteria will be met within the delay tolerance period. If a suitable time is predicted, the SMF rejects the request but includes the predicted time in the response.

Editor’s Note: Details for predicting whether and when the energy criteria will be met (e.g. using EIF and/or NWDAF) are FFS.

8.	The UE receives the response.

9.	If a predicted time is included, the UE schedules a new PDU Session establishment request for that time.

10-11. At the predicted time, the UE sends a new PDU Session Establishment Request (which may or may not include energy criteria again).

12-13. The SMF establishes the PDU Session following the normal procedure.

### 6.X.3	Impacts on existing services, entities and interfaces

The solution has the following impacts:

UE:

-	Indicate energy criteria and (optionally) maximum delay tolerance in PDU Session requests.

-	Handle PDU Session establishment reject message with a new cause value indicating "Energy criteria not met"

-	Schedule a new PDU Session establishment request at the predicted time indicated in the PDU Session establishment reject message.

AMF:

- Forward energy criteria and delay tolerance parameters from UE to SMF.

Nsmf_PDUSession_CreateSMContext service:

-	Handle conditional PDU Session creation based on energy criteria.

Editor’s Note: This solution needs to be modified to remove UE impacts, according to the scope of this TR.

*** End of change***

NOTE: To edit the figure, right-click on it  Display replacement text. Copy the text (PlantUML code), modify it and use e.g. http://www.plantuml.com to build the updated figure.