3GPP TSG-SA WG2 Meeting #170	S2-2507675

# Goteborg, Sweden, August 25 - 29, 2025                      (Revision of S2-2506885)

Source: 	InterDigital Inc

Title: 	KI #3, New Sol: N3IWF and TNGF Reselection

Document for: 	Approval

Agenda Item: 	20.4.1

Work Item / Release:	FS_EnergySys_Ph2 / Rel-20

Abstract of the contribution: This contribution is a Key Issue #3 solution for N3IWF and TNGF Reselection.

# 1	Discussion

In the Release 19 5G System, the UE can be configured with the following information for untrusted non-3GPP access:

- an N3IWF identifier configuration which contains the FQDN or IP address of the N3IWF in the HPLMN, and

- Extended Home N3IWF identifier configuration which contains one or multiple tuples of FQDN/IP address of the N3IWF in the HPLMN and the S-NSSAIs supported by this N3IWF.

The UE can use the configured information to select an N3IWF identifier or the UE can construct an N3IWF identifier based on the tracking area where the UE is currently located.

As described in clause 4.12.2.2 of TS 23.502, when a UE attempts to register with the 5GC via an N3IWF, the AMF may reject the registration attempt and the AMF may provide target N3IWF information (FQDN and/or IP address) to the UE within the Registration Reject message. For example, the AMF may determine that the UE selected N3IWF was not appropriate for the requested slices that the UE is allowed to access to.

The behaviour for trusted access is similar. Clause 4.12a.2.2 of TS 23.502 explains that, when a UE attempts to register with the 5GC via a TNGF, the AMF may reject the registration attempt and the AMF may provide target TNAN information (SSID, TNGF ID) to the UE within the Registration Reject message indicating the UE to build the NAI based on the TNGF ID. For example, the AMF may determine that the UE selected TNGF was not appropriate for the requested slices that the UE is allowed to access to.

This p-CR proposes a Key Issue #3 solution for N3IWF or TNGF reselection where the AMF can redirect the UE to a more energy efficient N3IWF or TNGF using the existing Registration Reject mechanism.

# 2	Proposal

It is proposed to capture the following solution in TR 23.700-67 v0.2.0.

First change (all new text)

## 6.x	Solution #X: N3IWF/TNGF Reselection Based on Energy Related Information

### 6.x.0	High-level solution principles

This Key Issue #3 solution proposes that the AMF can take Energy Priority Information or the Schedule of Energy Priority Information into account for N3IWF or TNGF reselection. The AMF can then use the Rel-19 mechanism of sending a Registration Reject message to the UE to redirect the UE to a more energy efficient N3IWF or TNGF.

NOTE:	Energy Priority Information and the Schedule of Energy Priority Information are defined in Solution #20. The Energy Priority Information indicates the priority to select a NF relative to other NFs of the same type considering the energy characteristics of the NF. The Schedule of the Energy Priority Information includes the value of Energy Priority information which applies at each time interval in the schedule

### 6.x.1	Description

This is a Key Issue #3 solution.

The solution proposes that when the AMF receives a registration request from a UE, via an N3IWF or TNGF, the AMF can obtain Energy Priority Information or a Schedule of Energy Priority Information for N3IWF(s) or TNGF(s) from the NRF (e.g. based on solution #20) and can use that information to decide if the AMF should redirect the UE to a different N3IWF or TNGF. The redirection would use the Release 19 mechanism which is to send a Registration Reject message that identifies the N3IWF or TNGF that the UE should register though.

The solution proposes that N3IWF and TNGF profiles are stored in the NRF. The N3IWF and TNGF profiles include Energy Priority Information or a Schedule of Energy Priority Information based on Solution #20. The AMF may obtain the profiles during the registration procedure or the profiles may be obtained before registration procedure and the information can be cached in the AMF.

### 6.X.2	Procedures

#### 6.X.2.1	N3IWF/TNGF Reselection Considering Energy Related Information

Figure 6.x.2.1-1 shows a procedure for how the AMF can redirect the UE to a more energy efficient N3IWF or TNGF.

Figure 6.x.2.1-1: N3IWF/TNGF reselection considering energy related information

- The UE sends a Registration Request to the AMF via a TNGF or N3IWF.

- The AMF queries the NRF for the NF profiles of N3IWF(s) or TNGF(s) that can serve the slices that UE is allowed to use. The AMF may have already cached the NF profiles for the N3IWF(s) or TNGF(s) from earlier queries. Based on the Energy Priority Information or a Schedule of Energy Priority Information in the NF profiles, the AMF decides that it would be more energy efficient if the UE used a different N3IWF or TNGF.

- The AMF sends a Registration Reject message. The Registration Reject message provides target N3IWF information (FQDN and/or IP address) to the UE within the Registration Reject message (per clause 4.12.2.2 of TS 23.502 [3]) or provides target TNAN information (SSID, TNGF ID) to the UE within the Registration Reject message indicating to the UE to build the NAI based on the TNGF ID (per clause 4.12a.2.2 of TS 23.502 [3]).

- Based on the information in the Registration Reject message, the UE sends a new Registration Request to the AMF via the more energy efficient N3IWF or TNGF.

### 6.X.3	Impacts on existing services, entities and interfaces

AMF:

-	obtains NF Profile information from the NRF for N3IWF(s) and/or TNGF(s).

NRF:

-	stores NF Profile information for N3IWF(s) and/or TNGF(s) and the NF Profiles include Energy Priority Information or a Schedule of Energy Priority Information.

-	sends NF Profile information to the AMF for N3IWF(s) and/or TNGF(s).

End of Changes