SA WG2 Meeting #170	S2-2507695

25 – 29 Aug, 2025, Goteborg , SE	was S2-2506287

Source:	T-Mobile USA

Title:	Architectural Requirements

Document for:	Agreement

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC/Rel-20

Abstract of the contribution: This contribution proposes some architectural assumptions/requirements for Sensing.

# 1.	Discussion

The following p-CR adds a few relevant architectural requirements that are misto ensure the solutions address these aspects.

# 2.	 Proposal

It is proposed to update the TR as follows

* * * Start of change * * * *

## 4.2	Architectural Requirements

-	The Sensing Service operations shall not interfere with delivering existing QoS targets and SLAs for communication services.

-	The Sensing Service shall support exposure of the sensing results to a Sensing Service consumer.

NOTE:	Some sensing consumers might require receiving sensing results in a timely fashion (with low latency) for certain usage scenarios.

-	The core network aspects of Sensing architecture need to be agnostic to 3GPP radio access technology (RAT) supporting sensing.

-	Sensing architecture needs to support environment and motion monitoring, object detection and tracking as defined in TS 22.137 [2].

-	The existing Core Network architecture specified in TS 23.501 [3] needs to be used as a starting point for this study.

Sensing ServiceEditor's note:	The scope of the sensing study will be further aligned with TSG RAN at TSGs #108, and therefore, the architecture requirements will be revisited once the final scope is determined.

-	Sensing needs to support reporting of one time, periodic or triggered events.

-    Sensing architecture should allow for caching and/or storage and/or retrieval of sensing data for use cases that require this functionality

-    Sensing architecture needs to support differentiation of sensing data in 5GC from other user plane traffic, for accounting / charging purposes

-	Sensing architecture needs to support privacy and confidentiality.

-	Potential privacy requirements if identified during the study will be considered for the study of the sensing architecture.

NOTE:	Privacy protection and other security aspects will be coordinated with SA WG3, and the related impact to architecture enhancement will be based on SA WG3 conclusion.

* * * End of change * * * *