3GPP TSG-WG2 Meeting #170	S2-2507704

August 25 – 29, 2025, Göteborg, Sweden		(revision of S2-2506345)

Source:	Ericsson

Title:	KI#5, a new solution: Baseline for exposing sensing results to an AF

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract of the contribution:

This contribution proposes a new solution for KI#5 that aims at providing a baseline for exposing sensing results to an AF. This solution is a revision of S2-2504945 that has not been handled at SA2#169.

# 1. Discussion

This document proposes a new solution for KI#5 that aims at providing a baseline for exposing sensing results to an AF.

# 2. Text Proposal

It is proposed to agree the following solution to 3GPP TR 23.700-14.

>>>> Start of Changes <<<<

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions | Key Issues |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | #1 | #2 | #3 | #4 | #5 | #6 |
| #1 | x |  | x | x | x |  |
| #2 |  | x |  |  | x |  |
| #3 | x | x | x | x | x | x |
| #4 | x |  |  |  |  |  |
| #5 | x |  |  | x |  |  |
| #6 | x | x | x |  |  |  |
| #7 | x |  |  |  |  |  |
| #8 |  | x |  |  |  |  |
| #9 |  | x |  |  |  |  |
| #10 |  | x |  |  |  |  |
| #11 |  | x |  |  |  |  |
| #12 |  | x |  |  |  |  |
| #13 |  |  | x |  |  |  |
| #14 |  |  | x |  |  |  |
| #15 |  |  | x |  |  | x |
| #16 |  |  | x |  |  |  |
| #17 |  |  | x |  |  |  |
| #18 |  |  |  | x |  |  |
| #19 | x | x |  |  | x |  |
| #20 |  |  |  |  |  | x |
| X | x |  |  |  | x |  |

>>>> Next Change (all new text)<<<<

## 6.X	Solution #X: Exposing Sensing Results to an AF

### 6.X.1	High-level solution principles

The solution is based on following principles:

-	AF invokes a sensing service request to an NEF. The NEF authorizes the AF, and, if authorized, selects a Sensing Core Function (SnCF). The SnCF is a new CN NF that handles the sensing service request in the core network.

-	The SnCF selects NG-RAN node(s) for the relevant Target Sensing Area of the authorized sensing service. The SnCF requests the selected NG-RAN node(s) to obtain the sensing data.

-	The NG-RAN node(s) provide sensing data to the SnCF for sensing result determination, if applicable, and/or for further result exposure towards the AF.

### 6.X.2	Impacts on the Key Issues

This solution addresses KI#5 aspect related to how the core network exposes sensing results to the sensing service consumer. However, the solution impacts also other key issue, and the impact could be the following:

-	KI#1: New core network functions are being considered – Sensing Core Function (SnCF) that handles sensing request in the core network, and NEF is used to expose the obtained sensing data and/or sensing results.

-	KI#2: AF authorization is performed by the NEF, while the SnCF performs sensing request specific authorization on the operation level.

-	KI#3: NEF selects an SnCF, the SnCF selects NG-RAN node(s) to perform sensing data collection.

-	KI#4: NG-RAN node(s) may determine the sensing result or transfer the sensing data to the SnCF for sensing result determination or sensing data exposure.

-	KI#6: SnCF includes the sensing operation specific parameters as inputs inside a sensing request to the selected NG-RAN node(s).

### 6.X.3.	Functional Description

The following scenario is considered for this solution. An application function (AF) invokes a sensing service to request the network to perform (i) object detection or (ii) object detection and tracking within the requested area. The AF may also request a specific form of the sensing result it expects to receive. In order to specify the form of the sensing results, the AF may include the Sensing Result Detail Level parameter in the request. Sensing Result Detail Level can take the following options, for example:

-	"Notify result only": for instance, in case of object detection, the network notifies the AF about whether an object has been detected in the requested area or not, i.e., Yes/No indication and no sensing data in any form is provided to the AF inside the notification;

-	"Notify result with additional data": for instance, in case of object detection, the network processes the sensing measurements and notifies the AF about the determined sensing result including some processed sensing data in accordance with the determined sensing result and in the form requested by the AF.

In this case, the AF acts as a sensing service consumer and a sensing service requester. The AF request is sent to the NEF.

The NEF selects a Sensing Core Function (SnCF) that will handle the sensing service request. The NEF invokes the corresponding sensing service operation to the selected SnCF. The SnCF selects a NG-RAN node(s) for the core network-requested sensing service and invokes the corresponding sensing service request(s).

The NG-RAN node(s) collects sensing measurements, apply a certain level of processing in order to further provide the sensing data to the SnCF. The NG-RAN, if capable, may also determine a sensing result. The SnCF may further use the NG-RAN provided sensing data to determine a sensing result. The SnCF provide the determined sensing result to the NEF for further exposure to the AF.

NOTE X1:	Roles and functionalities of the involved NFs (e.g. NEF, SnCF and NG-RAN) can change depending on the resolution of the ENs in the procedure, see clause 6.X.4.

### 6.X.4	Procedures

Procedure for sensing result exposure in cases when a sensing service consumer (e.g., AF) invokes the sensing service towards a core network is shown in Figure 6.X.4-1.

NEF

NG-RAN

Sensing service consumer (e.g. AF)

SnCF

1. Sensing service request (e.g. Nnef_SensingEvent_Subscribe)

2. AF authorization and, if authorized, SnCF selection

3. Sensing service request (e.g. Nsncf_SensingEvent_Subscribe)

4. Selection of NG-RAN node(s) to handle the authorized sensing service

5. Sensing service request to the selected NG-RAN node(s) with the relevant parameters

6. Determining the resources required for the sensing and obtaining sensing data

7. Sensing measurements processing by the NG-RAN

8. Providing the sensing data to the CN

9. Processing of the NG-RAN provided sensing data to determine a sensing result

10. Providing the determined sensing result to the NEF using e.g. Nsncf_SensingEvent_Notify

11. Exposing the sensing result to the sensing service consumer (AF) using e.g. Nnef_SensingEvent_Notify)

Figure 6.X.4-1: Procedure of sensing result exposure to an AF.

1.	The Sensing service consumer (e.g., AF) invokes a sensing service request (e.g., Nnef_SensingEvent_Subscribe) to the NEF. Inside the request the AF includes the following mandatory and optional parameters:

-	mandatory: AF Identifier, External Target Sensing Area information (e.g., geographical area information and, optionally, altitude information), Type of sensing to perform (e.g., SensingEventType = “Object Detection”, “Object Tracking”), sensing result detail level (e.g., "notify result only", "notify result with additional data");

-	optional: event reporting mode (e.g., one-time, event-triggered or periodic reporting), sensing result exposure parameters (e.g., time and/or spatial resolution of the processed sensing data, i.e. determined sensing result).

NOTE X3:	Other parameters that AF can include in a sensing service request can be added during the later phases.

NOTE X4:	The exact names of service operations and semantics can be determined in the later phase.

2.	The NEF performs the authorization of the AF for the sensing service.

NOTE X5:	Data used for the AF-requested sensing service authorization and other details on how the NEF authorizes the AF requesting the sensing service is outcome of KI#2.

Editor's note:	Whether the NEF or other core network NF (e.g., SnCF) needs to determine that the AF is allowed to perform the sensing in the requested Target Sensing Area is FFS and needs coordination with SA WG3.

If the AF request for sensing service is authorized, the NEF maps the External Target Sensing Area information in the AF request to a 3GPP-internal Target Sensing Area information and then discovers and selects an SnCF to handle the sensing service request.

NOTE X6:	How the NEF selects an SnCF is outcome of KI#3.

Editor's note:	 How Target Sensing Area information needs to be described is FFS and requires coordination with RAN WGs.

3.	The NEF invokes the corresponding sensing service request (e.g. Nsncf_SensingEvent_Subscribe) to the selected SnCF and provides the relevant sensing service parameters from the AF request in Step 1.

4.	The SnCF selects NG-RAN node(s) for the relevant Target Sensing Area of the authorized sensing service.

NOTE X7:	How the SnCF selects NG-RAN node(s) to handle the AF-requested sensing service is covered by solutions for KI#3 of this study.

NOTE X8:	How the SnCF authorizes the sensing service operation, if applicable, is the outcome of KI#2 of this study.

5.	The SnCF sends the sensing service request to the selected NG-RAN node(s). Inside the request, the SnCF includes, for instance, Target Sensing Area information, type of sensing to perform, relevant parameters from the AF request in Step 1, and other parameters derived by the SnCF for the authorized sensing service.

6.	Based on the received sensing service request from the SnCF, the NG-RAN node(s) starts obtaining sensing data.

NOTE X9:	How sensing measurements are performed based on the SnCF-provided sensing request parameters and how the NG-RAN node(s) obtain the sensing data are RAN WGs decisions. For instance, if the SnCF includes in the request the following parameters: SensingEventType = “Object Detection” and Event reporting mode = “periodic reporting”, the NG-RAN starts performing sensing measurements with determined periodicity (continuously) and required processing in order to provide (periodically or continuously) the obtained sensing data to the SnCF for a sensing result determination and exposure towards the sensing service consumer. However, if SensingEventType = “Object Detection” and Event reporting mode = “one-time reporting”, the NG-RAN makes the required sensing measurement and then provides (once) the obtained and processed sensing data to the SnCF.

7.	The NG-RAN node(s) apply processing to the obtained sensing measurements before providing the sensing data to the SnCF,

NOTE X10:	The form of sensing data provided by NG-RAN depends on level processing the NG-RAN applies to the sensing measurements and, thus, may include the determined result or pre-processed sensing data for subsequent processing at the SnCF for instance.

Editor’s note:	Whether NG-RAN nodes can perform a level of processing required to determine a sensing result (e.g., to detect an object) is RAN WGs decision and requires further coordination.

8.	The NG-RAN node(s) provides the sensing data (i.e., sensing measurements processed by the NG-RAN) to the SnCF.

NOTE X11:	How the NG-RAN node(s) provides the obtained sensing data to the SnCF is covered by KI#4 of this study.

9.	The SnCF may determine a sensing result by processing, if required, to the sensing data received from the NG-RAN node(s) and considering the parameters included in the AF request in Step 1.

NOTE X12:	How the SnCF uses the sensing data provided by the NG-RAN to determine a sensing result is up to implementation; the SnCF can cache the sensing data. However, sensing data processing/handling is limited to information provided by RAN.

10.	The SnCF provides the determined sensing result to the NEF using e.g. the Nsncf_SensingEvent_Notify service operation.

11.	The NEF exposes the sensing result to the AF via e.g., Nnef_SensingEvent_Notify service operation, considering the sensing result exposure parameters from the AF request in Step 1.

### 6.X.5	Impacts on existing services, entities and interfaces

AF:

-	acts as a Sensing Service Consumer by sending a sensing service request to the core network and consuming the sensing result.

Network Exposure Function (NEF):

-	performs sensing service consumer (e.g., AF) authorization;

-	selects a Sensing Core Function (SnCF) for handling the sensing service in the core network;

-	maps an External Target Sensing area information to an internal Target Sensing Area information;

-	exposes the sensing result to a sensing service consumer (e.g., AF).

Sensing Core Function (SnCF):

-	selects NG-RAN node(s) capable of sensing for a specific service;

-	may collect the sensing data from the NG-RAN node(s);

-	may determine a sensing result based on the inputs from NG-RAN node(s);

-	may cache and provide the sensing result to the NEF for exposure to the AF;

NG-RAN:

-	may obtain and process sensing measurements based on the sensing service request from the SnCF;

-	may provide the obtained sensing data to the SnCF;

>>>> End of Changes <<<<