3GPP TSG-WG SA2 Meeting #170	S2-2507754

# Goteborg, Sweden, 25 – 29 August 2025		(revision of S2-2507684, S2-2507340)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.369 | CR | 0063 | rev | 2 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to the Filtering Information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo, Huawei, LG Electronics, vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AmbientIoT-ARC |  |  |  |  |  | Date: |  |  | 2025-08-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 17) Rel-17 (Release 18) Rel-18 (Release 19) Rel-19 (Release 20) |  |
| Reason for change: |  | The description agreed during the SA#169 meeting to clause 5.8 has some ambiguities. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarifications are proposed to clause 5.8 to fix ambiguities. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear specification regarding filtering information. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

## 5.8	Filtering Information

The Filtering Information is used to identify or filter one or multiple AIoT Device(s). The Filtering Information is constructed by one or multiple filtering elements and each filtering element corresponds to a component (i.e. ID Type, PLMN Identifier, NID, Third Party Identifier or Identification Information) of the AIoT Device Permanent Identifier as defined in clause 5.7.

Each filtering element shall include:

-	Component Type Information indicating a component of the AIoT Device Permanent Identifier that is used to match the bitstring.

-	A bitstring which is used to compare with the component.

-	An Offset from the beginning of the indicated component of the AIoT Device Permanent Identifier, which indicates the start location in the indicated component to be used to compare with the corresponding bitstring.

When theComponent Type Information of a filtering element indicates a PLMN ID, NID or Third Party Identifier component, then the Offset is not included in the filtering element, i.e. the whole component is compared with the bitstring.

When the Component Type Information of the filtering element indicates an Identification Information component, the Offset is always included, i.e. the component from the start location indicated by the corresponding Offset is compared with the bitstring.

There may be multiple filtering elements corresponding to the component Identification Information of the AIoT Device Permanent Identifier.

NOTE 1:	It is assumed that the AIOTF includes at most one filtering element within the Filtering Information for component ID Type, PLMN ID, NID or Third Party Identifier of the AIoT Device Permanent Identifier. It is assumed that the AIoT Device does not validate the number of filtering element(s) for any indicated component within the Filtering Information.

NOTE 2:	The format of the Filtering Information is specified in clause 31.3 of TS 23.003 [6].

Editor's note:	Whether and how to secure the filtering information is up to SA WG3.

Figure 5.8-1: The example of the comparing the bitstring to the component

To determine whether an AIoT Device Permanent Identifier matches the Filtering Information, the bitstring of every filtering element within the Filtering Information is compared with the indicated component of the AIoT Device Permanent Identifier. If all the compared bitstrings match the corresponding component of the AIoT Device Permanent Identifier, then an AIoT Device Permanent Identifier matches the Filtering Information. If an AIoT Device Permanent Identifier does not contain an indicated component then it does not match the Filtering Information.

* * * * End of Changes * * * *