3GPP TSG-WG SA2 Meeting #170	S2-2507764

Gothenburg, Aug 25 – 29, 2025	(revision of S2-2506632)

Source:	Nokia

Title:	New solution: DO-A information transfer

Document for:	Approval

Agenda Item:	20.5.1

Work Item / Release:	FS_AmbientIoT_Ph2 / Rel-20

Abstract: a new solution for 5GC to support transferring DO-A traffic type.

# 1. Introduction/Discussion

A new solution is proposed to enable 5GC support transferring DO-A traffic type.

# 2. Text Proposal

It is proposed to approve the following changes in the new TR.

FIRST CHANGE (All text is new)

## 6.X	Solution #X: Enable 5G delivery AIoT device initiated traffic

### 6.X.0	High-level solution principles

The key technical principles proposed in this solution are summarized below:

- AIoT Device Profile Enhancement: The AIoT Device Profile is extended to include additional information, such as AF/NEF associations and the device’s registration status.

- Device Registration Requirement: Each AIoT Device must be registered with the 5G network to enable support for DO-A traffic.

- Energy-Aware Communication Initiation: An AIoT Device may initiate communication when sufficient energy is available from non-RF sources. The device can also request the network to supply energy for subsequent transmissions by indicating its Active Time.

- Redundant Traffic Management: The NG-RAN can identify and suppress redundant transmissions originating from multiple readers, indicating which reader is selected for subsequent forwarding. Readers maintain a local device context to prevent unnecessary traffic duplication.

- Per-Device Session Context at AIOTF: The AIOTF maintains a session context for each device, including session IDs that allow subsequent messages to bypass repeated authorization and NEF discovery, improving efficiency.

- Timestamp-Based Deduplication and Data Integrity: Each message carries a timestamp to enable detection and elimination of duplicate data, supporting network-side data cleaning and ensuring consistency.

NOTE :	The feasibility of using timestamp depends on the RAN WG.

- NEF Selection Based on Device Profile: The AIOTF determines the appropriate NEF for forwarding traffic based on the AIoT device profile and the requested AF.

### 6.X.1	Description

The introduction of Device Oriented-Autonomous (DO-A) traffic type addresses a significant gap in AIoT networks, where devices initiate communication autonomously based on available energy resources. This capability was not supported in previous releases and is now within the scope of this Release. The proposed solution outlines a series of procedures that enable efficient handling of DO-A traffic, ensuring optimized network resource utilization.

Key Procedures:

- AIoT Device Profile enhancement: the AIoT device profile stored in the ADM is enhanced to also include  supported AF IDs along with their corresponding NEF IDs, and the AIoT device registration status.

- Registration and Context Creation: AIoT devices register with the network by sending their device ID, which is encapsulated and forwarded through network elements for security validation. Upon successful authentication, the device profile is updated, and a context is created, detailing registration status and access permissions.

- DO-A Traffic Initiation: When an AIoT device has sufficient energy, it autonomously initiates communication by sending a DO-A request message. This message includes device identification, a timestamp for data management, and an Active Time parameter indicating communication duration capability.

- Traffic Routing and Optimization: The network elements, including AIoT readers and NG-RAN, manage traffic routing, ensuring efficient handling of DO-A messages. Duplicate data is identified and managed using timestamps, and session IDs are allocated for ongoing communications.

- Authorization and Forwarding: The AIOTF verifies device context and authorizes traffic based on stored profile data. It forwards authorized traffic to the appropriate NEF, which then communicates with the requested AF.

- Session Management: Subsequent communications utilize allocated session IDs, streamlining authorization processes and enabling direct traffic forwarding to known NEFs.

This proposal aims to enhance AIoT network functionality by supporting autonomous device communication, optimizing energy use, and improving network efficiency.

### 6.X.2	Procedures

Step 0: The AIoT device profile per an AIoT device is provisioned in the ADM (AIoT specific UDM). The AIoT device profile includes the following parameters:

- AIoT Device Permanent ID: Uniquely identifies the AIoT Device.

- Last known AIOTF information: Indicates the last known AIOTF that served the AIoT device, or unknown.

- AF ID together with NEF ID: Indicates which AFs the AIoT device is allowed to access and which NEFs are used for communication with the AFs.

- Registration status: Indicates if the AIoT device is already registered with the network or not.

Step 1: The AIoT device performs registration by sending a message containing its AIoT device ID to the AIoT reader. The reader forwards this message to the NG-RAN, which then passes it to the AIOTF for security validation. Upon successful authentication, the AIOTF updates the device profile status with the ADM, which responds with the updated profile data, including AF and NEF IDs, and provides a registration response message, which includes the registration result indicating whether it was successful or not to the AIoT device via the NG-RAN as well as the AIoT reader.

Step 2: The AIOTF generates the AIoT device context which includes whether it is registered, which AFs it can access, and which NEFs should be used to communicate with the AFs.

Step 3: When the AIoT device has something to send the network and has enough energy to do so, it initiates communication by sending the DO-A (Device Oriented-Autonomous) request message. This message includes the AIoT device ID (e.g., temporary ID, if it has been allocated by the network during the registration already) for the network to identify the AIoT device, timestamp indicating when this message was initiated, and the AIoT NAS message for the actual communication with the network and the AF (data consumer). The timestamp is later used by the network to perform data cleaning (e.g., removing data duplications). And the AIoT device ID and timestamp are not part of the AIoT NAS message to ensure they are understood by the AIoT reader and NG-RAN for proper traffic routing and optimization. The AIoT device also includes an Active Time parameter, which indicates how long the device can remain active to communicate with the network for subsequent messages. This Active Time can be zero if the AIoT device only had enough energy to send a single DO-A message.

Step 4: The AIoT reader forwards the AIoT NAS message encapsulated over RRC to the serving NG-RAN. Additionally, the AIoT reader may send its AIoT reader ID, and the received AIoT device ID and timestamp.

Step 5: The NG-RAN optionally checks if multiple AIoT readers are forwarding the same AIoT DO-A traffic from the same AIoT device by verifying the AIoT device ID, timestamp, and AIoT reader ID. This helps avoid sending the same traffic to the network multiple times unnecessarily.

NOTE :	Energy, Active Time and reader selection aspects depend on RAN WGs.

Step 6: In the RRC response message, the NG-RAN optionally includes the information indicating whether the AIoT reader is selected for the subsequent communications or not. Based on this indication, the AIoT reader can determine whether to keep forwarding the DO-A traffic from the same AIoT device. If it determines not to continue, it shall no longer forward any traffic received from the same A-IoT device for the same AIoT session. The selected AIoT reader locally stores the AIoT device context information including the Active Time.

Steps 7: The NG-RAN forwards the AIoT device ID, timestamp and the message carrying the AIoT NAS message to the AMF over the N2 UL message, and the AMF subsequently forwards the received AIoT NAS message to the responsible AIOTF over the SBI interface.

Step 8: The AIOTF verifies if an AIoT device context exists for the device identified by the AIoT device ID. If absent, it retrieves the AIoT device permanent ID and requests the ADM for the device profile data using this ID. Upon obtaining the profile data, the AIOTF authorizes the device to send DO-A traffic to the requested AF by matching the AF ID in the AIoT NAS message with those in the profile data. If duplicate data from the same device is received, the AIOTF retains only one instance based on the timestamp. It then allocates an AIoT session ID for future communications and updates the device context data as necessary.

Step 9: Based on the AIoT device profile data, the AIOTF determines which NEF to forward the AIoT DO-A traffic to and forwards it, including the AIoT device ID, data, and AF ID.

Step 10: The NEF forwards the received traffic to the requested AF, including the AIoT device ID and data.

Step 11: In response to the message in step 7, the AIOTF sends an message carrying the AIoT device AIoT device ID and the AIoT NAS message over the N2 DL message. The AIoT NAS message contains the allocated session ID.

Step 12: The NG-RAN forwards the message to the AIoT reader selected in step 5 over RRC.

Step 13: The AIoT reader provides the DO-A response message to the AIoT device, including the received AIoT NAS message. The AIoT reader provides energy separately or together with the response message if the Active Time stored in the AIoT device context information is shorter than the time required to send the response and the network needs further messages from the AIoT device.

Step 14: When the AIoT device wants to send additional data to the same AF it previously sent data to, the AIoT device includes the allocated session ID in the AIoT NAS message. This message is forwarded to the AIOTF using the same mechanisms explained in steps 4 to 7.

Step 15: When the session ID is included in the AIoT NAS message and is valid, the AIOTF may skip the authorization checking and NEF discovery processes.

Step 16: The AIOTF directly issues the notification message towards the known NEF.

Step 17: The NEF forwards the received traffic to the requested AF, including the AIoT device ID and data.

### 6.X.3	Impacts on Services, Entities and Interfaces

AIoT Device:

  - Energy Management: The AIoT device autonomously initiates communication based on its energy harvesting capabilities.

  - Timestamp support: The AIoT device provides time information indicating when the DO-A message was sent.

AIoT Reader:

  - The AIoT reader forwards DO-A messages.

  - It may provide energy to AIoT devices if needed.

  - The reader manages session continuity based on network indications.

AIOTF:

  - The AIOTF performs authorization checks to ensure devices are permitted to send DO-A traffic to specific AFs.

  - Allocation of session IDs for subsequent communications streamlines the process and reduces overhead for repeated authorizations.

  - Timestamp verification helps manage duplicate data.

ADM:

  - The ADM maintains detailed device profiles, including registration status and access permissions.

  - Efficient retrieval and updating of device profiles are necessary to support the proposed procedures.

NG-RAN:

  - The NG-RAN may need to implement mechanisms for selecting AIoT readers based on traffic patterns and device context.

END OF CHANGES