SA WG2 Meeting #170	S2-2507911

25 – 29 Aug, 2025, Goteborg , SE

Source:	Deutsche Telekom (pen holder), [Apple, AT&T, Boost Mobile Network, CATT, CEWiT, China Mobile, China Telecom, Cisco Systems, Echostar, Ericsson, ETRI, Futurewei, Google, HiSilicon, Huawei, IIT Bombay, InterDigital, KPN N.V., Lenovo, LG Electronics, MediaTek Inc., Nokia, Ofinno, OPPO, Qualcomm Incorporated, Rakuten Mobile, Samsung, SK Telecom, TCL, Telecom Italia, T-Mobile USA, vivo, Xiaomi, ZTE]?

Title:	[WT#1.1 NAS] Summary of KI on control aspects

Document for:	Approval

Agenda Item:	20.6.1.1

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution summarizes all contributions for WT#1.1 NAS regarding KI on Control aspects for communication service.

# 1. Contribution Overview

This contribution summarizes and attempt to merge all content regarding control aspects for connectivity services of the contributions AI 20.6.1.1 (NAS functionality).

## Contributions considered

Based on the Tdoc list provided in the TDoc_ordering folder.

| TDoc | Title | Sources |
| --- | --- | --- |
| S2-2506393 | 23.801-01: [WT#1.1] Support of Control Signalling in 6GS | OPPO |
| S2-2506465 | 23.801-01: [WT#1.1, control signalling] 6G NAS for connectivity and beyond connectivity services | China Telecom |
| S2-2506480 | 23.801-01: [WT#1.1] WT and KI for enhancement of NAS control signalling | China Mobile |
| S2-2506505 | 23.801-01: [WT#1.1, all topics] Control Signalling Design for 6G | MediaTek Inc. |
| S2-2506517 | 23.801-01: [WT#1.1] NAS for 6G | Deutsche Telekom, AT&T, Cisco Systems, KPN N.V., Rakuten Mobile, SK Telecom, T-Mobile USA, Telecom Italia |
| S2-2506523 | 23.801-01: [WT#1.1] NAS functionality for 6G | Ofinno |
| S2-2506555 | 23.801-01: [WT#1.1, Non-Access Stratum Functionality] Support for NAS Signalling in the 6G Architecture | InterDigital |
| S2-2506577 | 23.801-01: [WT#1.1, NAS functionalities] Scope and New Key Issue for NAS functionalities | LG Electronics |
| S2-2506611 | 23.801-01: [WT#1.1, 6G NAS] Support of 6G New NAS | vivo |
| S2-2506752 | 23.801-01: [WT#1.1] Discussion and proposal on key issue for work task#1.1 | CATT |
| S2-2506775 | 23.801-01: [WT#1.1, NAS] Discussion and proposal for NAS | ZTE |
| S2-2506822 | 23.801-01: [WT#1.1] UE-Core Network interaction in 6G | CEWiT |
| S2-2506840 | 23.801-01: [WT#1.1, all topics] Support of control signaling for connectivity and beyond connectivity services | Samsung |
| S2-2506868 | 23.801-01: [WT#1.1, Control Signalling for 6G] Scope and Key Issue proposal for UE-Core Network interaction | IIT Bombay |
| S2-2506871 | 23.801-01: [WT#1.1, Non-Access Stratum] Scope and Key Issue for Non-Access Stratum | Futurewei |
| S2-2506903 | 23.801-01: [WT#1.1] Introduction of NAS functionality without impacting other NAS functionalities | Apple |
| S2-2506942 | 23.801-01: [WT#1.1, Control signalling for 6G] UE-CN control signalling communication | Ericsson |
| S2-2506975 | 23.801-01: [WT #1.1, 6G NAS] Discussion and key issues for distributed NAS | TCL |
| S2-2506988 | 23.801-01: [WT#1.1] Update to WT#1.1 for clarification on flexible addition of new services in 3GPP | Qualcomm |
| S2-2507055 | 23.801-01: [WT#1.1, all topics] Work task #1.1 Support of 6G Control plane signaling | Nokia, AT&T, T-Mobile US |
| S2-2507124 | 23.801-01: [WT#1.1, NAS] New NAS and Interaction between UE and Network | Huawei, HiSilicon |
| S2-2507135 | 23.801-01: [WT#1.1] Technical Scope of WT#1.1 - Control Signalling in 6GS | ETRI |
| S2-2507200 | 23.801-01: [WT#1.1, NAS] Key Issues on NAS and operator services in 6G | Lenovo |
| S2-2507353 | 23.801-01: [WT#1.1] KI on UE-CN interaction for control signalling for 6G | Xiaomi |
| S2-2507378 | 23.801-01: WT#1.1, Study proposal to enhance a NAS functionality | NEC |

## Summary of aspects proposed as KI

| TDoc | Sources | KI summary for connectivity services excluding UP-based |
| --- | --- | --- |
| S2-2506393 | OPPO | How to route the NAS signalling between UE and the according NFs with less NAS signalling routing delay and avoiding potential NAS signalling congestion via a centralized AMF The UE ID (e.g., 6G-GUTI) allocation and structure. Network function selection and UE context storage |
| S2-2506465 | China Telecom | Whether and how to design 6G NAS to meet the requirement of well-structured modularity between NAS functionalities |
| S2-2506480 | China Mobile | How to categorize NAS functionalities Support for flexible and efficient introduction mechanism of new NAS functionality without any impact on other NAS functionality. Study the approaches for UE and Core Network interaction e.g., NAS enhancement, Service based interface between UE and Core Network. |
| S2-2506505 | MediaTek Inc. | enable the introduction of a new non-access stratum functionality without impacting other non-access stratum functionalities identify a minimal set of non-access stratum functionalities that does not get impacted by additional non-access stratum functionalities develop generic mechanisms for UE-Core Network interaction to support operator services How to route control messages Impact on termination point of interfaces between UE and CN NAS architecture and system procedures Where and how to handle authentication, security management and integrity protection |
| S2-2506517 | Deutsche Telekom, AT&T, Cisco Systems, KPN N.V., Rakuten Mobile, SK Telecom, T-Mobile USA, Telecom Italia | introduction of a new non-access stratum functionality in a variety of deployment scenarios without impacting other non-access stratum functionalities, especially without impacting the AMF identifying a minimal set of non-access stratum functionalities that does not get impacted by additional non-access stratum functionalities, especially dependency on the AMF How NAS functionality can be specified independent of each other and minimizing dependencies between NFs How NFs providing NAS functionality can be flexibly and efficiently deployed What interactions between UE-and-CN, UE-and-RAN and RAN-and-CN are required |
| S2-2506523 | Ofinno | How to determine whether the UE/network supports 6G NAS functionality. How to identify/discover a NF which is supporting 6G NAS functionality. How to transport NAS messages between the UE and the network that are supporting 6G NAS functionality with minimum involvements of other existing NFs. How to minimize congestion/overhead in NAS signalling. How to support no or minimum impacts on other existing NFs, when the NF supporting 6G NAS functionality is used. How to support the security of the 6G NAS functionality (e.g. which NF (AMF, or else) will be responsible for NAS security, and corresponding procedures), based on coordination with SA3 Identify a minimal set of NAS functionalities to be supported for all UEs using general connectivity services, and the additional NAS functionalities to be supported for certain types of UEs (e.g. NB-IoT) How to support the 6G NAS functionality in roaming scenarios |
| S2-2506555 | InterDigital | Whether and how to redefine the functional split between the Access and Mobility Management Function (AMF) and other Core Network Functions (NFs). Whether and how to remove dependencies between NAS features in order to support independent evolution and deployment. Reduce NAS routing constraints by supporting direct routing of NAS messages to target NFs Whether and how the 6G system can support distributed NAS design principles to enable minimal configurations. Whether there is a need to define criteria for determining functional independence among NAS module Whether and how new NAS functionalities can be integrated dynamically without introducing backward compatibility or dependency issues on the core minimal set. Whether and how to allow for flexible UE-CN and RAN-CN interactions, potentially bypassing the AMF where appropriate. Whether and how to support diverse 6G deployment scenarios (e.g., distributed, NPNs, NTNs) through adaptable signalling paths and transport protocols. Whether and how to enable the dynamic provisioning of NAS functionalities and policies in a service-agnostic, operator-customizable manner |
| S2-2506577 | LG Electronics | Whether and how to enable the introduction of a new non-access stratum functionality without impacting other non-access stratum functionalities. Whether and how to identify a minimal set of non-access stratum functionalities that does not get impacted by additional non-access stratum functionalities. Whether and how to develop generic mechanisms for UE-Core Network interaction to support operator services |
| S2-2506611 | Vivo | How to support distributed UE-CN communication with direct NAS signaling connection between UE and 6G NFs, i.e. no single NAS signaling anchor. How to define a minimal set of NAS functionalities to support modular UE-CN communication |
| S2-2506752 | CATT | Whether and how to enable the introduction of a new non-access stratum functionality without impacting other non-access stratum functionalities. Whether and how to identify a minimal set of non-access stratum functionalities that does not get impacted by additional non-access stratum functionalities. Whether and how to develop generic mechanisms for UE-Core Network interaction to support different deployment requirements. |
| S2-2506775 | ZTE | Study whether and how to enable the introduction of a new non-access stratum functionality without impacting other non-access stratum functionalities Study whether and how to identify a minimal set of non-access stratum functionalities that does not get impacted by additional non-access stratum functionalities. |
| S2-2506822 | CEWiT | Enhancements to NAS for newly added functionalities supporting connectivity and beyond connectivity services in 6G without impacting the existing NAS functionalities Identify a minimal set of NAS functionalities suitable for all devices, including those with limited capabilities like low- powered IoT devices How efficiently and securely the NAS messages can be transmitted, with reduced complexity and signalling overhead |
| S2-2506840 | Samsung | Identify a minimal set of NAS functionalities (e.g., mobility management, session management) that does not get impacted by a beyond connectivity operator services. How to mitigate the impact on existing NAS functionalities when introducing a new NAS functionality |
| S2-2506868 | IIT Bombay | How to enable the 6G system to handle higher signalling load How to reduce service delivery latency in the 6G System Study the placement and functionalities of network functions to support such mechanisms |
| S2-2506871 | Futurewei | study whether and how to distribute NAS functionality such that handling routing of NAS signalling is separate from functionality handles basic registration and mobility management, and other functionality for enhanced services. study whether and how to enhance mechanisms and procedures for signalling for paging and reduce congestion in the control plane. |
| S2-2506903 | Apple | whether and how to maintain connectivity between UE and legacy network functions, such as SMF, PCF, SMSF, etc., while enabling different mechanisms for the UE to have communication with new network functions that will be introduced in 6G |
| S2-2506942 | Ericsson | Identify if there is any potential improvement on the support for the UE connectivity-based functionalities in addition to the re-use of existing basic 5GS NAS mobility and session management handling mechanisms. Investigate the UE-CN communication to support the UE interworking between 6G system and 5G system. Investigate the possibility of designing a new common UE and CN communication mechanism for any new services without dependency to the UE connectivity-based functionalities |
| S2-2506975 | TCL | How to improve RAN-Core network interface to connect UE directly to any core network functions without the help of AMF? How to support UE discover the core network functions to acquire services? How to route the UE uplink service requests? For downlink messages, how to keep UE reachable if there is no core network anchor such as AMF? How to support UE mobility management among multiple core network functions? How to synchronize UE mobility and location information among multiple core network functions? How to manage, transfer and synchronize UE contexts between network functions during UE mobility when the UE status is different on each NF? |
| S2-2506988 | Qualcomm | Whether and how to enable the introduction of a new non-access stratum functionality without impacting other non-access stratum functionalities. Whether and how to identify a minimal set of non-access stratum functionalities that does not get impacted by additional non-access stratum functionalities. Whether and how to develop generic mechanisms for UE-Core Network interaction to support operator services offered over the control plane. |
| S2-2507055 | Nokia, AT&T, T-Mobile US | Whether and how to enable the introduction of a new non-access stratum functionality without impacting other non-access stratum functionalities. Whether and how to identify a minimal set of independent non-access stratum functionalities depending on the type of typical user equipment (e.g. IoT vs eMBB). Whether and how to develop generic mechanisms (e.g. authorization, discovery) for UE-Core Network interaction to support operator services. |
| S2-2507124 | Huawei, HiSilicon | How to avoid impacts on other non-access stratum functionalities when introducing a new non-access stratum functionality for both connectivity services and beyond connectivity services. How to support flexible NAS distribution considering the termination of the NAS message for both connectivity services and beyond connectivity services. How to develop a generic NAS control mechanism for agile deployment of beyond-connectivity services, including an efficient discovery mechanism of network provided services (e.g., computing, sensing, etc.) to enable the UE to invoke these services when required. |
| S2-2507135 | ETRI | How to define functional building blocks and their composition rules. How to ensure modularity across control, user, and management planes. How to enable component-level reusability and versioning. Mapping of modular functions to deployment environments Whether and how new NAS functionalities can be introduced independently of existing ones. Whether and how to define a minimal baseline NAS profile that can remain unaffected by additional extensions. Definition of generic UE-CN interaction patterns for extensible NAS support. |
| S2-2507200 | Lenovo | Whether and how to identify a minimal set of NAS functionalities that does not get impacted by additional NAS functionalities. How to efficiently route NAS message between UE and control plane NFs; How to enable direct security procedure (e.g. authentication, authorization) between the UE and the target core network NF. |
| S2-2507353 | Xiaomi | the system is architecturally modular, and implementation modular with as little independency from each self-contained module as possible; the system supports control signalling for connectivity services and beyond connectivity services; the system supports UE-CN interaction for control signalling over control plane; the system supports UE-CN interaction for control signalling over user plane; the design UE-CN interaction should be conceived considering at least the following aspects: cloud native, sustainability and energy efficiency, robustness and resiliency, etc. Whether and how to further modulize connectivity services with self-contained modules which are independent from each other; Whether and how to further modulize beyond connectivity services with self-contained modules which are independent from each other whether and how to enhance UE-CN interaction for control signalling over control plane considering at least the following aspects: cloud native, sustainability and energy efficiency, robustness and resiliency; whether and how to enhance UE-CN interaction for control signalling over user plane considering at least the following aspects: cloud native, sustainability and energy efficiency, robustness and resiliency; whether and how to define the principle to support the modulized connectivity services and beyond connectivity services on UE-CN interaction via control plane or via user plane with potential enhancements in the related study in this WT |
| S2-2507378 | NEC | Whether and how to enable the introduction of a new non-access stratum functionality without impacting other non-access stratum functionalities Whether and how to identify a minimal set of non-access stratum functionalities that does not get impacted by additional non-access stratum functionalities. |

## Summary of NOTEs and interdependencies

Potential impact on Interworking to be coordinated with KIs related to WT#2

“Interworking and migration to/from 6G” in technical impacts, and in WT/KI text requires solutions to include interworking aspects or justify why not

Potential security and privacy impact to be coordinated by SA3

Solutions are expected to include interworking aspects (see KI#Y) or argument why interworking aspects are not necessary to be considered

UE-Core interactions can result in UE-RAN and/or RAN-CN interactions

# 2. Contribution Summary

Almost all contributions support evolving NAS into a modular, distributed, and service-flexible architecture, with clear separation of minimal essential connectivity functions vs. optional/additional UEs or operator services, and reducing the AMF bottleneck by decentralizing routing.

The following themes emerge for connectivity services where contributions express a position:

- AMF as the single NAS termination point: majority of contributions do propose to decouple from, bypass, or distribute beyond a single AMF NAS termination/anchor, some contributions prefer to keep it, especially for existing services

- Decoupling of NAS-MM and NAS-SM: majority of contributions to propose a decoupling between the two, some contributions propose to inherit/keep 5GS NAS mobility and session management handling mechanisms

- Split of NAS-MM: Also proposals aiming to decouple Mobility Management (MM) responsibilities from today’s AMF-centered registration/MM bundle

- Transport of NAS for connectivity services: proposals route NAS/control signalling for beyond‑connectivity services over the user plane, but NAS for core connectivity services remains on the control plane; no contribution proposes moving connectivity NAS to the UP

- Identifying a minimum set of NAS functionalities: majority of contributions propose it, some contributions propose to keep existing functionality as-is. Some contributions reference SA1’s TR 22.870 as a requirement (clause 5.10.1.1: “It is also important to highlight that while there is expected diversity in the types of UEs, each UE type should have at its core a common set of basic capabilities, to maximise the commonalities and thus reducing market fragmentation”)

- Examples of UE types mentioned in the contributions:

  - NB‑IoT and low‑power IoT devices: cited as “certain types of UEs” needing additional NAS sets beyond the minimal baseline, and as targets for a device‑agnostic minimal NAS profile suitable for constrained devices.

  - Fixed Wireless Access (FWA): referenced in proposals to simplify NAS and optimize paging or routing, noting that some FWA devices do not require typical paging behavior.

  - Smartphones and sophisticated terminals: used as exemplars of “more advanced” UEs that can adopt optional extensions beyond the common basic capabilities.

  - Simple/limited‑capability UEs: described as “simple UEs with limited capabilities,” which should only need the stable minimal NAS profile to reduce complexity and power use.

  - XR/immersive and rich‑capability devices: implied under “immersive communication” and advanced use cases that may leverage beyond‑connectivity service modules and flexible routing.

  - Vehicles/automotive and industrial/enterprise devices: implied in discussions of localized services, privacy, and edge‑deployed NFs, benefiting from distributed NAS and local routing.

  - Sensing/AI/compute‑oriented UEs: explicitly tied to beyond‑connectivity services such as AIML, sensing, and computing, where “service NAS over UP” and per‑service control are proposed.

- NTN/constellation‑affected UEs: UEs operating with non‑terrestrial networks, including satellite constellations, highlighted for mobility and service‑continuity signaling across satellites and ground networks.Avoiding impact to other NAS functionality/NFs when adding NAS functionality: majority of contributions propose it, some contributions propose to keep existing functionality as-is

- Generic UE–Core Network interaction mechanisms for operator services: majority of contributions propose it

Other topics:

- One contribution proposes optimizing paging as an aspect of NAS scalability

# 3. Current references to N1/NAS in 5G specifications

Currently, the following references exist for 5G detailing functionality included and/or transported via a N1 NAS signalling connection:

From clause 8.2.2. of TS 23.501

The NAS protocol on N1 comprises a NAS-MM and a NAS-SM components. There are multiple cases of protocols between the UE and a core network function (excluding the AMF) that need to be transported over N1 via NAS-MM protocol. Such cases include: - Session Management Signalling. - SMS. - UE Policy. - LCS.

Additionally, clause 4.1. of TS 24.501 describes NAS as follows

The non-access stratum (NAS) described in the present document forms the highest stratum of the control plane between the UE and the AMF (reference point "N1" see 3GPP TS 23.501 [8]) for both 3GPP and non-3GPP access. Main functions of the protocols that are part of the NAS are: a) support of mobility of the user equipment (UE) including also common procedures such as authentication, identification, generic UE configuration update and security mode control procedures; b) support of session management procedures to establish and maintain data connectivity between the UE and the data network; and c) NAS transport procedure to provide a transport of SMS, LPP, SLPP, LCS, UPP-CMI container, UE policy container, SOR transparent container and UE parameters update information payload.

And a NAS signalling connection in clause 3.1 of TS 24.501

N1 NAS signalling connection: A peer to peer N1 mode connection between UE and AMF. An N1 NAS signalling connection is either the concatenation of an RRC connection via the Uu reference point and an NG connection via the N2 reference point for 3GPP access, or the concatenation of an IPsec tunnel via the NWu reference point and an NG connection via the N2 reference point for non-3GPP access.

The specifications currently state that certain functionality is “transported over NAS”, while some functionality “is” NAS functionality.

After discussions, the understanding is that when we refer to “NAS” we refer to both “NAS functionality” and what is “transported over NAS”.

**** First Change(all text new) ****

# 5.X. Key Issue #X: Control Signalling aspects

This Key issue studies the following aspects:

1)	Whether and how to enable the introduction of new non-access stratum functionality, such that other non-access stratum functionalities as well as associated functional entities are not affected.

NOTE: This includes whether and how to decouple NAS functionality. Coordination with CT1 could be required

2)	NAS architecture, NAS termination and NAS messages routing

3)	Whether and how to identify a minimum, common set of NAS functionalities

4)	Study signaling interactions and system procedures between UE-CN, RAN-CN and within CN

NOTE W: “CN” refers to the CN for 6G.

NOTE X: Coordination with RAN3 is required for RAN-CN.

NOTE Y: Interworking and migration aspects, especially inter-PLMN aspects (e.g. roaming), where applicable, are covered by WT#2.

NOTE Z: Potential security and privacy impacts require coordination with SA3.

**** End of Changes ****