3GPP TSG-SA2 Meeting #170	S2-2507934

# Goteborg, Sweden, 25th – 29th August, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6405 | rev | 1 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification for QoS monitoring parameters provided to UPF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_URLLC, TEI18 |  |  |  |  |  | Date: |  |  | 2025-08-26 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The Indication of direct event notification parameter does not have a corresponding IE in stage 3 and is in fact not necessary for the UPF as it is describing the same action that the Target of reporting parameter indicates. Since the Target of reporting parameter contains the specific information about where to send the QoS monitoring reports and has corresponding stage 3 IEs (i.e. Direct Reporting Information, see 29.244 clause 7.5.2.9), it is proposed to remove the Indication of direct event notification parameter from the QoS monitoring parameters provided to the UPF. It should be noted that the Indication of direct event notification parameter is meaningful for the QoS monitoring parameters provided to the SMF in order to differentiate between reporting from UPF and SMF/PCF (and it therefore remains in 23.503 clause 6.1.3.21). |  |  |  |  |  |  |  |  |
| Summary of change: |  | The Indication of direct event notification parameter is removed from the QoS monitoring parameters provided to the UPF. A note is added to clarify the relation with the Indication of direct event notification parameter in the QoS Monitoring policy of the PCC rule. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear statements in stage 2 and misalignment with stage 3. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.8.2.18, 5.8.5.11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 5.8.2.18	QoS Flow related QoS monitoring and reporting

The SMF may configure the UPF to perform QoS monitoring for a QoS Flow and to report the monitoring results with the help of the following parameters provided in the Session Reporting Rule (SRR) described in clause 5.8.5.11:

-	QoS monitoring parameter(s) indicating the subject of the QoS monitoring as defined in clause 5.45;

NOTE 1:	Clause 5.45 contains specific description about the UPF configuration for QoS monitoring for the respective QoS monitoring parameter.

-	Reporting period indicating the time interval in which a new measurement result and a potential report has to be available. Generally, if no measurement result is available to the UPF within the Reporting period, the UPF shall report a measurement failure; however, for some QoS monitoring parameters (e.g. congestion information, PDV and data rate, available bitrate), the measurement failure report is not applicable.

-	Reporting frequency indicating the type of the reporting as "periodic" or "event triggered":

-	If the Reporting frequency indicates "periodic", the UPF shall send a report each time the reporting period is over.

-	If the Reporting frequency indicates "event triggered", a Reporting threshold for each parameter in the QoS monitoring parameter(s) and a Minimum waiting time are provided as well. The UPF shall send a report when the measurement result matches or exceeds the indicated Reporting threshold. Subsequent reports should not be sent by the UPF during the Minimum waiting time. The UPF shall continue to report a measurement result that matches or exceeds the indicated Reporting Threshold when the Minimum waiting time is over.

NOTE 2:	As an implementation option, the UPF can be configured to send subsequent report(s) during the Minimum waiting time, e.g. if the UPF determines that this report is considerably different from the previous report.

-	(Optional) Target of reporting indicating that the UPF shall send the reports to a different NF than the SMF (e.g. to the NEF/AF or the NWDAF/DCCF/MFAF). The NF is identified by a Notification Target Address and a Notification Correlation ID. The SMF can also indicate that the UPF shall send the reports to both, the NF indicated by the Target of reporting and to the SMF, by providing the Duplicated notification information (as defined in TS 29.244 [65]). If so, the UPF shall send the reports to the SMF as well. If the Target of reporting is not provided, the UPF shall send the reports to the SMF.

NOTE 3:	The SMF provides the Target of reporting if it has received Target of reporting and Indication of direct event notification in the QoS Monitoring policy of the PCC rule (see clause 6.1.3.21 of TS 23.503 [45]).

-	(Optional) Reporting suggestion information as defined in clause 5.8.2.17 applicable to Target of reporting to reduce the UPF performance impacts. The SMF includes Reporting Suggestion information if received in a subscription to UPF QoS Monitoring event sent by the consumer via SMF for UPF Data Collection (see clause 4.15.4.5 of TS 23.502 [3]).

-	(Optional) Indication of QoS Flow associated with the default QoS Rule (see clause 4.15.4.5.1 of TS 23.502 [3]). The UPF shall forward this indication, that the QoS monitoring report is for the QoS Flow associated with the default QoS Rule, in the Nupf_EventExposure_Notify service operation when sending reports.

-	(Optional) Remaining data reporting indication. The SMF includes this indication if received in a subscription to UPF QoS Monitoring event sent by the consumer via SMF for UPF Data Collection (see clause 4.15.4.5). It indicates that the consumer requests that, at N4 session release, UPF reports any collected data not yet sent. UPF may send any collected data not yet sent to the UPF event consumer considering the Remaining data reporting indication and/or local configuration as described in clause 4.15.4.5.6 of TS 23.502 [3].

The UPF shall send the QoS Monitoring Report as follows:

-	when the UPF sends reports to the SMF, the UPF shall use QoS Monitoring Reports as described in clause 5.8.5.12; and/or

-	When the UPF sends reports to a different NF than the SMF (e.g. the NEF/AF or the NWDAF/DCCF/MFAF), the UPF shall use the Nupf_EventExposure_Notify service operation described in clause 5.2.26.2.2 of TS 23.502 [3].

# * * * * Next change * * * *

#### 5.8.5.11	Session Reporting Rule

The following table describes the Session Reporting Rule (SRR) that defines the detection and reporting events that the UPF shall report, that are not related to specific PDRs of the PDU Session, as follows:

-	Per QoS Flow per UE QoS Monitoring Report, as specified in clause 5.8.2.18.

-	Change of 3GPP or non-3GPP access availability, for an MA PDU session.

-	Per QoS Flow N6 Traffic Parameter Measurement Report.

Table 5.8.5.11-1: Attributes within Session Reporting Rule

| Attribute | Description | Comment |
| --- | --- | --- |
| N4 Session ID | Identifies the N4 session associated to this SRR. |  |
| Rule ID | Unique identifier to identify this information. | Used by UPF when reporting. |
| QoS Monitoring per QoS Flow Control Information | Indicates the UPF to perform the QoS Monitoring for one or more QoS Flows. | The IE is defined in clause 7.5.2.9 of the TS 29.244 [65]. See NOTE 1. |
| Access Availability Control Information | Indicates the UPF to report when an access type becomes available or unavailable for an MA PDU Session. | The IE is defined in clause 7.5.2.9 of TS 29.244 [65]. |
| N6 Traffic Parameter Measurement Control Information | Indicates the UPF to report N6 Traffic parameter measurements for one QoS Flow, e.g. a measurement of N6 jitter range for a DL Periodicity and conditionally, a measurement of the UL/DL periodicity. May indicate the DL Periodicity (See NOTE 2). | The IE is defined in clause 7.5.2.9 of TS 29.244 [65]. See NOTE 2. |
| NOTE 1: The QoS Monitoring per QoS Flow Control Information may contain a Notification Target Address (the details are described in clause 5.8.2.18) that identifies the recipient of the information being notified by the UPF (Local NEF/AF) via Nupf_EventExposure_Notify service operation. NOTE 2: The DL Periodicity is provided by the SMF in the N6 Traffic Parameters Measurement Control Information when the DL Periodicity is received from the PCF. |  |  |

# * * * * End of changes * * * *