3GPP TSG-SA2 Meeting #170	S2-2507875

Stor-Göteborg, Sweden, 25th Aug 2025 - 29th Aug 2025	(revision of S2-2506576)

Source:	LG Electronics

Title:	[General aspects] Architecture Requirements for 6G

Document for:	Approval

Agenda Item:	20.6.0

Work Item / Release:	FS_6G_ARC / Rel-20

Abstract of the contribution: This paper proposes Architectural Requirements.

Discussion

The architecture requirements proposed by the following Tdocs are listed to discuss and decide agreeable architecture requirements:

| Architecture Requirement |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 20.6.0 | S2-2506392 | P-CR | Approval | 23.801-01: [General aspects] Architecture Requirements for 6G | OPPO | Rel-20 | FS_6G_ARC |
| 20.6.0 | S2-2506509 | P-CR | Approval | 23.801-01: [General Aspects] 6G Architecture support Stateless NF Services | Boost Mobile Network, EchoStar, Telecom Italia S.p.A, China Mobile, Cisco Systems, SK Telecom | Rel-20 | FS_6G_ARC |
| 20.6.0 | S2-2506576 | P-CR | Approval | 23.801-01: [General aspects] Architecture Requirements for 6G | LG Electronics | Rel-20 | FS_6G_ARC |
| 20.6.0 | S2-2506635 | P-CR | Approval | 23.801-01: [General aspects] Architectural Assumptions and Requirements | vivo | Rel-20 | FS_6G_ARC |
| 20.6.0 | S2-2506774 | P-CR | Approval | 23.801-01: [General aspect] Architecture assumption and requirements | ZTE | Rel-20 | FS_6G_ARC |
| 20.6.0 | S2-2506829 | P-CR | Approval | 23.801-01: [General Aspects] 6G Architecture support Cloud Native | Boost Mobile Network, EchoStar, AT&T, Telecom Italia S.p.A, Rakuten, KPN N.V., Cisco Systems, SK Telecom | Rel-20 | FS_6G_ARC |
| 20.6.0 | S2-2506929 | P-CR | Approval | 23.801-01: [General aspects] Architecture assumptions and requiremnts for 6G architecture | Ericsson, T-Mobile USA | Rel-20 | FS_6G_ARC |
| 20.6.0 | S2-2507054 | P-CR | Approval | 23.801-01: [General aspects] Architecture Assumptions and Requirements | Nokia, AT&T, Ericsson, T-Mobile US | Rel-20 | FS_6G_ARC |
| 20.6.0 | S2-2507100 | P-CR | Approval | 23.801-01: [General aspects] 6G System Architectural Requirements and Assumptions | CATT | Rel-20 | FS_6G_ARC |
| 20.6.0 | S2-2507207 | P-CR | Approval | 23.801-01: [General aspects] Architecture Assumptions and Requirements on Energy Efficiency | NTT DOCOMO, KPN N.V., AT&T, Deutsche Telekom | Rel-20 | FS_6G_ARC |
| 20.6.1.4 | S2-2507416 | P-CR | Approval | 23.801-01: Voice for 6G - Key Issues and Architectural Assumptions | T-Mobile USA Inc. | Rel-20 | FS_6G_ARC |
| 20.6.2 | S2-2506994 | P-CR | Approval | 23.801-01: [WT#2, Interworking and Migration] Inter-System interworking and migration consideration | Huawei, HiSilicon | Rel-20 | FS_6G_ARC |
| 20.6.1.2 | S2-2506356 | P-CR | Approval | 23.801-01: [WT#1.2, Network Sharing] Architectural requirements on 6G network sharing | China Unicom, China Telecom, CATT | Rel-20 | FS_6G_ARC |

[1]	All architecture requirements from the above input contributions as following:

// LGE (S2-2506576) - The 6G system shall support PLMN as well as (S)NPN. - The 6G system shall support network sharing. // OPPO (S2-2506392) - Support the new RAT(s)and non-3GPP access types (e.g., WLAN access, Satellite radio access, etc.). NOTE A: The support of Satellite radio access network needs to be coordinated with RAN2 - Support multiple simultaneous connections of a UE via multiple access technologies. - Support a separation of Control plane and User plane functions. - Support transmission of IP packets, non-IP PDUs and Ethernet frames. - Support various session and service continuity mode(s) to accommodate different requirements of the application during UE mobility. - Support UE power consumption while UE is in periods w/o data traffic. - Support services that have different latency requirements between the UE and the Data Network. - Support minimizing the delay for transmission and handling of signalling. - Support access to applications (including 3rd party applications) with low latency requirements hosted close to the access network within the operator trust domain. - Support leveraging the communication and computing delay for different services. - Support signalling congestion control. - Support a large number of UEs in periods without data traffic efficiently. - Support AI. - Support sensing. - Support roaming (i.e., home routed, local breakout). - Support energy efficiency in the system design and overall network operation. - Support network capability exposure. - Support interworking with 5GS and EPS. - Support a common data framework for beyond-connectivity services. // Boost Mobile Network (S2-2506509) - 6G system architecture shall aim to support the cloud-native benefit where every NF service is fully stateless. - It shall be documented why a specific NF service in the 6G system architecture cannot achieve full statelessness, e.g., performance impact. // vivo (S2-2506635) - Support the new 6G RAT(s) and non 3GPP access. - Support the functional split between 6G core and 6G AN. - Define the interface between 6G Core and 6G RAN supporting 6G RAT(s). - Support multiple simultaneous connections of a UE via multiple access technologies. - Support and/or enhance the essential/regulatory services (i.e. voice, Messaging, location services, Emergency services, MPS, Mission Critical services, PWS). - Support network sharing, roaming, different levels of UE mobility, etc. - Leverage techniques for simplicity and flexibility for deploying and offering new services, e.g. support decoupling among different functionalities, e.g. when introducing new non-access stratum functionality to the 6G architecture, minimize the impact to other non-access stratum functionalities. - Support and enable network for AI and AI for network. - Support data framework for all aspects related to efficient and scalable data handling - Support different means for reducing UE power consumption, especially optimize the energy efficiency without reduce the large bandwidth transmission for a light-weight UE (e.g., AR glasses). - Support interworking to 5GS and support interworking between 6GS and 4G/5G NTN/satellite access that use EPS/5GS. // ZTE (S2-2506774) - The 6G system should support the following high-level principles: cloud nativeness, sustainability and energy efficiency, robustness and resiliency, and network simplicity. - At least the interfaces between UE and core, between RAN and core, and roaming interfaces shall support multi-vendor interoperability. Other interfaces will be determined during the study. - The 6G system shall support both connectivity service and beyond connectivity service - The 6G system shall support interworking with 5GS // Boost Mobile Network (S2-2506829) - All 3GPP 6G system defined services of any one Network Function (NF) shall not require any standardized interoperability testing for their interactions amongst them within the same NF. - 6G system architecture shall support standardized service-based interfaces between NFs for the purpose of making use of cloud native service-based high availability, scalability, and observability. // Ericsson (S2-2506929) - The functional split between Core and RAN in 6GS shall follow the same principles as for 5GS. NOTE B: As stated in the SID, the duplication of functionality in RAN and CN will be avoided, while maintaining the existing RAN and CN functionality split. - Tracking areas in 5G RAN and 6G RAN are separate. - 6GS should not require change in roaming agreements between VPLMN and HPLMN. // Nokia (S2-2507054) - This study aims at supporting multi-vendor interoperable interfaces within 6G System. - The 6G System procedures shall result in deterministic behaviour for connectivity and beyond connectivity services delivered by 3GPP system. - The study should consider the following high-level principles: cloud native deployment, sustainability and energy efficiency, robustness and resiliency. - As 5G System and 6G System are expected to co-exist in deployments, interworking with service continuity between 5G System and 6G System shall be supported. // CATT (S2-2507100) - Support and enable use of AI in 6G (e.g. AI agent, framework). - Support integration of Sensing and Communication over 3GPP access. - Support computing for UE, core network and application server. - Support data framework for all aspects related to efficient and scalable data handling. // NTT DOCOMO (S2-2507207) - The 6G architecture and all related system procedures shall take energy efficiency into account. This includes, but is not limited to: - Avoiding unnecessary signalling (or processing load) in both the control plane and user plane, thereby improving efficiency; - Providing architectural enablers for energy-related metrics (performance) monitoring, reporting/exposure, and sustainable/energy-efficient (policy) control; - Allowing on-demand power savings of individual NFs efficiently while maintaining the required QoS; and - Ensuring that interworking and migration solution(s) do not introduce disproportionate energy overhead. // T-Mobile USA (S2-2507416) - Voice and emergency voice services shall be defined without the need for any other access network for the services to operate. - Emergency voice service shall not require any other access network to provide the required UE location. // Huawei (S2-2506994) - Support the interworking with 5G network with minimum impact to the user experience. - Support the interworking between 6GS and 5GS work even if the UE has previously registered in 2G, 3G or 4G. // China Unicom (S2-2506356) - Support network sharing: - Support the network sharing architectures to share the 6G access resources (e.g., new 6G radio access resource, NTN access resource), at least including 6G Multi-Operator Core Network (6G MOCN) and 6G Indirect Network Sharing (6G INS). - Support to extend the scope and types of network resources and infrastructures to be shared in 6G, e.g. network sharing on 6G access network with sensing capability. - Support the on-demand network sharing or event triggered network sharing, e.g. considering the flexible trigger to turn on/off the 6G network sharing and reducing network sharing pre-configurations between the operators.

[2]	Removed duplications/overlaps in the above list and added numbering for discussion.

1. [S2-2506576 (LGE)] The 6G system shall support PLMN as well as (S)NPN. 2. [S2-2506392 (OPPO)] Support the new RAT(s)and non-3GPP access types (e.g., WLAN access, Satellite radio access, etc.). NOTE A: The support of Satellite radio access network needs to be coordinated with RAN2 3. [S2-2506392 (OPPO)] Support multiple simultaneous connections of a UE via multiple access technologies. 4. [S2-2506392 (OPPO)] Support a separation of Control plane and User plane functions. 5. [S2-2506392 (OPPO)] Support transmission of IP packets, non-IP PDUs and Ethernet frames. 6. [S2-2506392 (OPPO)] Support various session and service continuity mode(s) to accommodate different requirements of the application during UE mobility. 7. [S2-2506392 (OPPO)] Support services that have different latency requirements between the UE and the Data Network. 8. [S2-2506392 (OPPO)] Support access to applications (including 3rd party applications) with low latency requirements hosted close to the access network within the operator trust domain. 9. [S2-2506392 (OPPO)] Support minimizing the delay for transmission and handling of signalling. 10. [S2-2506392 (OPPO)] Support signalling congestion control. 11. [S2-2506392 (OPPO)] Support a large number of UEs in periods without data traffic efficiently. 12. [S2-2506392 (OPPO)] Support network capability exposure. 13. [S2-2506392 (OPPO)] Support interworking with 5GS and EPS. 14. [S2-2506635 (vivo)] Define the interface between 6G Core and 6G RAN supporting 6G RAT(s). 15. [S2-2506929 (Ericsson)] The functional split between Core and RAN in 6GS shall follow the same principles as for 5GS. NOTE B: As stated in the SID, the duplication of functionality in RAN and CN will be avoided, while maintaining the existing RAN and CN functionality split. 16. [S2-2506635 (vivo)] Support and/or enhance the essential/regulatory services (i.e. voice, Messaging, location services, Emergency services, MPS, Mission Critical services, PWS). 17. [S2-2507416 (T-Mobile USA)] Voice and emergency voice services shall be defined without the need for any other access network for the services to operate. 18. [S2-2507416 (T-Mobile USA)] Emergency voice service shall not require any other access network to provide the required UE location. 19. [S2-2506635 (vivo)] Support roaming (i.e., home routed, local breakout), different levels of UE mobility. 20. [S2-2506929 (Ericsson)] 6GS should not require change in roaming agreements between VPLMN and HPLMN. 21. [S2-2506635 (vivo)] Leverage techniques for simplicity and flexibility for deploying and offering new services, e.g. support decoupling among different functionalities, e.g. when introducing new non-access stratum functionality to the 6G architecture, minimize the impact to other non-access stratum functionalities. 22. [S2-2506635 (vivo)] Support data framework for all aspects related to efficient and scalable data handling 23. [S2-2506774 (ZTE)] The 6G system should support the following high-level principles: cloud nativeness, sustainability and energy efficiency, robustness and resiliency, and network simplicity. 24. [S2-2506509 (Boost Mobile Network)] 6G system architecture shall aim to support the cloud-native benefit where every NF service is fully stateless. 25. [S2-2506509 (Boost Mobile Network)] It shall be documented why a specific NF service in the 6G system architecture cannot achieve full statelessness, e.g., performance impact. 26. [S2-2506829 (Boost Mobile Network)] All 3GPP 6G system defined services of any one Network Function (NF) shall not require any standardized interoperability testing for their interactions amongst them within the same NF. 27. [S2-2506829 (Boost Mobile Network)] 6G system architecture shall support standardized service-based interfaces between NFs for the purpose of making use of cloud native service-based high availability, scalability, and observability. 28. [S2-2507207 (NTT DOCOMO)] The 6G architecture and all related system procedures shall take energy efficiency into account. This includes, but is not limited to: 28-A. [S2-2507207 (NTT DOCOMO)] Avoiding unnecessary signalling (or processing load) in both the control plane and user plane, thereby improving efficiency; 28-B. [S2-2507207 (NTT DOCOMO)] Providing architectural enablers for energy-related metrics (performance) monitoring, reporting/exposure, and sustainable/energy-efficient (policy) control; 28-C. [S2-2507207 (NTT DOCOMO)] Allowing on-demand power savings of individual NFs efficiently while maintaining the required QoS; and 28-D. [S2-2507207 (NTT DOCOMO)] Ensuring that interworking and migration solution(s) do not introduce disproportionate energy overhead. 29. [S2-2506774 (ZTE)] The 6G system shall support both connectivity service and beyond connectivity service 30. [S2-2507054 (Nokia)] The 6G System procedures shall result in deterministic behaviour for connectivity and beyond connectivity services delivered by 3GPP system. 31. [S2-2506929 (Ericsson)] Tracking areas in 5G RAN and 6G RAN are separate. 32. [S2-2507054 (Nokia)] This study aims at supporting multi-vendor interoperable interfaces within 6G System. 33. [S2-2507100 (CATT)] Support and enable use of AI in 6G (e.g. AI agent, framework). 34 [S2-2507100 (CATT)] Support integration of Sensing and Communication over 3GPP access. 35 [S2-2507100 (CATT)] Support computing for UE, core network and application server. 36. [S2-2506994 (Huawei)] Support the interworking with 5G network with minimum impact to the user experience. 37. [S2-2506994 (Huawei)] Support the interworking between 6GS and 5GS work even if the UE has previously registered in 2G, 3G or 4G. 38. [S2-2506356 (China Unicom)] Support network sharing: 38-A. [S2-2506356 (China Unicom)] Support the network sharing architectures to share the 6G access resources (e.g., new 6G radio access resource, NTN access resource), at least including 6G Multi-Operator Core Network (6G MOCN) and 6G Indirect Network Sharing (6G INS). 38-B. [S2-2506356 (China Unicom)] Support to extend the scope and types of network resources and infrastructures to be shared in 6G, e.g. network sharing on 6G access network with sensing capability. 38-C. [S2-2506356 (China Unicom)] Support the on-demand network sharing or event triggered network sharing, e.g. considering the flexible trigger to turn on/off the 6G network sharing and reducing network sharing pre-configurations between the operators.

Proposal

It is proposed to agree the following changes into TR 23.801-01.

# * * * * Start of 1st Change * * * *

4.2	Architectural Requirements

Editor's Note:	This clause defines the architectural requirements that serve as the foundation for the study.

The following architectural requirements are considered:

1.	The 6G system shall support roaming

3. The 6G system should shall consider the following high-level aspects: cloud nativeness, sustainability and energy efficiency, robustness and resiliency.

5. This study should support multi-vendor interoperable interfaces within 6G System.

6.	Support network sharing:

6-A.	Support the network sharing architectures to share the 6G access resources (e.g., new 6G radio access resource, NTN access resource), at least including 6G Multi-Operator Core Network (6G MOCN) and 6G Indirect Network Sharing (6G INS).

# * * * * End of Changes * * * *