SA WG2 Meeting #170	S2-2507943

25 – 29 Aug, 2025, Goteborg , SE

Source:	Samsung

Title:	[WT#1.1, all topics] Support of common aspects for beyond connectivity services

Document for:	Agreement

Agenda Item:	20.6.1.1

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: KIs on beyond connectivity services for Work task 1.1

1. Overview

This paper is a merged proposal of all relevant proposals on KIs on beyond connectivity services for WT#1.1 of the approved 6G SID in SP-250806.

2. Contributions

Relevant contributions are listed.

| 20.6.1.1 | S2-2506393 | P-CR | Approval | 23.801-01: [WT#1.1] Support of Control Signalling in 6GS | OPPO | Rel-20 | FS_6G_ARC |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 20.6.1.1 | S2-2506465 | P-CR | Approval | 23.801-01: [WT#1.1, control signalling] 6G NAS for connectivity and beyond connectivity services | China Telecom Corporation Ltd. | Rel-20 | FS_6G_ARC |
| 20.6.1.1 | S2-2506480 | P-CR | Approval | 23.801-01: [WT#1.1] WT and KI for enhancement of NAS control signalling | China Mobile | Rel-20 | FS_6G_ARC |
| 20.6.1.1 | S2-2506505 | P-CR | Approval | 23.801-01: [WT#1.1, all topics] Control Signalling Design for 6G | MediaTek Inc. | Rel-20 | FS_6G_ARC |
| 20.6.1.1 | S2-2506517 | P-CR | Approval | 23.801-01: [WT#1.1] NAS for 6G | Deutsche Telekom, AT&T, Cisco Systems, KPN N.V., Rakuten Mobile, SK Telecom, T-Mobile USA, Telecom Italia | Rel-20 | FS_6G_ARC |
| 20.6.1.1 | S2-2506523 | P-CR | Approval | 23.801-01: [WT#1.1] NAS functionality for 6G | Ofinno | Rel-20 | FS_6G_ARC |
| 20.6.1.1 | S2-2506555 | P-CR | Approval | 23.801-01: [WT#1.1, Non-Access Stratum Functionality] Support for NAS Signalling in the 6G Architecture | InterDigital Washington DC | Rel-20 | FS_6G_ARC |
| 20.6.1.1 | S2-2506577 | P-CR | Approval | 23.801-01: [WT#1.1, NAS functionalities] Scope and New Key Issue for NAS functionalities | LG Electronics | Rel-20 | FS_6G_ARC |
| 20.6.1.1 | S2-2506611 | P-CR | Approval | 23.801-01: [WT#1.1, 6G NAS] Support of 6G New NAS | vivo | Rel-20 | FS_6G_ARC |
| 20.6.1.1 | S2-2506752 | P-CR | Approval | 23.801-01: [WT#1.1] Discussion and proposal on key issue for work task#1.1 | CATT | Rel-20 | FS_6G_ARC |
| 20.6.1.1 | S2-2506775 | P-CR | Approval | 23.801-01: [WT#1.1, NAS] Discussion and proposal for NAS | ZTE | Rel-20 | FS_6G_ARC |
| 20.6.1.1 | S2-2506822 | P-CR | Approval | 23.801-01: [WT#1.1] UE-Core Network interaction in 6G | CEWiT | Rel-20 | FS_6G_ARC |
| 20.6.1.1 | S2-2506840 | P-CR | Approval | 23.801-01: [WT#1.1, all topics] Support of control signaling for connectivity and beyond connectivity services | Samsung | Rel-20 | FS_6G_ARC |
| 20.6.1.1 | S2-2506868 | P-CR | Approval | 23.801-01: [WT#1.1, Control Signalling for 6G] Scope and Key Issue proposal for UE-Core Network interaction | IIT Bombay | Rel-20 | FS_6G_ARC |
| 20.6.1.1 | S2-2506871 | P-CR | Approval | 23.801-01: [WT#1.1, Non-Access Stratum] Scope and Key Issue for Non-Access Stratum | Futurewei | Rel-20 | FS_6G_ARC |
| 20.6.1.1 | S2-2506903 | P-CR | Approval | 23.801-01: [WT#1.1] Introduction of NAS functionality without impacting other NAS functionalities | Apple | Rel-20 | FS_6G_ARC |
| 20.6.1.1 | S2-2506942 | P-CR | Approval | 23.801-01: [WT#1.1, Control signalling for 6G] UE-CN control signalling communication | Ericsson | Rel-20 | FS_6G_ARC |
| 20.6.1.1 | S2-2506975 | P-CR | Approval | 23.801-01: [WT #1.1, 6G NAS] Discussion and key issues for distributed NAS | TCL | Rel-20 | FS_6G_ARC |
| 20.6.1.1 | S2-2506988 | P-CR | Approval | 23.801-01: [WT#1.1] Update to WT#1.1 for clarification on flexible addition of new services in 3GPP | Qualcomm Korea | Rel-20 | FS_6G_ARC |
| 20.6.1.1 | S2-2507055 | P-CR | Approval | 23.801-01: [WT#1.1, all topics] Work task #1.1 Support of 6G Control plane signaling | Nokia, AT&T, T-Mobile US | Rel-20 | FS_6G_ARC |
| 20.6.1.1 | S2-2507124 | P-CR | Approval | 23.801-01: [WT#1.1, NAS] New NAS and Interaction between UE and Network | Huawei, HiSilicon | Rel-20 | FS_6G_ARC |
| 20.6.1.1 | S2-2507135 | P-CR | Approval | 23.801-01: [WT#1.1] Technical Scope of WT#1.1 - Control Signalling in 6GS | ETRI | Rel-20 | FS_6G_ARC |
| 20.6.1.1 | S2-2507200 | P-CR | Approval | 23.801-01: [WT#1.1, NAS] Key Issues on NAS and operator services in 6G | Lenovo | Rel-20 | FS_6G_ARC |
| 20.6.1.1 | S2-2507353 | P-CR | Approval | 23.801-01: [WT#1.1] KI on UE-CN interaction for control signalling for 6G | Xiaomi | Rel-20 | FS_6G_ARC |
| 20.6.1.1 | S2-2507378 | P-CR | Approval | 23.801-01: WT#1.1, Study proposal to enhance a NAS functionality | NEC Corporation (ARIB) | Rel-20 | FS_6G_ARC |

S2-2506393, OPPO

This key issue proposes to study how to support the control signalling (i.e., beyond connectivity service message) interaction for MNO service between UE and the according NFs. The key issue will investigate following aspects:

- Whether and how to route the control signalling for Beyond Connectivity services (e.g., LCS, Sensing, AI, etc.) between UE and the according NFs over user plane; This aspect can investigate whether new network function for control signalling routing is needed.

NOTE 1: Whether a generic mechanism should be defined for Beyond Connectivity service signalling transfer will be determined in TR conclusion.

- Network function selection and UE context storage.

S2-2506465, China Telecom

KI#X.2: Whether and how to design 6G NAS for beyond connectivity services, to meet the requirement of signalling scalability and efficiency.

S2-2506480, China Mobile

In order to improve the control signalling transmission between UE and Core Network, the NAS mechanism may need to be updated. This key issue includes following aspects:

- How to categorize NAS functionalities e.g., basic functionalities,

- Whether and how to identify enhanced NAS functionality for beyond connectivity

- Whether and how to support for flexible and efficient introduction mechanism of new NAS functionality without any impact on other NAS functionality.

- Study the approaches for UE and Core Network interaction e.g., NAS enhancement or over user plane, Service based interface between UE and Core Network.

S2-2506505, MediaTek

This Key Issue focuses on studying the support for control signalling for 6G for connectivity services and/or beyond connectivity services, including at least the following:

a.	Whether and how to enable the introduction of a new non-access stratum functionality without impacting other non-access stratum functionalities.

b.	Whether and how to identify a minimal set of non-access stratum functionalities that does not get impacted by additional non-access stratum functionalities.

c.	Whether and how to develop generic mechanisms for UE-Core Network interaction to support operator services.

In particular, this key issue addresses at least the below versus 5GS design as the baseline:

- How to route control messages for connectivity, legacy services (i.e., NAS) between UE and CN

- How to route control messages for beyond connectivity, new services between UE and CN

- Impact on termination point of interfaces between UE and CN

- Define NAS architecture and system procedures (e.g., registration, connection, mobility, session management, etc.)

- Where and how to handle authentication, security management and integrity protection

NOTE 1: Potential impact on Interworking to be coordinated with KIs related to WT#2.

NOTE 2: UE-CN interactions for beyond connectivity services to be coordinated with KIs related to WT#5, WT#6.

NOTE 3: Potential security and privacy impact to be coordinated by SA3.

S2-2506517, Deutsche Telekom, AT&T, Cisco Systems, KPN N.V., Rakuten Mobile, SK Telecom, T-Mobile USA, Telecom Italia

In 5G, NAS provides signaling between the UE and the 5GC, is based on a single N1 NAS signalling connection, which is used for each access to which the UE is connected. The single N1 termination point is located in the AMF, and also provides transport to SM, SMS, UE policy and LCS-related functionality, as described in clause 8.2.2.1 of TS 23.501 [X].

This key issue focuses on how to enable:

-	the introduction of a new non-access stratum functionality in a variety of deployment scenarios without impacting other non-access stratum functionalities, especially without impacting the AMF

-	identifying a minimal set of non-access stratum functionalities that does not get impacted by additional non-access stratum functionalities, especially dependency on the AMF

Solutions for this key issue will describe, based on the WT scope described in Annex A.1.1.X WT#1.1 Control signalling for 6G  services:

-	How NAS functionality can be specified independent of each other and minimizing dependencies between NFs

-	How NFs providing NAS functionality can be flexibly and efficiently deployed

-	What interactions between UE-and-CN, UE-and-RAN and RAN-and-CN are required

Solutions are expected to include interworking aspects (see KI#Y) or argument why interworking aspects are not necessary to be considered.

Editor’s Note: N1, N2, AMF all refer to functionality and interfaces for 6G. The final terminology (e.g. 6G-N2, eN2, etc.) has yet to be decided.

S2-2506523, Ofinno

This key issue will study how to extend the NAS functionality to support the enhanced system architecture and procedures for 6G services.

The basis of 6G services can be the connectivity services (e.g. mobility management such as registration, paging, and session management) and the beyond connectivity services (e.g. the operator services that can be existed over N6 such as AI/ML services or Sensing services). Based on the connectivity and beyond connectivity services to be supported and the methods for their support, this key issue will identify the essential and minimum elements as well as additional elements of the extended NAS functionality for 6G.

In 5GS, the AMF and SMF are the NFs which support NAS functionality. The NAS for Mobility Management terminates in AMF and the NAS for Session Management terminates in SMF via AMF. In this key issue, for 6G system, the extension of NAS functionality may introduce new NAS functionality into the NF type(s) which has been supported NAS or has not been supported NAS. It may include the split or add of NAS message transfer path that can work with minimum AMF involvements.

The following aspects will be studied for this key issue:

- How to introduce 6G NAS functionality to support the connectivity of UE and 6G network, and/or to support the connectivity of UE and operator services beyond the 6G core network. This includes:

  - How to determine whether the UE/network supports the 6G NAS functionality.

  - How to identify/discover a NF which is supporting the 6G NAS functionality.

  - How to transport the NAS messages between the UE and the network that are supporting 6G NAS functionality with minimum involvements of other existing NFs.

  - How to minimize congestion/overhead in the NAS signalling.

  - How to support no or minimum impacts on other existing NFs, when the NF supporting 6G NAS functionality is used.

  - How to support the security of the 6G NAS functionality (e.g. which NF (AMF, or else) will be responsible for NAS security, and corresponding procedures), based on coordination with SA3.

- Identify a minimal set of NAS functionalities to be supported for all UEs using general connectivity services (e.g. registration, mobility, paging, using PDU Sessions), and the additional NAS functionalities to be supported for certain types of UEs (e.g. NB-IoT) or for certain operator services (e.g. AI/ML services, Sensing services). This includes:

  - Identify the required NF(s) to serve the UEs using general connectivity services, and the corresponding architecture and procedures.

  - Identify the additional NF(s) to serve the UEs using certain operator services or to serve certain types of UEs, and the corresponding architecture and procedures.

- How to support the 6G NAS functionality in roaming scenarios. This includes:

  - How to determine whether the Home/Visited network supports the 6G NAS functionality.

S2-2506555, InterDigital

This key issue (KI) will study the support for control signalling in 6G for connectivity services and/or beyond connectivity services, including at least the following:

- Whether and how to enable the introduction of a new non-access stratum functionality without impacting other non-access stratum functionalities.

- Whether and how to redefine the functional split between the Access and Mobility Management Function (AMF) and other Core Network Functions (NFs).

- Whether and how to remove dependencies between NAS features in order   to support independent evolution and deployment.

- Reduce NAS routing constraints by supporting direct routing of NAS messages to target NFs..

- Whether and how to identify a minimal set of non-access stratum functionalities that does not get impacted by additional non-access stratum functionalities.

- Whether and how the 6G system can support distributed NAS design principles to enable minimal configurations.

- Whether there is a need to define criteria for determining functional independence among NAS module

- Whether and how new NAS functionalities can be integrated dynamically without introducing backward compatibility or dependency issues on the core minimal set.

c.	Whether and how to develop generic mechanisms for UE-Core Network interaction to support operator services.

- Whether and how to allow for flexible UE-CN and RAN-CN interactions, potentially bypassing the AMF where appropriate.

- Whether and how to support diverse 6G deployment scenarios (e.g., distributed, NPNs, NTNs) through adaptable signalling paths and transport protocols.

- Whether and how to enable the dynamic provisioning of NAS functionalities and policies in a service-agnostic, operator-customizable manner..

NOTE: Security aspects for distributed NAS aiming at reducing other NAS functionalities tight coupling with AMF are to be evaluated by SA3.

S2-2506577, LG Electronics

This key issue studies support of NAS functionalities in 6G system. Specifically, this Key Issue studies:

-	Support of control signalling in 6G system for connectivity services (e.g. mobility management, session management, policy management, etc.) and beyond connectivity services (e.g. Sensing) in case of roaming and non-roaming scenarios.

-	Whether and how to enable the introduction of a new non-access stratum functionality without impacting other non-access stratum functionalities.

-	Whether and how to identify a minimal set of non-access stratum functionalities that does not get impacted by additional non-access stratum functionalities.

-	Whether and how to develop generic mechanisms for UE-Core Network interaction to support operator services.

S2-2506611, vivo

This Key Issue aims at:

a)	How to support distributed UE-CN communication with direct NAS signaling connection between UE and 6G NFs, i.e. no single NAS signaling anchor.

b)	How to define a minimal set of NAS functionalities to support modular UE-CN communication.

c)	Whether and how to utilize the capabilities of, e.g. 6G User Plane, for service NAS transport for beyond connectivity services (e.g. AI/Sensing/Computing).

S2-2506752, CATT

6G supports connectivity services and beyond connectivity services. The connectivity services are responsible to provide connectivity to transfer user data. The beyond connectivity services are not associated to user data, e.g. AI service, sensing service.

To support non-access stratum functionality for different services, solutions for this key issue will study:

-	Whether and how to enable the introduction of a new non-access stratum functionality without impacting other non-access stratum functionalities.

-	Whether and how to identify a minimal set of non-access stratum functionalities that does not get impacted by additional non-access stratum functionalities.

-	Whether and how to develop generic mechanisms for UE-Core Network interaction to support different deployment requirements.

S2-2506775, ZTE

KI#Y: Study how to support NAS signaling between UE and network for beyond connectivity services. This will include the following aspects:

1.	Study whether and how to enable the introduction of a new beyond connectivity service without impacting the connectivity service and other beyond connectivity service.

2.	Whether and how to develop generic mechanisms for UE-Core Network interaction to support beyond connectivity services, e.g. connection establishment, service authentication/authorization, service exposure between UE and Core network, etc.

S2-2506822, CEWiT

5.x1. Key Issue #1.1.x1: How to enhance NAS for supporting new 6G services

This Key Issue aims to study on enhancements to NAS for newly added functionalities supporting connectivity and beyond connectivity services in 6G without impacting the existing NAS functionalities.

5.x2. Key Issue #1.1.x2:  Scope for identification of device- agnostic minimal set of NAS functionalities

This Key Issue aims to identify a minimal set of NAS functionalities suitable for all devices, including those with limited capabilities like low- powered IoT devices.

5.x3. Key Issue #1.1.x3: Efficient and secured NAS message transmission

This Key Issue aims to study how efficiently and securely the NAS messages can be transmitted, with reduced complexity and signalling overhead.

NOTE: The detailed study to a secured transmission of NAS messages will be coordinated and aligned with SA3.

S2-2506840, Samsung

- Identify a minimal set of NAS functionalities (e.g., mobility management, session management) that does not get impacted by a beyond connectivity operator services.

- How to mitigate the impact on existing NAS functionalities when introducing a new NAS functionality

- How to support a common framework to support operator services including beyond connectivity services (e.g., Sensing, AI/ML) without impacting the minimal set of NAS functionalities

  - Investigate an architecture to support a logical connection between the UE and Core that is used to transfer control/data signaling for operator services

  - How to support control (e.g., establish, update or terminate) of the logical connection

  - How to transfer and route the control/data signaling between the UE and the service NF (e.g., NF for sensing) over the established logical connection

  - How to support mobility management for operator service

  - How to support authentication to support operator services

S2-2506868, IIT Bombay

This key issue aims to explore efficient mechanisms for UE-Core Network interaction in 6G systems for signalling/data collection to support connectivity services and/or beyond connectivity services. In this context, it is proposed to study the following aspects:

-  How to enable the 6G system to handle higher signalling load

-  How to reduce service delivery latency in the 6G System

-  Study the placement and functionalities of network functions to support such mechanisms

S2-2506871, Futurewei

KI #X (1.1.X) Enhanced NAS

Study whether and how to 6G can support enhanced NAS functionality:

- study whether and how to distribute NAS functionality such that handling routing of NAS signalling is separate from functionality handles basic registration and mobility management, and other functionality for enhanced services.

- study whether and how to enhance mechanisms and procedures for signalling for paging and reduce congestion in the control plane.

S2-2506903, Apple

The following initial KI is with reference to WT#1.1 bullet a):

This key issue will analyse whether and how to maintain connectivity between UE and legacy network functions, such as SMF, PCF, SMSF, etc., while enabling different mechanisms for the UE to have communication with new network functions that will be introduced in 6G.

S2-2506942, Ericsson

This key issue study the support of control signalling communication between UE and CN (i.e. Non-Access Stratum level communication). The key issue covers the following aspects

-	Identify if there is any potential improvement (e.g., minimized UE impact for verticals, lower number of messages both OTA, between RAN/CN and also inside CN) on the support for the UE connectivity-based functionalities in addition to the re-use of existing basic 5GS NAS mobility and session management handling mechanisms.

-	Investigate the UE-CN communication to support the UE interworking between 6G system and 5G system.

-	Investigate the possibility of designing a new common UE and CN communication mechanism for any new services without dependency to the UE connectivity-based functionalities.

NOTE: Potential dependency to RAN WGs and SA3 WG for security.

S2-2506975, TCL

The 6G network will introduce more core network services which can be provided to UE, such as sensing, computing, AI and so on. All of these services need their own non-access stratum connections. In current architecture, these connections are concentrated on AMF. Thus, it can be foreseen that the implementation of AMF will become more complicated and the traffic can be heavier and cause higher single-point failure risk. It will be preferred that 6G network can provide direct connection between UE and core network functions providing services. Follow issue shall be solved:

- How to improve RAN-Core network interface to connect UE directly to any core network functions without the help of AMF?

- How to support UE discover the core network functions to acquire services? How to route the UE uplink service requests?

- For downlink messages, how to keep UE reachable if there is no core network anchor such as AMF?

S2-2506988, Qualcomm

For operator services offered over the user plane  (e.g. data collection for UE side model training, location services over user plane, and potentially any new 6G service if decided to be offered over user plane), 3GPP should avoid defining completely different solutions at all layers, and instead identify potential underlying functionality that is common to any service. For instance, all services require to be discoverable and/or configurable in the UE, the UE needs to be authorized for receiving the specific service, a session needs to be established to provide a user plane transport path between the UE and the service, and 3GPP security needs to be established.

This key issue should identify underlying functionality that may be common to different operator services offered over the user plane, and study a common framework for those functionalities, including:

-	Service Discovery & configuration

-	Service authorization

-	Session configuration and establishment

-	Security

NOTE: Security aspects require coordination with SA3.

S2-2507055, Nokia, AT&T, T-Mobile US

1.1 Study the support for control plane signalling for 6G for connectivity services and/or beyond connectivity services, including the following:

a.	Whether and how to enable the introduction of a new non-access stratum functionality without impacting other non-access stratum functionalities.

b.	Whether and how to identify a minimal set of independent non-access stratum functionalities depending on the type of typical user equipment (e.g. IoT vs eMBB).

c.	Whether and how to develop generic mechanisms (e.g. authorization, discovery) for UE-Core Network interaction to support operator services.

S2-2507124, Huawei, HiSilicon

The NAS mechanism is expected to enable the operator to provide both connectivity services and beyond-connectivity services to the UE. This key issue is to study the improvements to NAS mechanism between the UE and the network to enable the agile rollout of connectivity services and beyond connectivity services without impacting existing NAS functionalities, including:

- How to avoid impacts on other non-access stratum functionalities when introducing a new non-access stratum functionality for both connectivity services and beyond connectivity services.

- How to support flexible NAS distribution considering the termination of the NAS message for both connectivity services and beyond connectivity services.

- How to develop a generic NAS control mechanism for agile deployment of beyond-connectivity services, including an efficient discovery mechanism of network provided services (e.g., computing, sensing, etc.) to enable the UE to invoke these services when required.

Editor's Note: New KIs, derived from other WTs, such as WT#3, 4, 5, 6, are to be added.

S2-2507135, ETRI

1.X. Key Issue #X: NAS Modularity and Extensibility

This key issue investigates how to enable modular evolution of NAS functionalities in 6G. The goal is to introduce new NAS capabilities (e.g., for beyond-connectivity services).

This key issue will investigate:

-	Whether and how new NAS functionalities can be introduced independently of existing ones.

-	Whether and how to define a minimal baseline NAS profile that can remain unaffected by additional extensions.

-	Definition of generic UE-CN interaction patterns for extensible NAS support.

S2-2507200, Lenovo

This key issue proposes to study how to design a generic mechanism for UE to Core Network interaction to support operator services, i.e. beyond connectivity services, over the user plane.

The key issue will investigate the following aspects:

-	How to establish the signalling exchange for operator services (i.e. beyond connectivity services) between the UE and the target core network NFs.

-	How to route the data for operator services between the UE and the target core network NFs.

-	How to support target NF selection.

S2-2507353, Xiaomi

In order to facilitate commercialization, achieve better customer experience and make a more successful generation, 6G design for UE-CN interaction for control signalling should consider the following:

- the system is architecturally modular, and implementation modular with as little independency from each self-contained module as possible;

- the system supports control signalling for connectivity services and beyond connectivity services;

- the system supports UE-CN interaction for control signalling over control plane;

- the system supports UE-CN interaction for control signalling over user plane;

- the design UE-CN interaction should be conceived considering at least the following aspects: cloud native, sustainability and energy efficiency, robustness and resiliency, etc.

Comparing to the existing 5G architecture on UE-CN interaction for control signalling, the following questions need to be addressed during the study on UE-CN interaction for control signalling in 6G:

-	Whether and how to further modulize connectivity services with self-contained modules which are independent from each other;

-	Whether and how to further modulize beyond connectivity services with self-contained modules which are independent from each other;

NOTE:	This WT discusses generic mechanism for all beyond connectivity services, specific details for beyond connectivity services covered by other WTs are not covered in this WT, specific details for beyond connectivity services not covered by other WTs are covered in this WT.

-	whether and how to enhance UE-CN interaction for control signalling over control plane considering at least the following aspects: cloud native, sustainability and energy efficiency, robustness and resiliency;

-	whether and how to enhance UE-CN interaction for control signalling over user plane considering at least the following aspects: cloud native, sustainability and energy efficiency, robustness and resiliency;

-	whether and how to define the principle to support the modulized connectivity services and beyond connectivity services on UE-CN interaction via control plane or via user plane with potential enhancements in the related study in this WT.

S2-2507378, NEC

This key issue (KI) will study to cover the following aspects:

- 	Whether and how to enable the introduction of a new non-access stratum functionality without impacting other non-access stratum functionalities

- 	Whether and how to identify a minimal set of non-access stratum functionalities that does not get impacted by additional non-access stratum functionalities.

**** First Change ****

5.X. Key Issue #X: Support of Common Aspects for Beyond Connectivity Services

This Key Issue focuses on studying the support for signalling for UE-Core Network interactions for beyond connectivity services, including the following:

- Identify what operator services can be supported over other than NAS for connectivity services

- Define architecture and procedures to support common functionalities for beyond connectivity services

  - How to support transfer and routing of signaling (i.e., control signaling, service-specific messages including data) for beyond connectivity services between UE and the CN

  - How to support transfer and routing of signaling (i.e., control signaling, service-specific messages including data) for beyond connectivity services between RAN and the CN

  - Whether and how to support generic mechanisms for UE-Core Network and RAN-Core Network interactions for beyond connectivity services

  - Investigate impact on interfaces between UE and CN, and RAN and CN

NOTE 1: Potential dependency to RAN WGs

NOTE 2: Potential impact on Interworking to be coordinated with KIs related to WT#2.

NOTE 3: UE-CN interactions for beyond connectivity services to be coordinated with KIs related to WT#3, WT#4, WT#5, WT#6.

NOTE 4: Potential security and privacy impact to be coordinated by SA3.

**** End of Changes ****