3GPP TSG-WG2 Meeting #170	S2-2507997

# Göteborg, Sweden, August 25-29, 2025									  revision of S2-2506327

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1495 | rev | 1 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to Outputs of Relative Proximity Predictions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UAS_Ph3 |  |  |  |  |  | Date: |  |  | 2025-08-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to the CT3 LS on time to collision prediction accuracy (S2-2506124/C3-252367), there are unclear places in the outputs of Relative Proximity Predictions for time to collision. This paper proposes to fix the issue by adjusting the format of the IEs under the time to collision prediction. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The main changes include: - Move the Confidence IE as sub-IE under Collision space. - Move the Confidence and Accuracy IEs as sub-IEs under Collision direction. - Update the description of the IEs under Time To Collision. #---r01--- Removing ”Accuracy” sub-IE for collision direction and time to collision as they are not applicable. #--- |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The outputs of the Relative Proximity Predictions for time to collision is unclear and may cause misunderstanding during implementation in stage 3. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.19.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

>>>> Start of Changes <<<<

### 6.19.3	Output analytics

The NWDAF services as defined in the clauses 7.2 and 7.3 are used to expose the analytics.

-	Relative proximity statistics information is defined in Table 6.19.3-1.

-	Relative proximity predictions information is defined in Table 6.19.3-2.

Table 6.19.3-1: Relative proximity statistics

| Information | Description |
| --- | --- |
| UE group ID or set of UE IDs | Identifies the UE(s) for which the statistics apply by a list of SUPIs or GPSIs, or a group of UEs by a list of Internal-Group-Ids. |
| Time slot entry (1..max) | List of time slots during the Analytics target period. |
| > Time slot start | Time slot start within the Analytics target period. |
| > Duration | Duration of the time slot. |
| > UE proximity attributes | Observed proximity data. This includes distance between UEs in the group/list and other proximity data as mentioned in clause 6.19.1 |
| >> relative proximity information | Observed proximity information. |
| >> Sampling Ratio | Percentage of UEs accounted based on proximity criteria. |
| NOTE 1: Analytics subset that can be used in "list of analytics subsets that are requested" and "Preferred level of accuracy per analytics subset". |  |

Table 6.19.3-2: Relative proximity predictions

| Information | Description |
| --- | --- |
| UE group ID or set of UE IDs | Identifies the UE(s) for which the statistics apply by a list of SUPIs or GPSIs, or a group of UEs by a list of Internal-Group-Ids. |
| Time slot entry (1..max) | List of predicted time slots. |
| >Time slot start | Time slot start time within the Analytics target period. |
| > Duration | Duration of the time slot. |
| > UE proximity attributes | Predicted proximity data. This includes distance between UEs in the group/list and other proximity data as mentioned in clause 6.19.1. |
| >> relative proximity information | Predicted proximity information. |
| >> Confidence | Confidence of this prediction. |
| >> Sampling Ratio | Percentage of UEs accounted based on proximity criteria. |
| Time To Collision (TTC) information (NOTE) |  |
| > Time To Collision | Time until a collision with another UE happens. |
| >> Collision space | Predicted space of the collision at the collision time, e.g. Ellipsoid point with altitude and uncertainty ellipsoid in TS 23.032 [34]. |
| >>> Confidence | Confidence of the prediction for Collision space |
| >> Collision direction | Predicted collision direction of the target UE at the collision time. |
| >>> Confidence | Confidence of the prediction for collision direction. |
| > Confidence | Confidence of the prediction. |
| NOTE: Analytics subset that can be used in "list of analytics subsets that are requested" and "Preferred level of accuracy per analytics subset". |  |

>>>> End of Changes <<<<