3GPP TSG-WG2 Meeting #170	S2-2508001

# Göteborg, Sweden, August 25-29, 2025 									   revision of S2-2506328

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1496 | rev | 1 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to Movement Behaviour Analytics for Pre-flight Planning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UAS_Ph3 |  |  |  |  |  | Date: |  |  | 2025-08-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | USS/UTM may request (via NEF) NWDAF analytics service Movement Behaviour Analytics as assistance information for pre-flight planning. The movement behaviour analytics provides analytics information. For UAV movement, usually 3D direction is considered. For pre-flight planning, the information on 3D direction and corresponding velocity in each dimension can be used for better plan flight path. Altitude informaiton is one of the important locations information attributes for UAV flight path relevant Movement Behaviour Analytics and, therefore, it is used, for instance, in Step 9 of clause 5.12.2 of TS 23.256. However, the preferred granularity of location information for Movement Behaviour Analytics only includes longitude and latitude level, altitude attribute is missing. Lacking altitude information in the Movement Behaviour Analytics may cause inaccurate flight path planning. This paper adds altitude level to the preferred granularity of location information for Movement Behaviour Analytics and adds an input from AF on UE alititude information, which enables the NWDAF to fetch the altitude information from the AF in cases when a UAV UE reports its altitude values over application layer as specified in Option A, clause 5.16.3 of TS 23.256. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The main changes include: - Added height/altitude (if available) level to the preferred granularity of location information for Movement Behaviour Analytics. - Added input from AF to NWDAF UE altitude/height information for the analytics. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Movement Behaviour Analytics cannot be used to support pre-flight planning when altitude information is needed for the planning. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.21.1, 6.21.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

>>>> Start of Changes <<<<

## 6.21	Movement Behaviour Analytics

### 6.21.1	General

The movement behaviour analytics provides analytics information (statistics or predictions) regarding e.g. the number, direction and velocity of UEs during an analytics target period in a target area. The analytics of movement behaviour requires to collect UE movements in a certain region in a certain period of time. In addition, the consumers can further derive the congestion situation in the target area through analytics.

The NWDAF provides movement behaviour analytics to a NF (e.g. NEF, AF).

The consumer of these analytics shall indicate in the request or subscription:

-	Analytics ID = "movement behaviour";

-	Target of Analytics Reporting: any UE, a list of UEs;

-	Analytics Filter Information:

-	Area of Interest (AOI): restricts the scope of the movement behaviour analytics to the provided area. The AOI may be described as shown in clause 5.5 of TS 23.273 [39];

-	Optionally, the list of analytics subsets that are requested among those specified in clause 6.21.3;

-	An Analytics target period indicates the time period over which the statistics or predictions are requested;

-	Optionally, preferred level of accuracy of the analytics;

-	Optionally, preferred level of accuracy per analytics subset (see clause 6.21.3);

-	Optionally, preferred granularity of location information: "longitude, latitude, and/or altitude (if available) level";

-	Optionally, preferred orientation of location information: ("horizontal", "vertical", "both")"; and

-	Optionally, maximum number of objects.

>>>> Next Changes <<<<

### 6.21.2	Input data

The NWDAF collects the UE location and velocity information within the area of interest from the sources listed in Table 6.21.2-1.

Table 6.21.2-1: Data collection by NWDAF for "movement behaviour" analytics

| Information | Source | Description |
| --- | --- | --- |
| UE ID | AMF | SUPI; a list of SUPIs |
| UE locations (1..max) | AMF | UE positions |
| >UE location |  | TA or cells that the UE enters |
| >Timestamp |  | A time stamp when the AMF detects the UE enters this location |
| UE altitude information (1..max) (NOTE X) | AF | UE altitude information |
| >UE altitude |  | Altitude information of the UE |
| >Timestamp |  | A time stamp when the altitude information has been collected |
| Fine granularity locations (1 …max) | LCS (NOTE 1) | UE locations |
| …>Timestamp |  | Time information associated with the collected information |
| …>LCS QoS |  | The achieved LCS QoS accuracy (see TS 23.273 [39]) |
| …>Location estimate |  | Geographical location or location in local coordinates (as defined in TS 23.032 [34]) |
| …>Velocity estimate |  | Speed and direction information |
| …>Indoors/outdoor indication |  | Indicate the location estimate is indoor or outdoor |
| …>Age of location |  | Valid period of the location estimate |
| NOTE 1: The procedure to collect location data using LCS is described in clause 6.2.12. NOTE X: The UE altitude information is applicable for UAV UE only as specified in TS 23.256 [53]; timestamp in the UE altitude information is optional. |  |  |

>>>> End of Changes <<<<