3GPP TSG-SA2 Meeting #170	S2-2508047

25 - 29 August 2025, Goteborg, Sweden                         (Revision of S2-2507963/6820/4931)

# Source:	China Mobile

# Title:	Dynamic NITZ Update

# Document for:	Approval

# Agenda Item:	30.1

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: New WID on Dynamic NITZ Update

Acronym: TEI20_DNU

Unique identifier:	TBD

Potential target Release:	Rel-20

1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | X |  |
| No | X | X | X |  |  |
| Don't know |  |  |  |  |  |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a Feature

| X | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| N/A |  |  |  |

2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| N/A |  |  |

3	Justification

After the UE is registered to the 5GC, the AMF is allowed to configure the NITZ (Network Identity and Time Zone) to the UE via UE Configuration Update procedure as defined in 3GPP TS 23.502.

When referring to the definition in 3GPP TS 22.042, the following text is captured as below:

The Network Operator may change the network identity at any time. However the change of network identity need not force immediate transfer of information to the MS.

So far, the only way to update the network identity is by OAM via configuration in the AMF, which does not support dynamically update of the NITZ at any time to the UE:

4	Objective

The objective of this work item:

WT-1: Ability of the 5GC to dynamically update the Network Identity to the AMF, i.e. the PCF updates the Network Identity based on operator policy, so that the AMF can then provide the NITZ to the UE by reusing the current UCU procedure.

NOTE 1: Inter-PLMN cases are not covered in this WID, i.e. the NITZ is updated to UE only by the UE’s PLMN holding the UE’s subscription.

NOTE 2: It is assumed that a PLMN, which deployed dynamic update of NITZ via NAS, also supports the delivery of default NITZ via NAS when the conditions for the dynamically provided NITZ are no longer applicable. This is to avoid the possibility of displaying inaccurately at the UE side the dynamic NITZ, which is only applicable in certain conditions.

NOTE 3: This is only applicable for non-roaming scenario.

TU estimates and dependencies

| Work Tas ID | TU Estimate (Study) | TU Estimate (Normative) | RAN Dependency (Yes/No/Maybe) | Inter Work Tasks Dependency |
| --- | --- | --- | --- | --- |
| WT-1 |  | 0.5 | No | None |

5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TS 23.501 | Support for dynamic network identity mechanism. | SA#108 (Jun 2025) |  |
| TS 23.502 | Network Identity received from the PCF is stored as UE Context in AMF | SA#108 (Jun 2025) |  |
| TS 23.503 | Network Identity management is involved as Access and Mobility management related policy and can be provided by the PCF to the UE via AMF. | SA#108 (Jun 2025) |  |

6	Work item Rapporteur(s)

Aihua Li, China Mobile <liaihua@chinamobile.com>

7	Work item leadership

SA2

8	Aspects that involve other WGs

None identified.

9	Supporting Individual Members

Supporting IM name

China Mobile

SK Telecom

China Unicom

CBN

Huawei

CATT

Tencent

Ericsson

MediaTek Inc

AT&T

Deutsche Telekom

ZTE