SA WG2 Meeting #170	S2-2507502

25 - 29 August 2025, Goteborg, Sweden

Source:	ZTE

Title:	Solution#4 update

Document for:	Approval

Agenda Item:	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2 / Rel-20

Abstract: This contribution proposes the triggering procedure and NF selection for UE data collection.

# 1. Discussion

3GPP SA2 Release 20 has approved the study item "AI/ML Core Network Enhancements Phase 2", which includes the research task of "whether and how to support standardized user plane transmission for AI/ML-related UE data collection." This paper proposes a NWDAF selection mechanism for the NWDAF that supports user plane UE data collection and some trigger procedures for the UE training data collection.

# 2. Proposal

This paper proposes to update solution #4 for Key Issue #1 to TR 23.700-04 as follows.

---------- Start of changes ----------

6.4	Solution #4: UP Building for UE data collection

6.4.1	High level principles

This solution addresses KI#1 by defining triggering mechanisms, NF selection and UP building procedures for UE data collection. The data collection process can be initiated by either a trusted AF or an untrusted AF.

6.4.2	Description

This solution addresses Key Issue #1.

This solution proposes a user plane connection between an NF and UE, with some enhancements in the UP building request. The NF received the request from AF and trigger UP building. After UE training data collection finished, the NF checks the training data and sends them to NEF/AF. This solution is based on the user plane positioning procedure as described in clause 6.18.1 of TS 23.273 [6]. Besides UP building procedure, The data collection trigger and NF selection procedures for UE data collection are also included in this solution.

6.4.3	Procedures

6.4.3.1       The UP building procedure

UEr

NWDAF

AF

NEF

UPF

SMF

AMF

0. Data collection triggered by AF or untrust AF

1. UP building request

2. URSP matching

3. PDU session establishment

4.N4 session establishment

5. PDU session establishment accept

6. UP binding request

7. UP binding response

8. User plane data collection acknowledge

9. UE training data transfer

10. UE training data check and confirm

11. UE training data transfer

Figure 6.4.2-1: User plane building between an NF and UE

NOTE:	The user consent check is not addressed of this solution.

Editor's note:	Whether and how to separate UE data collection traffic over user plane from end user generated user plane traffic is FFS.

Editor's note:	The service operations will be updated.

0.	The data collection triggering procedures are defined in clause 6.4.3.2 and 6.4.3.3

1.	The NWDAF send UP building request to UE via AMF and gNB for AI/ML data collection, the message may contain an NWDAF address information, UP binding ID (i.e. to associate the user plane connection to be established with the target UE), purpose of building user plane (e.g. AI/ML user plane data collection). If the NWDAF received UE ID from AF, it send the UP building request to the UE. If the NWDAF received location information, it retrieve the list of UE ID in the area from AMF and send the UP building request accordingly.

Editor’s note:  Whether NWDAF transfer the collected data is FFS.

2.	The UE performs URSP rule matching based on the received parameters. For example, based on the AI/ML data collection indication within the URSP rule's Connection Capability of Traffic Descriptor (TD) to match the corresponding Routing Selection Descriptor (RSD) parameters and decides whether to establish a new PDU session or use an existing PDU session for transmitting AI/ML-related application traffic..

3.	The UE sends a PDU session establishment request to the SMF and the request message includes the DNN and S-NSSAI.

4.	The SMF selects the appropriate UPF based on the S-NSSAI and DNN and initiates the N4 session establishment.

5.	The SMF replies with a PDU session establishment acceptance message to the UE and the PDU connection service between the UE and the NF is established.

6.	The UE initiates the user plane connection binding process. It sends a user plane connection binding request to the NF and the request message includes the user plane binding ID received in step 1.

7.	The NWDAF replies with a user plane connection binding acceptance message to the UE.

Editor's note:	It is FFS whether and how to binding UP.

8.	UE sends an acknowledgement to the NF through AMF to indicate a success of user plane connection establishment for data collection or a failure to utilize the user plane connection.

9.	The UE sends UE training data for AI/ML through the user plane to the NF.

10.	The NWDAF check whether the collected UE training data align with the requirements (e.g. number of data sample, type of data) from AF.

11.	The NWDAF transfer the AI/ML-related data collected from the UE to the NEF. This message may include one or more data types requested by the AF along with the corresponding data.

12.	The NEF maps the information received from the NF into external information (e.g. mapping SUPI to GPSI) and transfer the training data to the AF.

Editor's note:	How to support visibility of standardized data is FFS.

6.4.3.2     UE training data collection triggered by trust AF

NWDAF

AF

NRF

UDM

1. Nnrf_NFDiscovery_Request

2. Nnrf_NFDiscovery_Response

3. AI/ML training data collection request

4. Parameters verification and User consent check

5. AI/ML training data collection response

Figure 6.4.2-2: Trust AF triggered UE training data collection

- 1. AF invokes NRF services to send a message such as Nnrf_NFDiscovery_Request, which may include the following information: NF Type = "NWDAF", AI/ML user plane data collection capability, Location information (e.g. TAI, AoI), Time information (e.g. data collection time period).

- 2. NRF responds with  Nnrf_NFDiscovery_Response, which includes information about one or more NWDAF(s).

- 3. AF  can invoke NWDAF services such as Nnwdaf_DataManagement_Subscribe or a new service specifically for AI/ML training data collection. The training data collection request may include following parameters:

- Indication for AI/ML-related (user) data collection

- Application information, such as Application ID, AF ID

- UE information, including UE ID (e.g. GPSI), UE Group Information (e.g. Group ID) or indications for data collection from any UE.

- Time-related information, including data collection time window (i.e. the time duration for data collection), required data validity period.

- Location/area information for data collection, such TAI, Cell ID.

- Data type, for example, reference signal information (e.g. CSI, SSB) and corresponding RSRP information, UE positioning information (e.g. latitude and longitude), UE antenna orientation information, codebook information, and radio measurement data information

- Data sample information, such as number of data samples and required data quality thresholds

- Subscription and notification address for collected AI/ML-related (user) data information

- 4. NWDAF performs user consent confirmation for one or more parameters in the AF request. This may involve invoking UDM services, where NWDAF sends relevant parameters (e.g. UE ID) to UDM for user consent check and UDM returns the validation result.

- 5. NWDAF responds to AF with a response message, which may include: Assigned AI/ML data collection reference ID or  AI/ML data collection association ID, Authorized UE list or unauthorized UE list, expected data collection completion time or time when collected data will be provided to AF.

6.4.3.2     UE training data collection triggered by untrust AF

AF

UDM

NEF

NRF

NWDAF

1. AI/ML training data collection request

2. Parameters mapping, user consent check, NWDAF selection

3. AI/ML training data collection request

4. AI/ML training data collection response

5. UE training data collection completion time

Figure 6.4.2-3: Untrust AF triggered UE training data collection

1. The untrsuted AF sends a AI/ML training data collection request to NEF, providing parameters listed in the step 3 of 6.x.2.2.

2. NEF may perform various operations, including mapping external parameters to core network internal parameters, such as converting location information into TAI  and Cell ID. It can also invoke UDM services (e.g. Nudm_SDM_Get) for parameter mapping, like converting GPSI to SUPI or mapping an external group ID to an internal group ID. Additionally, NEF is responsible for user consent confirmation for one or more UE IDs in the AF request, by sending the UE ID to UDM for verification and receiving the result. NEF also performs NWDAF selection, where if the selection is based on location information (e.g. TAI), NEF may invoke NRF services and find a NWDAF has user plane AI/ML data collection capability and covers the target TAI.

If it is based on UE information (e.g. UE ID), NEF may invoke UDM services to query the NWDAF serving the UE.  Alternatively, NEF may select NWDAF based on local configurations. Finally, NEF is also responsible for assigning a transaction reference ID or subscription association ID.

3. NEF sends the AI/ML data collection request to NWDAF, with mapped parameters and authorized UE list.

4. The NWDAF sends a response with estimated UE training data collection completion time to NEF.

5. The NEF forwards UE training data collection completion time to AF.The message can also include authorized UE list or unauthorized UE list, types of data collection authorized or not authorized for one UE or group of UE, together with the assigned reference ID or association ID.

6.4.3.2   NF profile registration and update procedure

NRF

NWDAF

1.Nnrf_NFmanagement_NFResiter/NFupdate_Request

2. NF profile storage/update

3. Nnrf_NFManagement_NFRegister/Update_Response

Figure 6.4.2-4: NWDAF capability registration and update

1. NWDAF registers or updates its AI/ML user plane data collection capability and the time that it support AI/ML user plane data collection capability to NRF.

2. NRF stores or updates the capability

3. NRF returns registration or update result indication.

6.4.4	Impacts on services, entities and interface

The NWDAF  needs to be enhanced to build a user plane with UE. The NWDAF may provide a purpose of building user plane (e.g. AI/ML user plane data collection) and data types/sample information to UE in the UP building request message. Also, the NWDAF needs to check and confirm the collected training data are aligned with the requirements before it sends to NEF/AF. On the other side, the NWDAF registers its AI/ML data collection capability with NRF, receives AI/ML data collection parameters from NEF or trust AF, and performs user consent verification for the provided parameters. It then sends a response back to NEF or trust AF, which may contain a list of authorized/unauthorized UEs and the expected data collection completion time.

The trust AF is responsible for selecting the appropriate NWDAF, sending data collection request with AI/ML data collection parameters to NWDAF, and receiving a response from NWDAF.

The untrust AF is responsible for sending data collection request with AI/ML data collection parameters to NEF.

The NEF receives AI/ML data collection parameters from untrust AF, performs user consent verification, and carries out NWDAF selection. Additionally, NEF forwards the AI/ML data collection parameters to NWDAF and subsequently receives a response from NWDAF, including the expected data collection completion time. It then sends a response to AF, which may include data collection authorization details and expected data collection completion time.

---------- End of changes ----------