SA WG2 Meeting #170	S2-2507503

25 - 29 August 2025, Goteborg, Sweden

Source:	ZTE

Title:	Malicious traffic identification and mitigation

Document for:	Approval

Agenda Item:	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2 / Rel-20

Abstract: This contribution proposes ways for malicious traffic detection and mitigation.

# 1. Discussion

In the 5G core network, malicious traffic typically refers to data flows used to carry out network attacks, fraud, or illegal activities. Such traffic may appear similar to normal user behavior but is often intended to steal personal information, spread malware, or conduct fraudulent actions.

Characteristics of malicious data flows include:

Abnormal data usage patterns, such as large volumes of data transferred in a short time;

Unauthorized access to specific IP addresses or domains;

Interactions with known malicious applications or services;

Unusually long session durations or frequent session establishment and release;

Common examples of malicious traffic include:

Phishing websites: These sites mimic the interfaces of banks, social platforms, or other trusted entities to lure users into entering sensitive information (such as account credentials, passwords, or verification codes).

Telecommunication fraud websites or apps: For example, websites or apps pretending to be customer service or government agencies, aiming to trick users into transferring money or installing malicious software. Such traffic often has identifiable service data flow characteristics.

Malicious emails: Emails transmitted via protocols such as SMTP or IMAP may contain phishing links or malicious attachments.

To ensure network security and user protection, the 5G core network must include proactive mechanisms to detect, classify, and block malicious traffic in real time

# 2. Proposal

This paper proposes to add a solution for Key Issue #2 to TR 23.700-04 as follows.

---------- Start of change ----------

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  | Use Cases (optional) |  |
| --- | --- | --- | --- | --- |
| Solutions | <Key Issue #1> | <Key Issue #2> | <Use Case #1> | <Use Case #2> |
| #1 |  | X | X |  |

---------- Next change (all new text) ----------

6.x	Solution #X: Malicious traffic identification and mitigation

6.x.1	High level principles

This solution addresses KI#2.The NWDAF is enhanced to support ‘traffic analyse’ Analytics ID. The NF consumer can input the Application identifier, QFI or IP-5 tuple to the ML Model related to this Analytics ID and the model outputs a binary value (i.e. it is a normal application/data flow or malicious application/data flow) and optionally the malicious type.

6.x.2	Functional Description

6.x.2.1	General Description

This solution enables intelligent and dynamic threat mitigation by allowing NWDAF to collect and analyze multi-source network data (including flow characteristics, usage patterns), detect suspicious or malicious flows or applications, and coordinate with PCF, SMF, and UPF to enforce real-time policy control and traffic blocking.

6.x.2.2	Input data of the Analytics

| Information | Source | Description |
| --- | --- | --- |
| IP-5 tuple | AF/UPF/SMF | The IP 5-tuple includes the source IP address, destination IP address, source port, destination port, and transport layer protocol (e.g. TCP or UDP). |
| Malicious information | TBD | The ground truth data used for ML Model training. Whether the data flow or application ID is malicious and the type of malicious data at some time stamps in the past, used as label for model training. |
| User Data Usage Measures（NOTE 1） | SMF/UPF | Data usage information (e.g., uplink and downlink byte counts, packet counts). This can be used to detect malicous data usage patterns (such as sudden bursts of high traffic in a short time), assisting in the identification of fraudulent sessions . |
| User Data Usage Trends（NOTE 1） | SMF/UPF | Statistical trends of data usage over a period of time. This can be used to analyse user behavior and identify traffic that deviates from normal behavior patterns. |
| Session Management Event（NOTE 1） | SMF | Including events such as PDU session establishment, modification, and release. These can be used to identify abnormal behaviors involving frequent session establishment and release. |
| URL list | UPF | If available. Malicious URLs often contain misleading domain names, unusual file extensions (e.g., .exe, .scr), or encoded characters to obfuscate intent. They may also include suspicious parameters like token=, login=, cmd=, or redirect= commonly used in phishing. |
| time stamp | - | The time stamp of the collected data |
| NOTE 1: These events can be filtered by Application ID or Traffic Filtering Information (e.g. IP 5-tuple) to narrow the subscription scope. |  |  |

Editor’s Note: It is FFS whether and how to collect the malicious information (ground truth data).

Editor’s Note: Input data to identify malicious packets in a traffic flow is FFS.

6.x.2.2	Output data of the Analytics

| Information | Description |
| --- | --- |
| Flow identifier | QFI, Application ID or IP 5-tuple. |
| > Malicious identification | Whether the data flow or Application ID input by the NF consumer is malicious. |
| > Malicious type | Malicious type of the identified Application ID or data flow: e.g. Phishing websites, Telecommunication fraud websites or apps, Malicious emails. |
| > suspicious identification | Whether the data flow or Application ID input by the NF consumer is suspicious |

6.x.3	Procedures

NWDAF

SMFr

UPFr

AFr

NF consumer

0. Discover appropriate NWDAF

1.Nnwdaf_AnalyticsSubscription/Nnwdaf_AnalyticsInfo_request

2. Data collection

3. Determine the applications or flows are malicious or not

4.Nnwdaf_AnalyticsSubscription_Notify/Nnwdaf_AnalyticsInfo_response

5. Policy update based on the Analytics results

Figure 6.x.3-1:  Analytics procedure for flow or application

0. The NWDAF has registered support for the “traffic analysis” analytics ID into the NRF. NF consumers (e.g. PCF, SMF) discover and select the appropriate NWDAF through the NRF using the existing procedure.

NOTE: The PCF/SMF may trigger the Analytics request based on local configuration.

1. The NF consumer sends a analytics request or analytics subscription to the NWDAF, which includes the Analytics ID = 'traffic analyse'. Target of Analytics reporting = Any UE/list of UE IDs or Any UPF or list of UPF ID; Analytics filtering information = combinations of DNN, S-NSSAI, AoI. The analytics request or analytics subscription may also include Application ID, QFI or IP 5-tuple and other parameters defined in clause 7.2.2 and 7.3.2 TS 23.288 [5].

2. The NWDAF collects model training or Analytics data from SMF, UPF and AF as specified in 6.x.2.2.

3. The NWDAF performs analytics to determine whether the data flow identified by the QFI is malicious, whether the Application ID is malicious, and whether the data flow associated with the IP 5-tuple is considered malicious. If the application or data flow is identified as malicious, the NWDAF may also indicate the type of malicious traffic.

4. The NWDAF returns to the NF Consumer an indication of whether the data flow (e.g. identified by QFI or IP 5-tuple) or the Application ID is considered malicious. Optionally, the NWDAF may also provide the associated type of malicious traffic, if applicable.

5. Based on the Analytics result received, the PCF makes a policy decision on whether to update the PCC rules (e.g. setting the Gate Status of the PCC rule associated with a specific QoS Flow to "closed"). If the NWDAF indicates that a specific Application ID is identified as a source of malicious traffic, the PCF may identify all PCC rules associated with that Application ID and set the Gate Control of each corresponding PCC rule to "closed".

Subsequently, the PCF updates the session management policy towards the SMF, including the modified PCC rules. The SMF then generates the corresponding QER(s)/FAR(s) based on the updated PCC rules and sends them to the UPF. Based on the received FAR(s)/QER(s), the UPF blocks or discards the suspicious or malicious packets accordingly.

If the NF consumer is SMF, the SMF may update FAR based on the Analytics result without instruction from PCF.

6.X.4	Impacts on services, entities and interface

NWDAF:

- Registers or updates traffic analyse capability (a new Analytics ID) to NRF

- Receives analysis requests from consumer NF

- Collects analytics input data defined in 6.X.2.2

- Outputs whether the application ID or flow is malicious and the type of maliciousness

- NF consumer (e.g. PCF, SMF):

- Discovers NWDAF that can support traffic analyse capability (a new Analytics ID) from NRF

- Sends Analytics request requests to NWDAF

- Receives analysis results returned by NWDAF

- Updates PCC rules or FAR based on the analytic results

---------- End of changes ----------