3GPP TSG SA WG2 Meeting #170	S2-2507519

25 - 29 August 2025, Goteborg, Sweden	(revi                si(revision of S2-2506515)

Source:	China Mobile

Title:	Solution#27 update on support of user plane management analytics

Document for:	Approval

Agenda Item:	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2 / Rel-20

Abstract: Proposed Solution#27 update on support of user plane management analytics.

1. Discussion

1. The UPF can be a consumer to the analytics data from NWDAF, to control the number of DLDR (Downlink Data Report) messages sent to SMF by e.g. adjusting message frequency; or apply downlink traffic suppression (e.g. selective packet dropping) for AS-initiated traffic bursts.

Editor's note: It is FFS whether the UPF can be a consumer or not.

- 2. As indicated in Table 6.27.2.2, the data can be collected from both UPF and OAM.

Editor's note: Collection of data from both UPF and OAM is FFS.

- 3. It is up to the UPF’s implementation to detect traffic hijacking and traffic spoofing, one NOTE is added to make clarification.

Editor's note: It is FFS whether and how the UPF can detect traffic hijacking and traffic spoofing.

- 4. To reduce UPF reporting load, UPF reports the data only based on the subscription, these two ENs below can be removed.

Editor's note: It is FFS whether the UPF can reports the specific data based on operator's pre-configuration or not.

Editor's note: It is FFS if configuration needs to be done by the UPF itself or via the SMF using analytics.

- Text Proposal

It is proposed to capture the following changes in TR 23.700-04.

# * * * * First change * * * *

6.27	Solution #27: Support of user plane management analytics

6.27.1	High level principles

In this contribution, a new NWDAF-based analytics, plane management analytics, is introduced to support user plane management, including.

-	Based on consumer's request, the user plane management analytics is able to provide the statistics and predictions of the characteristics of the traffic to be transferred via user plane, UP healthy status, cause of (potential) UP performance degradation, etc.

-	The consumer may take the outputs of the user plane management analytics into account to mitigate or prevent the user plane issues, reserve or allocated resources for user plane date transfer, update corresponding policies; and therefore, to maintain the health working condition of user plane and the corresponding UPF.

-	In order to derive the analytics outputs, the NWDAF may collect the relevant input data from different data sources, e.g. UPF, SMF, NRF OAM, AF, etc.

-	Corresponding procedure is also detailed in this contribution.

6.27.2	Description

6.27.2.1	General Description of UP Management Analytics

This solution aims to address the issues in KI#2, including:

-	How to provide analytics (e.g. enhancing existing in TS 23.288 [5] or introducing new) including the aspects of required inputs/outputs to improve performance of user plan.

-	Which consumers can subscribe/request the analytics and how the consumers use the analytics to take actions to improve user plane performance.

-	Whether and how to train ML models and perform the inference for the user plane analytics, considering solutions that minimize the load on Control Plane NFs.

Based on the two use cases of KI#2 documented in clause 5.1 of TR 23.700-04, the abnormal packets may cause some issues such as UE malfunctions, UPF performance degradation and system errors. Therefore, it would be beneficial to provide the statistics and/or perdition information of traffic characteristics from/to the application server. By taking this information into account, the network can take action to improve the network performance; for example the network would be able to allocate or reserve sufficient resources for data transmission, determine the appropriate configuration for user plane management. It would be also helpful to provide the statistics and/or perdition of UP working status and the corresponding causes of (potential or detected) UP failure or UP performance degradation. Therefore, the consumer of the analytics could take the above information into account to maintain the healthy working condition of user plane in a failure preventative manner.

The traffic characteristics may include the UL/DL/ round trip of: Data Burst Size, traffic patterns, traffic volume or the number of data packets, traffic rate (e.g. packet rate, bit flow rate, etc.), delay, loss rate, throughput, packet size distribution, jitter, (average) packet interval, etc.

The service consumer of the analytics may be a UPF, SMF, PCF, OAM, etc.

The consumer of these analytics indicates the following information in the request or subscription:

-	Analytics ID = "User Plane Management"

-	Target of Analytics Reporting: a list of NF instance ID(s) or NF Set ID(s), any UE, specific UE or group of UEs. E.g. the UPF (instance/set) IDs (list of) SUPIs or GPSIs.

-	Optional, matrix for UP performance level evaluation, e.g. the thresholds of the parameters of traffic characteristics.

-	Optional, application ID(s).

-	Optional, Area of Interest (AOI(s)).

-	Analytics Filter Information optionally includes:

-	(list of) UPF ID(s) .

-	DNN

-	Area of Interest (AOI(s))

-	S-NSSAI and NSI ID

-	RAT Type

-	List of analytics subsets that are requested, (see clause 6.27.2.3).

-	IP Packet Filter Set or PDR

-	An Analytics target period indicates the time period over which the analytics are requested.

-	In a subscription, the Notification Correlation Id and the Notification Target Address are included.

-	Optionally, reporting Thresholds, which apply only for subscriptions and indicate conditions on the levels to be reached for the reporting of respective analytics subsets (see clause 6.23.3 of TS 23.288 [5]).

6.27.2.2	Input Data

The NWDAF supporting UP management analytics shall be able to collect information from AF, OAM and 5GC NFs.

Table 6.27.2.2-1: Input data related to UPF performance

| Information | Source | Description |
| --- | --- | --- |
| UPF info | UPF/ SMF/NRF | Identifying the UPF(s) that is associated to the collected information, e.g. the NF instance ID(s) or NF set ID(s) of the UPF(s), or the IP address or FQDN of the UPF. |
| UPF load | NRF | Load information indicates the current load of the UPF, e.g. CPU, memory, or percentage of load information. |
| DL/ UL Throughput | UPF/ AF | The throughput UL/DL over the measurement period, .e.g. peak value or average value. |
| UPF service area | UPF/ NRF | The service area information of the corresponding UPF(s). |
| Capacity GTP | UPF/ OAM | The maximum achievable UL and/or DL GTP transmission rate between PSA UPF and NG-RAN and between PSA UPF and UE, e.g. as defined in clause 6.3.11 of TS 28.554 [16]. |
| Data transmission over N3 interface | UPF/ OAM | Data packets or traffic volume of incoming and/or outgoing GTP data packets on the N3 interface (from (R)AN to UPF), e.g. as defined in clause 5.4.1 of TS 28.552 [15]. |
| Data loss over N3 interface | UPF/ OAM | Incoming GTP Data Packet Loss or Rejection in UPF and/or outgoing GTP Data Packet Loss that are not successfully received at gNB over N3, e.g. as defined in clause 5.4.1 of TS 28.552 [15]. |
| Throughput of N3 interface | UPF/ OAM | Upstream and/or downstream throughput at N3 interface, e.g. as defined in clauses 6.3.3 and 6.3.4 of TS 28.554 [16]. |
| Data transmission over N6 interface | UPF/ OAM | The link usage of N6 interface, including the usage of incoming traffic and/or outgoing traffic, e.g. as defined in clause 5.4.2 of TS 28.552 [15]. |
| N4 interface information | SMF/ OAM | The N4 interface information may include the number of requested, successful or failed N4 session establishments and/or N4 session reports, e.g. as defined in clause 5.4.3 of TS 28.552 [15]. |
| Data transmission over N9 interface | UPF/ OAM | The Number of incoming and/or outgoing GTP data packets on the N9 interface associated to the PSA UPF, e.g. as defined in clause 5.4.4.2 of TS 28.552 [15]. This may include the overall, successful or failed data packets transfer. |
| Delay | SMF/UPF/ OAM | The UL/DL/ round trip delay between UPF and NG-RAN, or between UE and UPF, e.g. as defined in clauses 5.4.7, 5.4.8 and 5.4.9 of TS 28.552 [15] or collected from UPF/ SMF. The delay could be the values of average, distribution, maximum, or variance. |
| Time stamp or measurement period | UPF/SMF/NRF/OAM | The associated timing information of the collected information. |

Table 6.27.2.2-2: Input data related to the service flow via user plane

| Information | Source | Description |
| --- | --- | --- |
| Application info | UPF/ AF | Identifying the application associated to traffic/ service flow, e.g. application ID, IP address or FDQN of the application. The data transmission might be from or to the application server. |
| UE IDs | SMF/ UPF/ AF | Identifying the UE(s) (e.g. SUPIs, GPSIs) that are associated to the corresponding data transmission from/to the application server(s). |
| Transport protocol information | SMF/UPF/AF/ | The transport protocol associated to the traffic transmission, e.g. RTP, SRTP, TCP, UDP, DCCP, SCTP, QUIC, etc. |
| IP Packet Filter Set or PDR | UPF/ AF | IP Packet Filter set as defined in clause 5.8.2 of TS 23.501 [2] or the PDR for traffic detection at UPF. |
| Data communication type | SMF/UPF/AF | Identify the data communication type, e.g. IP type, Ethernet type, or unstructured type data communication. |
| Traffic characteristics | UPF/ AF | The traffic characteristics may include the UL/DL/ round trip of: Data Burst Size, traffic patterns, traffic volume or the number of data packets, traffic rate (e.g. packet rate, bit flow rate, etc.), packet delay, loss rate, throughput, etc. within the measurement period. |
| Variation of traffic characteristics | UPF/ AF | The variation or standard deviation of the parameters belong to traffic characteristics within the measurement period, e.g. the variation of the traffic volume or data burst size, etc. within a given time period. |
| (Abnormal) data packet information | UPF | For example, malformed packets, duplicate packets, packet retransmission, traffic hijacking and traffic spoofing. The UPF may report the abnormal data packet base on operation's pre-configuration or information in consumer's request. |
| Time stamp or measurement period | UPF/SMF/NRF/OAM | The associated timing information of the collected information |

Editor's note: It is FFS whether and how the UPF can detect traffic hijacking and traffic spoofing.

NOTEY:	UPF reports the specific data based on subscription from NWDAF by using, e.g.,analytics filter, to reduce UPF reporting load.

# * * * * End of Changes* * * *