SA WG2 Meeting #170	S2-2507588

# Goteborg, Sweden, August 25 – 29, 2025		     revision of S2-2507504

Source: 	NTT DOCOMO

Title: 	KI#1: Sol#12 Update: UP-based UE data collection

Document for: 	Approval

Agenda Item: 	20.3.1 (FS_AIML_CN_Ph2)

Work Item / Release:	FS_AIML_CN_Ph2 / Rel-20

Abstract of the contribution: This contribution updates the solution #12 for KI#1 to transfer data from UE via UP where the data transfer is controlled via CP.

# 1	Discussion

RAN2 made the following agreements on UE-side data collection, as documented in RP-250310 and RP-251185:

- Extend the following agreements on data collection configuration in AI/ML based beam management to general UE-side data collection configuration: - Data collection related configuration(s) and associated ID(s)(if needed) can be included in training data collection configuration. - For data collection configuration UE-side model training, the UE can send a request for data collection (e.g. start/stop). FFS whether a suggested data collection configuration/associated IDs (if specified)/parameters can be provided to the network. - The network can provide or release the data collection configuration (at any point in time), with or without UE request. - The following methods for network control of the initiation and configuration for data collection: - The network can decide when to start/stop the data collection and send configuration. - The network can configure whether UE is allowed to initiate request for data collection (e.g. start/stop indication). - Discussion on UE-side data collection configuration and related procedures: - The UE can request measurement configuration for data collection of AI/ML based beam management. The request can contain one or more of the following: - An indication on start/stop of data collection - Preferred configuration from a list of candidate configurations provided by NW. Details of signaling are FFS. It is up to network what it configures at the end. - Introduce UAI message for UE request of data collection measurement configuration. And it is up to UE implementation when to send the request - Discussion on UE-side data collection configuration and related procedures: - The UE doesn’t need to measure the candidate data collection configuration(s). This will be specified in RAN2 specs. - For beam management, candidate data collection configuration includes at least: - CSI-ResourceConfigId of Set A - CSI-ResourceConfigId of Set B - One/two associated IDs (up to whether Set B is equal/subset of Set A or not) according to RAN1 agreements - FFS the details of how this is signalled (e.g. CSIReport config or simplified signaling)

In short, the agreements made by RAN2 assume the following procedure for data collection by UE:

- Step1 (Optional): gNB (network) provides a list of candidate data collection configurations for UE requests.

- Step2 (Optional): The UE sends a UAI message requesting to start/stop data collection with a preferred configuration selected from the candidate list in Step 1.

- Step3: The gNB starts or stops data collection by providing the data collection configuration to the UE for DL measurements. This configuration includes CSI-ResourceConfigId.

In summary, the data collection procedure agreed in RAN2 assumes that gNB (network) controls the start and stop of data collection by providing the data collection configuration to the UE. This configuration includes which DL reference signals (e.g., CSI-RS or SSB) are to be measured. However, the actual content to be measured and collected should reflect the requirements of the data-requesting entity. Therefore, gNB (network) should be able to receive data collection requirement information as part of the overall solution. Based on this observation, the coordination with gNB for downlink measurement has been included in this revision of the solution#12.

In addition, it remains unclear whether the data transfer control command from the DCF to the UE should be transmitted over C-plane or U-plane signalling. Our understanding is that data transfer control commands may be exchanged multiple times during the lifetime of a single PDU session. In such cases, it is reasonable to use U-plane signalling instead of C-plane signalling for the following reasons:

- Direct communication: U-plane signalling enables direct exchange between the UE and the DCF, reducing the involvement and signalling overhead of intermediate nodes such as the gNB and AMF.

- Implementation simplicity: Using U-plane does not impact the structure or content of lower-layer signalling messages (e.g., RRC or NAS messages), which improves implementation flexibility.

For these reasons, this revision of Solution #12 assumes that data transfer control commands are exchanged over the U-plane rather than the C-plane.

# 2 	Proposal

It is proposed to update TR 23.700-04 according to the following text.

********** First Change **********

# 3	Solutions

## 6.12	Solution #12: UP-based UE data collection

### 6.12.1	High level principles

The high-level principles of the proposed solution are as follow:

- To take the “futureproof and extendable solutions” requirement by RAN WGs, a new network function is proposed as the “first termination entity” to fulfil the MON’s controllability and visibility requirements.

- The UE data collection entity is deployed as a dedicated DNNI in the 5GC, and it is configured to receive traffic from the user plane. Using the existing mechanisms, the SMF configures the UPF(s) to forward the traffic from UE(s) to the DNNI via the User Plane to the UE data collection entity. The PCF is also configured to provide the appropriate PCC rules for usage monitoring of the traffic to the DNNI.

- The UE data collection entity manages (including initiating and terminating) the data transfer process based on operator’s policy and considering the consumer (e.g., the UE model training entity server) request (which is already authorized by the NEF). It also provides visibility for the MNO via i) keep trace of all the standardized data requests sent to the UE and UE response, ii) match data received from UE with respect to the standardized data format requested from the UE, iii) providing data content visibility based on operator’s request.

- A new NAS message is introduced to control the data transfer. It includes ithe information of the data collection entity as termination point (including DNNI, IP, port).

- Based on the information received by the NAS message, the UE may stablish/release QoS Flow (and PDU session) to the data collection entity.

- The data collection entity requests the gNB to initiate DL reference signal transmission for UE data collection. The gNB determines whether to initiate DL measurements and responds to the data collection entity accordingly.

Editor’s Note: How the data collection entity interacts with gNB for DL measurement in UE data collection needs coordination with RAN WG.

- The data collection entity exchanges messages with the UE over an established PDU session. These messages include a list of standardized data to be transferred and data transfer control information (e.g., start or stop indication).

### 6.12.2	Description

In the solution, data collection and transfer from UE is realized via

- Control plane signalling to manage the user plane connection via providing the information about data collection entity deployed in the 5GC (e.g., DNN, FQDN, Port #, …)

- User plane control messages to manage the initiation, modification, and termination of data transfer

- User plane data transfer from the UE to the data collection entity in the 5GC

The solution is composed of three main phases:

- The data collection entity (e.g., a new NF) is deployed in the MNO’s network with a FQDN and with a dedicated DNN which is reachable to UPFs. During the PDU session establishment process, the SMF configures the UPFs to forward the traffic targeted to the specified FQDN and/or DNN to the data collection entity. Moreover, the PCF provides the appropriate PCC rules for usage monitoring of the traffic to differentiate the traffic of UE data collection from the regular UE traffic.

- For the UEs that support data collection and also subject to UE consent (which may be extended to consider air interface data collection), the data collection entity sends the end point information (including IP address, port, etc.) of the data collection entity deployed in 5GC to the UE via existing “UE Policy Container” update mechanism or a dedicated NAS message.

- Based on the information and the instruction received from the network, UE establishes a dedicated PDU session, if it is not already established, and requests a QoS flow to the specified data collection entity. The data collection entity can then instruct the UE on what standardized data to collect and when to collect it over the established user plane connection. Then UE subsequently starts collecting and transferring the data via the UP path.

An example realization of the proposed solution is depicted in Figure 6.12.2-1.

Figure 6.12.2-1: An example of proposed solution

When there is a request for UE data, e.g., UE model training server needs training data, the consumer requests the data from the data collection entity in step 1. In the case of external data consumer, NEF authorizes the request and applies the operator’s policies on the data collection. The data collection entity, in step 2, triggers the user plane connection establishment via AMF.

Based on the configuration/instruction received, the UE may establish a PDU session and a QoS Flow to the specified FQDN/DNN in step 3 using the existing procedures wherein the SMF configures UPF (considering the PCC rules provided by the PCF) in step 5. Also, DCF should coordinate the downlink measurements with gNB for the data collection before data collection in step 4. Once the user plane connection establishment and the downlink measurement coordination are done, the data collection entity can trigger UE to collect data and transfer it to the data collection entity in step 6. The collected data is transferred to the data collection entity via the UP connection. Consequently, the data collection entity sends the data to the consumer (in step 8) and may also optionally store it in ADRF in step 9.

### 6.12.3	Procedures

The procedure of the proposed solution is shown in Figure 6.12.3-1.

Figure 6.12.3-1: CP-managed UP-based UE data Transfer Procedure

1.	The consumer (e.g., UE model training entity server) subscribes to the data collection entity for the UE data via NEF. The request is authorized, and the appropriate data collection entity is discovered/selected by the NEF.

2.	[optional] The data collection entity checks the UE consent for data collection. If the consent is granted and if needed (e.g., UE has not already established a data transfer PDU session or QoS flow), it prepares the appropriate UP connection configuration that includes the data collection entity end point information for U-plane connection establishment . Then it requests AMF to deliver it to the UE via invoking Namf_Communication_N1N2MessageTransfer.

3.	[optional] If the UE is not reachable, AMF performs the existing Network Triggered Service Request procedure.

Editor's note: Whether data collection from UEs in Idle state is required would need to be checked with RAN.

4.	[optional] The AMF though the RAN delivers the configuration/instruction message to the UE via a dedicated NAS message.

5.	[optional] The UE responds to the result of the request to the AMF.

6.	[optional] The AMF notifies the data collection entity about the UE’s response to the data collection request.

7.	[optional] The UE establishes a PDU session and a QoS flow to transfer the data according to the information received from AMF.

8.	During the PDU session establishment process, the SMF may contact PCF and the PCF provides the appropriate PCC rules to differentiate the data collection traffic.

9.	During the PDU session establishment process, The SMF configures the UPF to forward the UE data on this dedicated PDU session to the data collection entity.

10. The data collection entity generates a request for the downlink measurement initiation/update and requests the AMF to deliver it to the NG-RAN.

11. The AMF forwards the request to the NG-RAN.

12. The NG-RAN determines whether to accept or reject the request for the downlink measurement initiation/update and responds to the data collection entity accordingly. If the request is accepted, the NG-RAN initiates the downlink measurements as requested. If the request is rejected, the NG-RAN responds with the appropriate error to the data collection entity.

13. The NG-RAN provides measurement configuration to the UE via RRC signaling.

14.	AMF forwards the response of request for the downlink measurement initiation/update to the data collection entity.

15. Data collection entity sends the instruction on data transfer, where the instruction includes ID of standardized data to be transferred and the indication on the data transfer start.

16. UE collects the needed standardized data specified in the request from the network based on RAN instructions if it is not collected already.

17. UE sends the standardized data via the established PDU session/QoS Flow to the data collection entity.

18. The data collection entity sends the request data to the consumer.

19.	The data collection entity stores data in the ADRF.

### 6.12.4	Impacts on existing services, entities and interfaces

The solution has the following impacts:

The data collection entity (a new NF):

-	Support service operations to be used by the consumers to request UE data

-	Support providing appropriate configuration/instruction messages for the UE for data collection

- 	Support data reception via user plane

-  Support coordination with RAN on downlink measurements for data collection

- 	Support data content check whether the data received from UE matches the requested standardized data format, and, if necessary, processing the received data.

Editor's note: How to provide full data visibility is FFS.

- 	Support data transfer controllability (e.g., configuration of data transfer including allowed data to transfer, data collection period, …)

NG-RAN;

-  Support coordination with the data collection entity regarding downlink measurements for data collection

UE:

-	Understand the configuration/instructions received the network and perform the required action (e.g., establish a PDU session or QoS Flow, start/stop/pause data collection, …)

-	Support air interface data collection and transfer via UP

AMF:

-	Support delivery of new NAS messages to the UE to for data transfer configuration information

PCF:

-	Providing PCC rules for usage monitoring of UE data collection traffic

-	Providing configuration/instruction to the UE in the UE registration process (optional)

UDM:

-	Extension of user consent to control “air interface data collection”

Editor’s Note: Whether and how to extend user consent needs coordination with SA3.

********** End of Changes **********