3GPP TSG-SA2 Meeting #170	S2-2507690

Goteborg, Sweden, 25 - 29 August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.369 | CR | 0065 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to AIoT Area related |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AmbientIoT-ARC |  |  |  |  |  | Date: |  |  | 2025-08-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In S2-2506141 “LS from RAN WG3: Reply LS on Ambient IoT progress of RAN3 on OAM requirements”, the introduction of A-IoT Area ID should be reflected in the SA2 specification TS 23.369. - The AIOTF is aware of the supported “A-IoT Areas” of a gNB via OAM. - A new A-IoT Area is represented by an A-IoT Area ID - A-IoT Area ID = PLMN ID +NID (optional) + A-IoT Area Code (OCTET STRING (SIZE(3))). - OAM configures in the AIOTF the mapping relationships among gNBs, readers and A-IoT areas, as needed. NG-RAN selection information is not used for RAN selection only, but also can be used for reader selection, so the term is not suitable. |  |  |  |  |  |  |  |  |
| Summary of change: |  | NG-RAN selection information->NG-RAN configuration information. Add the mapping relationships among NG-RAN/RAN readers and A-IoT areas in the OAM configured RAN configuration information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TS 23.369 will not reflect the latest RAN3 progress. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

### 5.3.3	NG-RAN Node and RAN Reader Selection

The AIOTF selects NG-RAN node(s) and optionally RAN Readers based on the configured NG-RAN node configuration information and the AIoT service operation request from the AF. The AIOTF may also select a RAN Reader based on the stored last known RAN Reader information.

The AIOTF obtains the NG-RAN configuration information (NG-RAN ID, AIoT Area ID list, Optionally RAN Reader list, and, optionally, the location served by each RAN Reader, the mapping relationships between RAN Readers and A-IoT areas) via OAM or local configuration.

When the AIOTF receives an AIoT service operation request, based on the received Target Area information in the AIoT service operation request and the NG-RAN configuration information, the AIOTF selects the NG-RAN node(s) and optionally RAN Reader(s) to handle the request and the AIOTF determines the AIoT area(s) for the request. If the RAN Readers within NG-RAN configuration information, the AIOTF can directly select the RAN Reader(s).

The AIOTF sends a Requested Service Area Information for each selected NG-RAN node in the AIoT service operation request. The Requested Service Area Information provided to each NG-RAN node can be empty, or includes a list of AIoT Area IDs, a list of RAN Reader IDs, or both a list of AIoT Area IDs and a list of RAN Reader IDs.

- If the Requested Service Area Information is empty, the NG-RAN node selects all the served RAN Readers supported by the NG-RAN node to handle the requested AIoT service operation.

- If the Requested Service Area Information includes a list of AIoT area IDs, the NG-RAN node selects the RAN Readers within the indicated AIoT area to handle the requested AIoT service operation.

- If the Requested Service Area Information includes a list of RAN Reader IDs, the NG-RAN node selects the indicated RAN Readers to handle the requested AIoT service operation.

- If the Requested Service Area Information includes both a list of AIoT Area IDs and a list of RAN Reader IDs, the NG-RAN node selects the indicated RAN Readers to handle the requested AIoT service operation in the indicated AIoT area.

NOTE 1:	The Target Area information in a received AIoT service operation request can span multiple NG-RAN node(s) or can be a subset of the supported AIoT areas of a single NG-RAN node.

The AIOTF sends the AIoT service operation request to each selected NG-RAN node, either directly or through the selected AMF, including the Requested Service Area Information for the NG-RAN node.

If an AIoT service operation request received by the AIOTF includes individual target AIoT Device Perminant ID(s), the AIOTF may consider the last known serving RAN Reader, if available from the AIoT Device context to determine the NG-RAN node and RAN Reader(s) for the request. In this case the last known serving RAN Reader ID is sent as the Requested Service Area to the selected NG-RAN node.

NOTE 2:	The RAN Reader ID is not exposed to the AF.

NOTE 3:	An AIOTF receives the Inventory Report from an NG-RAN node includes a RAN Reader ID that represents the AIoT Device’s location at Reader granularity.

NOTE 4:	The AIOTF uses the RAN Reader ID and the NG-RAN node to update the last known serving RAN Reader information in the local AIoT Device context.

End of CHANGES