# 3GPP TSG-SA WG2 Meeting #170	S2-2507696

25 - 29 August 2025, Goteborg, Sweden

Title:	[Draft] LS on NF idle energy consumption

Response to:	-

Release:	Rel-20

Work Item:	FS_EnergySys_Ph2

Source:	NTT DOCOMO (to be SA2)

To:	SA5

Cc:	-

Contact person:	Bahador Bakhshi

bakhshi (at) docomolab-euro (dot) com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

# 1	Overall description

In Rel-19, SA2 introduced EIF (Energy Information Function), which provides estimation of energy consumption (EC) at granularities finer than the NF level (which is already provided by OAM) such as PDU session level EC. The functionalities of EIF are specified in clause 5.51 of TS 23.501. The EIF calculates the ECs at these levels based on the EC of gNB and UPF(s) obtained from OAM (and also the information on the data volume), according to the procedures specified in clause 4.29 of TS 23.502 and using the estimation formula documented in Annex T of TS 23.501.

SA2 understands that the “NF Energy Consumption” specified in clause 6.7.3.1 of TS 28.554 is the total amount of energy consumed by the NF including both idle EC consumed for powering up the NF and the EC of providing services by the NF.  However, based on the operator's policy, deployment condition, and/or SLA agreements, the operator may need to consider only the amount of energy consumed to serve the user traffic (not including the idle EC).

To fulfill the requirement, it was proposed to exclude the idle EC of UPFs from the calculation, which would require, OAM provides only the EC for serving user traffic. SA2 understands that it is within the responsibility of SA5 to decide whether to define a new KPI for this purpose or to customize the existing KPI (as defined in clause 6.7.3.1 of TS 28.554), in the case where the EIF acts as the consumer.

SA2 kindly asks SA5 to provide answers to the following questions:

Question: Is it feasible that OAM, with some potential enhancements, would be capable of providing the EC of UPF only for handling user traffic (i.e., excluding the idle EC of UPF)?

# 2	Actions

To SA5

ACTION 1: SA2 kindly asks SA5 to answer the question above.

# 3	Dates of next TSG SA WG-2 meetings

SA2#171		2025-10-13 – 2025-10-17			Wuhan, China

SA2#172		2025-11-17 – 2025-11-21			Dallas, US