3GPP SA WG2 Meeting #170	S2-2507761

Goteborg, Sweden, 25 – 29 August 2025	(revision of S2-2507708 of S2-2506730)

Source: 	LG Electronics, OPPO, Ericsson, China Mobile, Huawei, HiSilicon, Samsung, NTT DOCOMO, Ofinno, Lenovo, Xiaomi

Title: 	Architectural Assumptions

Document for: 	Approval

Agenda Item: 	20.5.1

Work Item / Release:	FS_AmbientIoT_Ph2_ARC / Rel-20

Abstract of the contribution: This paper proposes Architectural Assumptions in TR 23.700-30.

1	Discussion

TR 23.700-13v19.0.0 "Study on Architecture support of Ambient power-enabled Internet of Things " includes the following principle under clause 8.1 (Conclusion on Key Issue #1).

3. The AIoT Device does not distinguish whether the connectivity topology is Topology 1 or Topology 2, nor the transport used by the AIoT Reader. NOTE 12: The AIoT device is also agnostic to the potential different architectures if more than one architecture is concluded for both the topology 1 and topology 2.

Therefore, it is proposed to add Architectural Assumptions related to the above principle to TR 23.700-30.

**************************

The architectural assumptions proposed by the following tdocs were added to S2-2506730 for discussion and decision:

| S2-2506354 | 23.700-30: Architectural Assumptions and Requirements | OPPO |
| --- | --- | --- |
| S2-2506537 | 23.700-30: Architectural Assumptions and Requirements | Ericsson |
| S2-2507010 | 23.700-30: Architectural Assumptions | China Mobile |
| S2-2507248 | 23.700-30: Architectural Assumptions and Requirements | Huawei, HiSilicon |

2	Text Proposal

It is proposed to agree the following changes into TR 23.700-30.

* * * * Start of 1st Change * * * *

4.1	Architectural Assumptions

The following traffic types for AIoT Device are to be studied:

-	DT: Device-terminated;

-	DO-DTT: Device-originated - device-terminated triggered; and

-	DO-A: Device-originated - autonomous.

Common architectural assumptions:

-	Architecture reference models defined in TS 23.369 [3] are used as the baseline architecture for this study.

-	The AIoT Device is agnostic to topologies, i.e. Topology 1 and Topology 2.

Architectural assumptions for support of Topology 2 for Key Issue#1:

-	No Rel-19 AIoT Device impact is expected.

-	The following topology as defined in TR 38.848 [x] is assumed:

-	BS <--> intermediate node <--> Ambient IoT Device: Only a UE can act as an intermediate node which is under the network control

-	Only Indirect Connectivity (i.e. NG-RAN <-> AMF <-> AIOTF) is applied.

-	The support of Topology 2 in this study is based on the RRC-based option and the interim conclusions of Rel-19 in TR 23.700-13 [7] is the baseline.

Architectural assumptions for support of DO-A Capable AIoT Devices:

-	The AIoT Device is agnostic to Topology 1 architecture types, i.e. Direct Connectivity architecture and Indirect Connectivity architecture.

NOTE 1:	Coordination with RAN WGs is required to determine the AIoT Device capabilities in relation to system level of functionality (considering e.g. traffic scenarios, connectivity topologies etc.).

NOTE 2:	The security aspects for Ambient IoT require coordination with SA WG3.

NOTE 3:	The charging aspects for Ambient IoT will be studied by SA WG5.

* * * * End of Changes * * * *