SA WG2 Meeting #170	S2-2507765

# Goteborg, Sweden, 25 – 29 August 2025                       (revision of S2-2507712)

Source: 	CATT

Title: 	KI#2, New sol: AIoT services support for DO-A capable AIoT Devices

Document for: 	Approval

Agenda Item: 	20.5.1

Work Item / Release:	FS_AmbientIoT_Ph2_ARC / Rel-20

Abstract of the contribution: This paper proposes a new solution for KI#2.

1	Discussion

1) WT#2 of FS_AmbientIoT_Ph2_ARC SID (SP-250834)

WT#2: Study the support of DO-A Capable AIoT Devices, including:

-	Support of the AIoT Device informing the network of its presence autonomously (e.g., an AIoT Device initiated registration-like procedure).

-	Support for an autonomous AIoT Device originated procedure to send data to the AIOTF, and support for routing the received data by AIOTF.

-	Naiotf and Nnef interface enhancements to provide the data received from an AIoT Device to the AF.

NOTE 4:	topology 2 aspect of WT#2 has dependency on WT#1.

This paper proposes a new solution for KI#2.

2	Proposal

It is proposed to update the TR 23.700-30 as followings.

# * * * * Start of change (All Text New) * * * *

6.X	Solution #X: AIoT services support for DO-A capable AIoT Devices

6.X.0	High-level Solution Principles

This solution addresses Key Issue #2 "Support of DO-A Capable AIoT Devices".

The solution combines registration-like procedure and data transfer procedure, and is based on the following general principles to support AIoT services for DO-A capable AIoT Devices:

-	The DO-A capable AIoT Device initiates registration-like procedure autonomously when it has pending data to be sent to the network.

-	After registration-like procedure, the AIOTF uses the Command Procedure as defined in TS 23.369 [xx] to read the AIoT data from the AIoT Device and sends the AIoT data to the AF via the NEF.

6.X.1	Description

This solution is applicable to both the Direct Connectivity and the Indirect Connectivity architectures as defined in TS 23.369 [xx].

For a DO-A (Device-originated - autonomous) capable AIoT Device, the device originated traffic is triggered by the AIoT Device itself for, e.g. sending data to the network.

The principles of this solution are as followings:

-	When a DO-A capable AIoT Device has pending data to be sent to the network, the AIoT Device sends Registration Request to the AIOTF.

-	The AIOTF authorizes and authenticates the AIoT Device and sends Registration Accept to the AIoT Device.

-	The AIOTF uses the Command Procedure as defined in TS 23.369 to read the AIoT data from the AIoT Device.

-	The AIOTF sends the AIoT data received from the AIoT Device to the AF via the NEF.

6.X.2	Procedures

Figure 6.X.2-1 describes the DO-A Procedure.

Figure 6.X.2-1: DO-A Procedure

0.	AIoT Device obtains radio resource for D2R message.

Editor’s note:	How AIoT Device obtains radio resource for D2R message is FFS and is to be decided by RAN WGs.

1.	The AIoT Device sends the AS D2R message (Registration Request) to the NG-RAN. The AS D2R message includes the AIOTF selection information including PLMN ID and/or NID and/or Third Party ID which is used by the NG-RAN to select the AIOTF. The Registration Request message is an AIoT NAS message and includes the AIoT Device ID and the Pending data indication indicating there is pending AIoT data in the AIoT Device.

NOTE 2:	The AS layer message in this step and the following steps are to be defined by RAN WGs.

2.	The NG-RAN selects the AIOTF based on the AIOTF selection information received in step 1.

Editor’s note:	Further details on AIOTF selection is FFS.

3.	The NG-RAN sends the Uplink AIoT information message (Registration Request) to the AIOTF directly or via an AMF.

4.	The AITOF performs Authorization and authentication procedure for the AIoT Device.

NOTE 3:	Security aspects are to be defined by SA3.

5.	The AIOTF sends the Downlink AIoT information message (Registration Accept, AIOTF AIoT Device NGAP ID) to the NG-RAN directly or via the AMF.

6.	The NG-RAN sends the AS R2D message (Registration Accept) to the AIoT Device. The Registration Accept message is an AIoT NAS message.

7.	Based on the Pending data indication received, the AIOTF sends the Command Request message (NAS Command Request, AIOTF AIoT Device NGAP ID, Size of the Command Response message) to the NG-RAN directly or via the AMF. The NAS Command Request is Read Request. The AIOTF AIoT Device NGAP ID is used by the NG-RAN to determine the AIoT device context in NG-RAN.

Editor’s note:	Other triggers for the Command Request are FFS.

Editor’s note:	How AIOTF derives the parameters included in the NAS Command Request is FFS.

Editor’s note:	Whether Registration Accept and NAS Command Request are separate or can be combined is FFS.

8.	The NG-RAN sends the AS R2D message (NAS Command Request) to the AIoT Device.

9.	The AIoT Device sends the D2R message (NAS Command Response) to the NG-RAN. The NAS Command Response is Read Response and includes the AIoT data.

10.	The NG-RAN sends the Command Response message (NAS Command Response, AIOTF AIoT Device NGAP ID) to the AIOTF directly or via the AMF. The AIOTF determines the AIoT device context by the AIOTF AIoT Device NGAP ID received.

11.	The AIOTF reports the AIoT data to the AF via the NEF.

Editor’s note:	Details on routing of AIoT data from the AIOTF to the AF via the NEF is FFS.

6.X.3	Impacts on Services, Entities and Interfaces

Editor's note:	This clause captures impacts on existing services, entities and interfaces.

# * * * * End of Changes * * * *