SA WG2 Meeting #170	S2-2507766

25 – 29 Aug, 2025, Goteborg , SE	(Revision of S2-2507713, 6686)

Source:	Samsung, Futurewei

Title:	Solution for KI#2 to Support Sensor Data Collection

Document for:	Agreement

Agenda Item:	20.5.1

Work Item / Release:	FS_AmbientIoT_Ph2_ARC/Rel-20

Abstract of the contribution: This contribution proposes a solution to support sensor data collection regarding Key Issues for WT#2.

1. Discussion

The existing description of WT#2 as in the agreed SID titled as “Study on Architecture support of Ambient power-enabled Internet of Things - Phase 2” [SP-250834] is described as follows:

WT#2: Study the support of DO-A Capable AIoT Devices, including:

-	Support of the AIoT Device informing the network of its presence autonomously (e.g., an AIoT Device initiated registration-like procedure).

-	Support for an autonomous AIoT Device originated procedure to send data to the AIOTF, and support for routing the received data by AIOTF.

-	Naiotf and Nnef interface enhancements to provide the data received from an AIoT Device to the AF.

NOTE 4:	topology 2 aspect of WT#2 has dependency on WT#1.

Ambient IoT Service use cases in TS 22.369 include Sensor Data Collection. With sensor data collection, the Ambient IoT device is associated with a sensor. Transfer of sensor data can be initiated by the Ambient IoT device, for example,  periodically.

It is assumed KI#2 is key issue for WT#2.

2. Proposal

It is proposed to agree the following changes to 3GPP TS 23.700-30.

* * * Start of Change (all new text) * * * *

6.X	Solution #2.X: Support of Sensor Data Collection

6.X.0	High-level solution Principles

This solution addresses KI #2. It includes a Service Request from AF to 5GS, Parameter Configuration for the service, and Data Transfer for the service

6.X.1	Description

The solution is based on the following assumptions:

-	Sensor Data Collection Service utilizes Periodic DO-A Sensor Data Collection Service from Device including AF, AIOTF, and AIoT Device. It uses Read/Write command type with the parameter Period or separate comand types.

-	Sensor Data Collection Service utilizes Periodic Periodic DO-A Sensor Data Transfer (Device, AIoT Reader, AIOTF)

-	Sensor Data Collection Service Paramters can be pre-configured on AioT Devices or configured by AF. For example, Sensing Period and StartTime.

-	The AIoT Reader maintains (Device ID and Correlation ID) context for every periodic behavior

6..X.2	Procedures

The procedure is shown in Figure 6.X.2-1. It includes Service Reqeust from AF to 5GS, Parameter Configuration for the Service, and Data Transfer for the Service.

Figure 6.X.2-1: Sensor Data Collection Procedure

1.	Step 1 to Step 6 of Procedure for Command in clause 6.2.3 of TS 23.369 with additional paramenters. AF sends AIoT Command Request to NEF. In includes Device(s) Info, Location (External), AF Transaction ID, Command Type Read, Sensing Result Offset, Length, Period, and Sensing Parameters ([Sensing Period Offset, Period], [StartTime Offset, Now/time/Off]). Sensing Parameters are optional.

NEF selects AIOTF, and then NEF sends AIoT Command Request to AIOTF.

AIOTF chesks AF Authroization by retriving AF Authorization Profile in ADM.

AIOTF sends AIoT Command Response to NEF. It includes Status (Success/Failure), Transaction ID.

NEF sends AIoT Command Response to AF. It includes Status (Success/Failure), AF Transaction ID.

2.	Step 7 to Step 10 of Procedure for Inventory in clause 6.2.2 of TS 23.369

3.	AIOTF retrieves Device Profile Data Including DO-A Capability. If the device does not support DO-A Capability, steps 15 to 19 are skipped.

4a~8a, 4b~8b.	Parameter configuration with Write Command can be done instead of parameter configuration included in steps 4~8 .

4.	AIOTF sends AIoT Command Request to AIoT Reader. This includes Device ID (temproary), (Reader ID), Command Type Read, Correlation ID, Sensing Result Offset, Length, Period, and Sensing Parameters ([Sensing Period Offset, Period], [StartTime Offset, Now/time/Off]). AIoT Reader stores Correlation ID with AIoT Device ID as context data.

5.	AIoT Reader sends AIoT Command Request to AIoT Device. This includes Device ID (temproary), (Reader ID), Command Type Read, Correlation ID, Sensing Result Offset, Length, Period, and Sensing Parameters ([Sensing Period Offset, Period], [StartTime Offset, Now/time/Off]). When 4a~8a, 4b~8b are done, Sensing Parmeters are omitted.

6.	When AIoT Device receives Period with Command Type Read, it starts periodic Read behavior based on the Period value. When AIoT Device receives Sensing Parameters, it sets the parameters to the device. 
For example, it sets Sensing Period to a specific Period, and/or sets StartTime to Now, a specific time, or Off. If StartTime is set to Now, the device reads and report Sensor Result immediately and every Period. 
If StartTime is set to a specific time, then the device reads and reports Sensor Result at that time and every Period after that. 
If StartTime is set to Off, the device reads and reports Sensor Result once, then stops periodic reading and transfer.

7.	AIoT Device sends AIoT Command Response to AIoT Reader. This includes Device ID (temproary), and Sensing Result.

8.	AIoT Reader sends AIoT Command Response to AIOTF. This includes Device ID (temproary), Sensing Result, and Correlation ID. AIoT Reader retrieves Correlation ID with AIoT Device ID from the stored context.

9.	AIOTF sends AIoT Command Notify to NEF. This includes Device ID (permanent), Sensing Result, and Transaction ID. AIOTF retrieves Transaction ID with Correlation ID from the stored context.

10.	NEF sends AIoT Command Notify to AF. This includes Device ID (permanent), Sensing Result, and AF Transaction ID. NEF retrieves AF Transaction ID with Transaction ID from the stored context.

11.	Every Period, when the energy is enough, the device reads and reports Sensor Result. Depending on the size of the report, the radio resource allocation procedure may vary.

Editor’s note: How the AIoT Device supports timer is FFS.

12.	AIoT Device sends AIoT Command Report to AIoT Reader. This includes Device ID (temproary), Sensing Result.

13.	AIoT Reader sends AIoT Command Report to AIOTF. This includes Device ID (temproary), Sensing Result, and Correlation ID. AIoT Reader retrieves Correlation ID with AIoT Device ID from the stored context. If the context is not stored, AIoT Reader sends AIoT Comamand Report with Device ID (temproary), Sensing Result, and temp-Correlation ID, which AIoT Reader generates as indication of empty Correlation ID.

14.	AIOTF sends AIoT Command Notify to NEF. This includes Device ID (permanent), Sensing Result, and Transaction ID. AIOTF retrieves Transaction ID with Correlation ID from the stored context. If step 13 includes temp-Correlation ID, AIOTF retrieves Transaction ID with Device ID (temproary).

15.	NEF sends AIoT Command Notify to AF. This includes Device ID (permanent), Sensing Result, AF Transaction ID. NEF retrieves AF Transaction ID with Transaction ID from the stored context.

6..X.3	Impacts on services, entities and interfaces

Impacts on existing entities:

AIOTF:

-	Receives AIoT Command Request for Sensor Data Collection Service from AF through NEF.

-	Performs Inventory procedure and then sends Read Command Request with Period and Sensing Parameters.

-	Performs Inventory procedure and then sends Write Command Request with Sensing Parameters.

-	Retrieves Device Profile Data including DO-A Capability. If the device does not support DO-A Capability, AIOTF skips DO-A related steps.

-	Receives AIoT Command Report from Device(s) through AIoT Reader(s) and deliveres it to the AF through the NEF.

-	Sends AIoT Command Notify to AF via NEF.

ADM:

-	Supports the AIoT Device profile, including DO-A Capability.

AIoT Reader:

-	Supports the AIoT TempID handling, including Device ID and Corrleation ID mapping, and maintains the mapping context.

-	Supports DO-A procedure for AIoT Command Report from AIoT Device(s).

AIoT Device:

-	Supports DO-A procedure for AIoT Command Report from AIoT Device.

-	Reads and reports Sensing Data to AIoT Reader periodically via the DO-A procedure.

-	Supports timer.

-	Considers energy status for its behavor.

-	Sets Sensing Parameters and behave according to the Sensing Parameters.

-	Performs radio resource allocation procedure depending on the size of the report.

* * * End of Change * * * *