3GPP TSG-SA WG2 Meeting #170	S2- 2507782

25 - 29 August, 2025, Goteborg, Sweden	(revision of S2-2507722)

Title:	23.700-14: New Solution for Key Issue#5: Sensing Result Exposure

Source:	ETRI

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract:	This contribution proposes a solution to support Key Issue #5: Sensing Result Exposure.

Proposal

The following solution is proposed for inclusion in TR 23.700-14 to address Key Issue #5, which concerns the exposure of sensing results to support sensing services. This solution specifies mechanisms by which the Core Network can expose sensing results, with or without contextual information as defined in TS 22.137 [2], to authorized service consumers, including both trusted and untrusted Application Functions (AFs).

Key aspects to be studied for this solution include:

-	Types of sensing results: The nature and format of sensing results, optionally enriched with contextual information (e.g., timestamp, location, velocity, object type, confidence score).

-	Exposure mechanisms: Interfaces and procedures for delivering sensing results directly to trusted AFs or indirectly via the NEF for untrusted AFs, with applicable access control.

-	Reporting modes: Support for one-time delivery, periodic reporting, and event-triggered notifications to meet different service requirements.

* * * First Change * * * *

6	Solutions

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Solutions | #1 | #2 | #3 | #4 | #5 | #6 |
| #1 | X |  | X | X | X |  |
| #2 |  | X |  |  | X |  |
| #3 | X | X | X | X | X | X |
| #4 | X |  |  |  |  |  |
| #5 | X |  |  | X |  |  |
| #6 | X | X | X |  |  |  |
| #7 | X |  |  |  |  |  |
| #8 |  | X |  |  |  |  |
| #9 |  | X |  |  |  |  |
| #10 |  | X |  |  |  |  |
| #11 |  | X |  |  |  |  |
| #12 |  | X |  |  |  |  |
| #13 |  |  | X |  |  |  |
| #14 |  |  | X |  |  |  |
| #15 |  |  | X |  |  | X |
| #16 |  |  | X |  |  |  |
| #17 |  |  | X |  |  |  |
| #18 |  |  |  | X |  |  |
| #19 | X | X |  |  | X |  |
| #20 |  |  |  |  |  | X |
| #X |  |  |  |  | X |  |

6.X	Solution #X: Sensing Result Exposure for (un)trusted Application Functions

6.X.0	High-level Solution Principles

Editor's note:	Where possible similar/overlapping solution proposals should be documented as a single solution in the TR.

Documentation of more than one solution from the same company for the same KI is allowed but it shouldn't be expected that all of them will be documented in the TR.

This solution defines a mechanism for exposing sensing results, along with associated service parameters and procedures, to both trusted and untrusted Application Functions (AFs). The focus is on specifying a clear service interface and parameter set that supports one-time requests, continuous monitoring, updates, and notifications for sensing services. Flexible and secure delivery modes are supported, including one-time, periodic, and event-triggered reporting, with interfaces reflecting the consumer’s trust relationship with the network:

- Trusted AFs obtain sensing results directly from the Sensing Function via interfaces such as Nsf_SensingService, supporting (un)subscribe, and notify operations.

- Untrusted AFs access sensing services via the NEF using Nnef_EventExposure, with authorization.

Editor's note: Whether untrusted AF scenario is supported depends on the conclusions of KI#5.

The Sensing Function aggregates data from distributed sensing entities, processes the information, and delivers results enriched with contextual information such as map information, area information, time of capture defined in TS 22.137 [2]. Optional sensing assistance information (e.g., area of interest, motion thresholds, target object profile) may be provided to improve sensing accuracy and relevance.

6.X.1	Description

Editor's note:	This clause will describe the solution principles and architecture assumptions for corresponding key issue(s). Clause(s) may be added to capture details.

This solution addresses Key Issue #5: Sensing Result Exposure by specifying how sensing results, optionally enriched with contextual information, are exposed to authorized sensing service consumers. These consumers may include trusted or untrusted AFs requesting sensing data for use cases such as object detection, environmental monitoring, or motion monitoring as specified in clause 4.2.

A Sensing Service Consumer can request sensing results:

- Via NEF, for untrusted AFs, ensuring authorization.

- Directly to the sensing function, for trusted AFs, using a dedicated interface.

The Sensing Function is responsible for:

- Collecting and aggregating sensing data from distributed sensing entities.

- Processing sensing data to generate sensing results.

- Exposing sensing results to authorized consumers, potentially enriched with contextual information (e.g., timestamp, service area, velocity, confidence level).

NOTE:	The Sensing Function may correspond to the network function responsible for sensing result exposure, as defined in the outcome of the study on Key Issue #1: System Architecture to Support Sensing.

The solution supports the following reporting modes:

- One-time delivery (on-demand)

- Periodic reporting (e.g., every 10s, 1 min)

- Event-triggered reporting (e.g., object movement, area entry/exit)

The sensing function exposes these results using:

- Direct interface (Nsf_SensingService) for trusted AFs.

- Indirect interface (Nnef_EventExposure) via NEF for untrusted AFs.

Optionally, sensing assistance information, as described in TS 22.137 [2], may be provided by authorized third parties (e.g., area of interest, target object parameters, velocity threshold) to improve sensing performance.

6.X.2	Procedures

Editor's note:	This clause describes high-level procedures and information flows for the solution.

This clause details how trusted or untrusted AFs interact with the sensing function to subscribe to, and receive sensing results. The process encompasses sensing service request initiation, authorization, sensing operation execution, and result notification.

The Sensing Function exposes sensing services as described in Table 6.X.2-1. The Nsf_SensingService interface supports subscription-based delivery of sensing results, accessible directly by trusted AFs or indirectly via the NEF for untrusted AFs.

Table 6.X.2-1: Sensing Function Exposure Services

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nst_SensingService | Subscribe | Subscribe/Notify | NEF, AF |
|  | Unsubscribe | Subscribe/Notify | NEF, AF |
|  | Notify | Subscribe/Notify | NEF, AF |

6.X.2.1	Sensing Service Request from AFs

A trusted AF subscribes to sensing services using the Nsf_SensingService_Subscribe directly to the sensing function. In contrast, an untrusted AF initiates subscriptions via the NEF using Nnef_EventExposure_Subscribe, which forwards the request to the sensing function. Each request includes:

- Sensing type: object detection, tracking, environment monitoring, etc

- Target: specific object(s) or area

- Sensing Quality: accuracy, resolution, latency

- Reporting mode: one-time, periodic, or event-triggered

- Optional sensing assistance information: object related information (e.g., expected type), area information (e.g., coordinates, boundaries), motion information (e.g., velocity, direction)

Editor’s note: Details of Target (e.g., a drone) are FFS.

Editor’s note: Details of Sensing Quality are FFS.

6.X.2.2	Sensing Result Exposure to AFs

Upon receiving a valid subscription, the sensing function configures sensing operations among the relevant sensing entities. When results become available, the sensing function transmits them either directly to the trusted AF using the Nsf_ SensingService_Notify or indirectly through the NEF using Nnef_EventExposure_Notify for untrusted AFs. Each notification includes:

- Sensing result payload: object-specific information such as type, location, speed, direction, and geometric representation

- Contextual information: timestamp, accuracy estimation, confidence level (as defined in TS 22.137 [2]), and serving area

Editor’s note: Details of confidence level are FFS.

Figure 6.X.2-1: Sensing Service exposure procedure for sensing result

The sensing result exposure process generally follows these sequential steps:

- Sensing Service Subscribe Request: The AF sends a subscription request to the Sensing Function, indicating the sensing service type, area of interest, reporting mode, and optional assistance information.

- Sensing Service Authorization: The Sensing Function performs authorization based on operator policies and service profiles to ensure the AF is permitted to access the requested sensing service (as per KI#2)

NOTE: The authorization aspects are within the scope of KI#2 conclusions.

- Sensing Service Subscribe Response: Upon successful authorization, the Sensing Function acknowledges the subscription request by sending a subscribe response to the AF.

- Sensing Configuration / Trigger: The Sensing configures and triggers the selected sensing entities according to the subscription parameters. (as per KI#3 and KI#6)

- Sensing Notification / Data Collection: The sensing entities perform the configured sensing operations and provide measurement data to the Sensing Function. (as per KI#4)

- Sensing Service Notify: The Sensing Function processes the collected data, generates sensing results (optionally enriched with contextual information, and notifies the AF in accordance with the subscribed reporting.

Editor’s note: Interaction with NEF for untrusted AFs is FFS

6.X.3	Impacts on Services, Entities and Interfaces

Editor's note:	This clause captures impacts on existing services, entities and interfaces.

Sensing Function:

-	Introduced as a new functional entity responsible for processing sensing data and exposing sensing results to authorized consumers.

Sensing Service Consumer (e.g., AF):

-	External application that consumes sensing results to enable vertical use cases such as motion tracking, object detection, or environmental sensing.

NEF:

-	Optionally mediates sensing result exposure for untrusted AFs, providing authorization.

* * * End of Changes * * * *