SA WG2 Meeting #170	S2-2507817

25 – 29 Aug 2025, Goteborg, SE	(revision of S2-2507781, S2-2507705, S2-2506816)

Source:	China Mobile

Title:	New solution for KI#5: Support sensing result exposure to AF

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract: This paper proposes a new solution for KI#5 of FS_Sensing_ARC.

# 1. Introduction

KI#5 of FS_Sensing_ARC has been agreed and documented in TR 23.700-14. This paper proposes a new solution to address the following aspects to support sensing result exposure to AF.

-	Study what types of sensing results with or without the sensing contextual information defined in TS 22.137 [2] could be exposed from the Core Network to the sensing service consumer.

-	How the Core Network exposes sensing results to the sensing service consumer (e.g., one time, periodic or event trigger).

# 2. Text Proposal

It is proposed to capture the following changes in TR 23.700-14.

# * * * * First change * * * *

# 6	Solutions

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 | 6 |
| X |  |  |  |  | X |  |

# * * * * Second change (All new text) * * * *

## 6.X	Solution #X: Support sensing result exposure to AF

### 6.X.1	Description

This solution aims to provide new solution to support sensing result exposure to AF.

To enable flexible scaling of Sensing Functions, it’s assumed that the Sensing Function splits into Sensing Control Function (SCF) and the Sensing Processing Function (SPF). SCF is responsible for sensing service control, while SPF is responsible for sensing data collection and processing, etc.

The main types of sensing results are the following:

-	Real-time sensing results: These information may include real-time latitude and longitude, speed, flight altitude and other sensing information of the sensing target. The scenarios may include real-time trajectory reporting for UAV and Internet of Vehicles. This type of sensing results have the characteristics of high exposure frequency and large data volume.

-	Subscribe/Notification sensing results: AF subscribes sensing services to the SCF. When SCF determines that the sensing result reported by SPF meets the subscribed event, it reports the sensing result to AF. The scenarios include route change notification and illegal intrusion alarm, etc. This type of sensing results have the characteristics of low frequency and small data volume.

SCF should determine the exposure method by the request of AF or the characteristics of requested sensing service. For the real-time sensing results, considering high exposure frequency and large data volume, exposing sensing result via signal connection may introduce huge load pressure and instability on the network. Hence, it’s suggested to expose this type of sensing results by the data connection between SPF and AF. For the event triggering sensing results, taking low frequency and small data volume into account, both the methods of the signal and data connection are available.

### 6.X.2	Procedure

Figure 6.X.2: Procedure of Support sensing result exposure to AF.

1. AF requests to NEF to obtain the Sensing result from the network via Nnef_Sensingservice Request. The service request need include sensing parameters and AF address.The sensing parameters may include sensing type, sensing area, sensing reporting frequency, and so on.

2. NEF discovers and selects the candidate SCF via NRF or local configuration, which is addressed by solutions to Key Issue #3.

3. NEF sends the service request from AF to SCF via Nscf_Sensingservice Request, which may include sensing parameters and AF address.

4. SCF performs service authorization which is addressed by solutions to Key Issue #2.

5&6. If service authorization is successful, SCF indicates AF that the request has been accept through NEF. If service authorization is fail, SCF indicates AF that the request has been rejected through NEF and the following step is skipped.

7. SCF performs the discovery and selection for the Sensing Entity/Entities which is addressed by solutions to Key Issue #3.

8. SCF performs the discovery and selection for the SPF which may be addressed by solutions to Key Issue #3.

9. SCF determines the exposure method based on local pre-configuration, considering the AF ID and/or other information (e.g., sensing service type, sensing service QoS) included in the sensing service request from the AF.

If SCF selects the data connection based exposure method, then the SCF may send AF address to SPF to expose the sensing result via a data connection in step 11.

If SCF selects the signal connection based exposure method, then the SCF may instruct the SPF to send the sensing result to the SCF in step 11.

Editor’s note: The details of how the SCF determines the exposure method based on local pre-configuration are FFS.

10. SCF sends sensing request to Sensing Entity/Entities, including the configuration parameters which is addressed by solutions to Key Issue #6.

11. SCF configures SPF with sensing processing rule and AF address. Sensing processing rule may indicate SPF how to process and transmit the sensing data, which is addressed by solutions to Key Issue #4.

12. Sensing Entity/Entities obtain and transmit sensing data to SPF, and SPF performs the sensing data processing.

13. SPF exposes the sensing result to AF with the AF address received in step 10 if data connection based exposure method is used.

NOTE 1: The AF in step 13 is within the operator’s trusted domain.

14. SPF transmit the sensing result to SCF, and SCF further exposes the sensing result to AF via NEF if signal connection based exposure method is used.

Editor’s note: The details of how the data connection between SPF and AF is established are FFS.

NOTE 2: How to guarantee the security of sensing result is determined by SA3.

### 6.X.3	Impacts on services, entities and interfaces

NEF:

-	Interaction with AF and SCF to provide sensing parameters and AF address to SCF.

SCF:

-	Perform the discovery and selection for the SPF and providing sensing processing rule and AF address to SPF.

-	Determine the exposure method taking the request of AFor the characteristics of requested sensing service into account.

SPF:

-	Determine the sensing result and expose the sensing result to AF with data or signal connection.

AF:

-	Provide sensing parameters and AF address to SCF.

-	Acquisition Sensing result(s).

# * * * * End of changes * * * *