SA WG2 Meeting #170	S2-2507874

25 – 29 Aug, 2025, Goteborg, SE

Source:	NTT DOCOMO (pen holder)

Title:	[General aspects] Summary of Architectural Assumptions

Document for:	Approval

Agenda Item:	20.6.0

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution summarizes all contributions on Architectural Assumptions

# 1. Contributions considered

| TDoc | Title | Sources |
| --- | --- | --- |
| S2-2506391 | 23.801-01: [General aspects] Architecture Assumptions for 6G | OPPO |
| S2-2506635 | 23.801-01: [General aspects] Architectural Assumptions and Requirements | vivo |
| S2-2506774 | 23.801-01: [General aspect] Architecture assumption and requirements | ZTE |
| S2-2506842 | 23.801-01: [Arch. Assumptions] Architecture Assumptions | Samsung |
| S2-2506929 | 23.801-01: [General aspects] Architecture assumptions and requiremnts for 6G architecture | Ericsson, T-Mobile USA |
| S2-2507054 | 23.801-01: [General aspects] Architecture Assumptions and Requirements | Nokia, AT&T, Ericsson, T-Mobile US |
| S2-2507100 | 23.801-01: [General aspects] 6G System Architectural Requirements and Assumptions | CATT |
| S2-2507207 | 23.801-01: [General aspects] Architecture Assumptions and Requirements on Energy Efficiency | NTT DOCOMO, KPN N.V., AT&T, Deutsche Telekom |
| S2-2507416 | 23.801-01: Voice for 6G - Key Issues and Architectural Assumptions | T-Mobile USA Inc. |
| S2-2506994 | 23.801-01: [WT#2, Interworking and Migration] Inter-System interworking and migration consideration | Huawei, HiSilicon |

**** First Change ****

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Definition format (Normal)

<defined term>: <definition>.

6GC: In the context of this study, the 6GC is the Core Network of the 6G System.

**** Next Change (all text new) ****

## 4.1	Architectural Assumptions

Editor's Note: This clause documents the common architecture assumptions identified for the study.

The following is assumed for 6GS architecture:

1. 	6G System architecture should minimize the adoption of multiple options for the same functionality. (Samsung, SA2 6G SID)

2.	The framework of SBA specified in clauses 7.1 and 7.2.1 in TS 23.501 [ref], is assumed as a starting point for discussion for Core network for 6G System. Nokia et al, Ericsson et al)

3.	The Core Network for 6G System may use some 5GC NFs as basis with any necessary enhancement. Which 5GC NFs can be used or need to be redesigned will be identified during the study. (ZTE, Samsung, CATT)

4. 	6G RAN will only be connected to Core Network for 6G System. (Nokia et al, OPPO, Samsung, Huawei)

5.	The 6G system is assumed to maintain the existing RAN and CN functionality split as supported in 5GS, and avoid duplication of functionality in 6G RAN and 6GC. (ZTE, Vivo, SA2 6G SID)

6.	rSupport of  multi-media services such as voice, video (essential and regulatory services) will be using IMS. (Nokia et al, TMO-US)

NOTE: Enhancements to IMS architecture as part of IMS architecture evolution study expected in Rel-20 are not precluded. (Nokia et al)

7.	The study assumes the 6G System is designed to be energy-efficient by default for both network and UE. Energy efficiency considerations will be applied at the earliest stages of architecture design and system procedures, including the discovery and selection of functionalities and services, energy-efficiency-aware policies, and network deployment options, if any. (NTT DOCOMO et al)

8.	Interworking with 2G/3G are not considered in this study. Interworking between 6GS and 5GS is assumed to work even if the UE has previously registered in 2G, 3G or 4G. (Huawei, SA2 6G SID)

[Proposals that may require more discussion]

9a.	6G System architecture is composed of 6G UE, 6G RAN and 6G CN.  (Samsung)

9b.	The 6G system is assumed to include 6G UE, 6G RAN and 6GC. (ZTE)

10a. 6G CN shall support only 6G RAN. (Samsung)

10b. Only standalone architecture of 6GS will be studied. (Huawei)

11.	For voice services, when transitioning between different RATs the IP address of the UE for the IMS session will not change. (TMO-US)

12.	Location for emergency voice service will be supplied by the 6G location service system. (TMO-US)

13.	6G New Core shall support new 6G NFs, redesigned 6G NFs (CATT)

14.	This study assumes that Network Functions continue as the only granularity for defining open interfaces within Core network for 6G System. (Nokia et al)

NOTE x: This includes the possibility to re-use, to enhance existing 5G Network Functions or to define new 6G specific Network Functions. (Nokia et al)

15.	This study assumes NF Set is reused. (Nokia et al)

16.	AF should be able to obtain existing services from north bound APIs (exposure) also from 6GS. (Ericsson et al)

17.	(Relevant) Data structures related to the UE are inherited from 5GS. Enahancements and some modifications are not precluded. (Ericsson et al)

18.	6G RAN shall connect only one termination point of 6G CN for a given UE. (e.g. point-to-point interface between 6G RAN and 6G CN is assumed.) (Samsung)

19.	6G beyond connectivity services in 6G shall not impact neither control plane for connectivity services (e.g., MM and SM) nor User Plane for connectivity service (e.g., exchanges PDU between the UE and the Data Network). (Samsung)

20.	NF in 6G CN shall be newly designed, re-designed or extended according to the following guidelines: (Samsung)

-	New NFs that are introduced for new 6G capabilities are newly designed. (e.g., new NF for AI and Sensing)

-	NFs terminating 6G NAS are re-designed. (e.g., 6G AMF and 6G SMF)

-	NFs that are required to support similar functionalities in 5G but mandatory functionalities are changed in 6G can be considered as re-designed.

-	NF that is required to support similar functionalities in 5G, and can support 6G without changing mandatory behaviour can be considered to be extended. (e.g., UDM and NEF)

21. A unique kind of 6GS architecture. (Vivo)

Editor's Note:  standalone architecture or non-standalone architecture will be determined in SA2#171. (Vivo)

22.  If network slice is supported, it is not mandatory for a UE and the feature should be simplified to minimize the impact to the UE. (Vivo)

23.  if the UE policy is supported, it should be simplified to minimize the impact to the UE. (Vivo)

24.  For supporting interworking, support the scenario that the UE transfer to 4G immediately after transfer from 6G to 5G. (Vivo)

25. A uniform QoS/Policy framework is assumed to compatible for both communication and beyond communication services. (Vivo)

26.  Support the following new services and each of them is allowed on demand deployment. (Vivo)

-		Support integration of Sensing and Communication over 3GPP access.

-		Support of computing for UE, core network and application server in 6G.

27. 6G UE may or may not have AI capabilities. (Vivo)

28.	The Policy Framework defined in TS 23.503 is used as baseline. (OPPO)

29.	When the UE is in idle state, the UE location is known at the TA list granularity as defined in EPS and 5GS. (OPPO)

**** End of Changes ****