SA WG2 Meeting #170	S2-2507981

25 – 29 Aug, 2025, Goteborg , SE	(was S2-2507897)

Source:	Xiaomi (pen holder), OPPO(?), MediaTek Inc(?), Ofinno(?), LG Electronics(?), vivo(?), Samsung(?), ZTE(?), NTT DOCOMO(?), Apple(?), China Telecom, Huawei(?), HiSilicon(?), CATT(?), Nokia(?), T-Mobile US(?), Qualcomm(?), AT&T(?), InterDigital Inc(?)

Title:	[WT#2, Migration and Interworking WT scope] Work Task Scope of Migration and Interworking

Document for:	Approval

Agenda Item:	20.6.2

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution summarizes the work task scope from all contributions for WT#2.

1. Contribution Overview

This contribution summarizes the work task scope from all contributions for WT#2 and attempt to merge all content into this contribution.

  - Contributions considered

The following is proposed as baseline as starting point for drafting the WT scope of WT#2 Migration and Interworking based on the Tdoc list provided in the TDoc_ordering folder.

| TDoc | Type | Title | Source |
| --- | --- | --- | --- |
| S2-2506421 | P-CR | 23.801-01: [WT#2] Migration and Interworking | OPPO |
| S2-2506444 | P-CR | 23.801-01: [WT#2, 3GPP access Interworking] Study of 3GPP access interworking | MediaTek Inc. |
| S2-2506525 | P-CR | 23.801-01: [WT#2] Interworking and migration in 6G | Ofinno |
| S2-2506584 | P-CR | 23.801-01: [WT#2, Migration and Interworking] Scope and New Key Issue for migration and interworking | LG Electronics |
| S2-2506642 | P-CR | 23.801-01: [WT#2, Migration and interworking] Migration and interworking | vivo |
| S2-2506762 | P-CR | 23.801-01: [WT#2] Discussion and proposal for Interworking and Migration | ZTE |
| S2-2506809 | P-CR | 23.801-01: [WT#2, Migration and Interworking] Study items for migration and interworking | NTT DOCOMO |
| S2-2506875 | P-CR | 23.801-01: [WT#2] Migration and Interworking | Apple |
| S2-2506928 | P-CR | 23.801-01: [WT#2, Migration and Interworking] Support for migration and interworking | China Telecom |
| S2-2506994 | P-CR | 23.801-01: [WT#2] Inter-System interworking and migration consideration | Huawei, HiSilicon |
| S2-2507194 | P-CR | 23.801-01: WT#2, Migration and Interworking and study proposal | CATT |
| S2-2507230 | P-CR | 23.801-01: [WT#2, Interworking] Kis for Migration and Interworking-6G | Xiaomi |
| S2-2507375 | P-CR | 23.801-01: [WT#2] Justification: of work task scope for interworking | Nokia, T-Mobile US |
| S2-2507399 | P-CR | 23.801-01: [WT#2] Update to WT#2 and new Key issue to clarify migration options for multi-RAT connectivity. | Qualcomm Korea |
| S2-2507447 | P-CR | 23.801-01: [WT#2] 6G Migration Paths | Samsung, AT&T |
| S2-2506321 | P-CR | 23.801-01: [WT#2, Interworking with 5GS] Principles for Interworking with 5GS | InterDigital Inc. |

  - Summary of aspects of WT scope

S2-2506584(LG Electronics), S2-2506642(vivo), S2-2506762(ZTE), S2-2506994(Huawei, HiSilicon), S2-2507194(CATT) use WT#2 Scope as agreed in the 6G study item.

The rest of the TDocs propose to the WT#2 Scope based on what is agreed in FS_6G_ARC with the following additions:

| TDoc | Sources | Scope summary for migration and interworking |
| --- | --- | --- |
| S2-2506421 | OPPO | The Work Task also consider roaming aspects |
| S2-2506444 | MediaTek Inc. | For supporting interworking with EPS, interworking from 6GS to EPS is prioritized For supporting interworking with 5GS, ng-eNB is not in the scope of this study Interworking with non-3GPP access is not considered in this WT All these are reflected in the KIs description |
| S2-2506525 | Ofinno | How to support optimized use of 5GS and 6GS, with efficient/minimized signalling How a 6G core supports both NG-RAN and 6G-RAN How to define architectures suitable for each migration and interworking/mobility scenarios How to support roaming cases |
| S2-2506809 | NTT DOCOMO | Propose detailed aspects to study for each item of the WT defined in SID. For how to support migration to 6GS, including migration path identification, interface identification whether backward and/or forward compatible, 6GC NFs communicate with 5GC NFs, 6G UE communicates with 5GC, or 5G UE communicates with 6GC, etc For how to support interworking with 5GS, including inter-system handover and idle mobility between 6GS and 5GS, single or dual registration, IP address preservation, service continuity for IMS voice, etc For how to support interworking with EPS, including inter-system handover idle mobility between 6GS and EPS, mobility from 6GS to EPS directly or via 5GS, IP address preservation, etc For how to supporting interworking with NTN, including handover across satellites, mobility of satellite across different system, IP address preservation, etc. All these are reflected in the KIs description |
| S2-2506875 | Apple | The study aims to identify a migration path that can be supported by all types of 6G UEs, including single radio, dual radio, and devices of varying levels of complexity (e.g. low complexity devices). The study considers non-roaming interworking scenarios within an operator as well as interworking scenarios for roaming. |
| S2-2506928 | China Telecom | Identify what interfaces and procedures need to be defined to support the migration from 5GC to 6GC. Identify what level of service continuity, possibly specific per service, shall be maintained for UEs changing between 6GC and legacy 3GPP core network, e.g. the 5GC, EPC, 4G/5G NTN/satellite access that use EPS/5GS. Identify what interfaces and procedures need to be defined to support the interworking with the legacy network to ensure the service continuity on non-roaming and roaming case. All these are reflected in the KIs description |
| S2-2507230 | Xiaomi | Propose detailed aspects to study for each item of the WT defined in SID For migration and interworking, including identify migration or interworking scenarios, identify migration and interworking aspects and UE, CN requirements All these are reflected in the KIs description |
| S2-2507375 | Nokia, T-Mobile US | Specific requirements of interworking from any newly introduced 6GS feature are not within scope of this WT (will be addressed in the corresponding dedicated WT). Interworking between 5G PNI-NPN/SNPN, 5G Femto and 6G is within WT2 scope. 5G-6G interworking scope does not preclude any access control mechanism used in each RAT. The work task will consider roaming aspects. All these are reflected in the KIs description |
| S2-2507399 | Qualcomm Korea | For supporting migration to 6GS, study Whether and how to support Multi-RAT connectivity between 5G and 6G, including traffic steering, switching and aggregation across 5G and 6G. Propose to update NOTE 3 as The detailed migration study outcome will be coordinated and aligned with RAN |
| S2-2507447 | Samsung, AT&T | Propose Migration paths should be studied first for WT#2, including: - Migration path 1 from 5G SA into 6G SA - Migration path 2 from 5G NSA into 6G SA - Migration Path 3 from 5G NSA into 6G SA via 5G SA - Migration Path 4 from 5G NSA and 5G SA into 6G SA This was summarized as identifying potential migration paths and was added in sub-bullet 1.1 |
| S2-2506321 | InterDigital Inc. | For supporting interworking with 5GS, including: - How to minimize the use of combined network nodes. - What principles should be followed when the UE is associated with a 6G Service and moves to the 5G network (e.g. if the 6G service needs to contact the UE while the UE is registered to 5G or vice versa). |

The following updates have been done based on the summary of the input documents:

**** First Change (all text new) ****

Annex A.X. WT#2 Migration and Interworking Scope

Editor's Note: Describe the technical scope of the proposed Work Task. If applicable, suggest logical subdivision of this WT into smaller sub-WT. This clause is part of the TR Annex.

WT#2: Study migration and interworking, including

1	How to support migration to 6GS.

2	How to support interworking with 5GS (including when using 5G NTN/satellite access).

3	Whether and how to support interworking with EPS (including when using 4G NTN/satellite access).

NOTE 1:	Interworking with 2G/3G are not considered in this study. Interworking between 6GS and 5GS is assumed to work even if the UE has previously registered in 2G, 3G or 4G.

NOTE 2: The detailed migration study scope will be coordinated and aligned with RAN.

NOTE 3: The Work Task should consider roaming aspects.

**** End of Changes ****