SA WG2 Meeting #170	S2-2507983

25 – 29 Aug, 2025, Goteborg, SE

Source:	Nokia, NEC, Vivo, Lenovo, InterDigital, IIT Bombay, Orange, AT&T, T-Mobile USA, Samsung

Title:	[WT#1.x] On energy-related aspects

Document for:	Agreement

Agenda Item:	20.6.0

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: proposes that an extra WT/KI exists for energy related specific features not covered by other WT/KI.

# 1.	Discussion

The following documents proposes WT scopes and KI descriptions

| Tdoc | Title | Source |
| --- | --- | --- |
| S2-2506494 | [WT#1.x] On energy-related aspects | Nokia, NEC |
| S2-2506636 | [General aspects, Energy efficiency] End-to-end energy saving and energy efficiency | vivo |

# 2. Justifications

S2-2506494 Nokia, NECWT#1 of the approved 6G SID in SP-250806 includes the need to assess what 5GS features not mentioned already in the WT#1 itself should be supported in 6G (“In addition to the work tasks above, this study will identify other 5G features that will be supported and/or adapted in 6G.”). This paper proposes that energy-related aspects ought to be considered.

Architectural Motivation: The 6G system should be designed in all aspects with Energy efficiency and sustainability criteria as a basis of the specification work. However, there are also some features, not already covered by other work tasks, that specifically are aimed at addressing energy saving, energy-related information exposure and other energy-related features (e.g. covered by stage 1 requirements), and this may require some ad hoc procedures, dedicated services and impacting or requiring some new NF(s). Since rel-21 will provide an opportunity for feature level alignment among UEs, it is important to identify e.g. any UE impacting aspects of featured that are aimed at e.g. exposing information or saving energy in the network.

Gap Analysis: The KI will discuss whether any change needs to be made to the way the existing EnergySys features are specified. The existing features related to exposure e.g. lack some more precise indication on energy consumption, or precise indication of which the UEs that cause consumption are. Ther may also be dependencies on whether any applicable APIs or procedures currently used by dedicated features underwent any changes in 6G which at present is unknown. Tighter UE and network coordination for the purpose of reducing Network Energy Consumption end to end (e.g. taking into account user preference) could also be considered and 6G study offers a unique opportunity for this.

Technical Impact:

- Dedicated NF(s) could be required or 5GS EIF evolved or alternative functional allocation to NFs used also for other reasons can be determined.

- RAN/CN interaction or UE-network interaction may be impacted to e.g provide indication of high energy consumption for a UE, or to collect user input.

- Interactions between Network Functions (including the PCF)

- Interactions between NFs (including the PCF) and OAM

- If the existing 5GS Energy-related solutions are evolved and if new energy-related solutions are defined for 6G, how these interwork with existing 5GS features (if applicable) needs to be documented.

S2-2506636  vivo

Observation-1: UE power saving mechanisms is important for the devices, including UE energy efficiency, UE energy saving services.

- Motivation-1.1: UE energy efficiency

- In 6G system, more and more types of terminals, such as AR glasses, robots, AI enabled smart phones, will emerge and become popular. These terminals will integrate into our daily lives as an essential part. Certain immersive terminals, especially the wearable and light-weighted XR terminals such as AR glasses, are very sensitive to the power consumption due to the limitation of the battery capacity and weight. Furthermore, these light-weighted XR terminals also require to guarantee the QoS requirements for better service experience.

So it is critical and required for the 6G network to improve the terminals’ energy efficiency, i.e., guarantee the QoS requirements and reduce the devices’ communication power consumption.

- Motivation-1.2: New business model for UE energy saving

- UE energy efficiency and power saving technologies are beneficial for the UE and subscribers. Considering the fact that these power saving technologies may require the radio resources consumption, if there is a new business model for the UE energy saving, it will be the driven for the operators to deploy the UE power saving technologies.

The UE energy saving service could be a new business model beneficial for both operators and users. Certain devices or VIP users may subscribe the UE energy saving service in order to improve the UE energy efficiency, i.e., guarantee the QoS requirements and reduce the communication power consumption for the UE.

- Gap Analysis: Gaps on UE energy efficiency

  - UE energy efficiency is related with both the UE power consumption and QoS performance, e.g., data rate, latency. In 5G system, some UE power saving technologies, e.g., LP-WUS, C-DRX, etc., have been specified. However, the improvement of the UE energy efficiency needs to consider both the usage of the power saving technologies and the guarantee of QoS requirements.

In 6G era, considering more and more light-weighted XR devices, e.g., AR glasses, are integrated into our daily lives, the RAN needs to consider the both the UE’s QoS requirements and power saving technologies in order to meet the UE energy efficiency requirements for these devices or applications.

  - In 5G network, the main business model between the operators and subscribers is based on the data traffic transmission. Considering more and more communication technologies are applied into the 6G system, the exploration for the business model is also an important part for the operators to consider whether to deploy these technologies.

UE energy saving as service is a new business model in 6G system. This service is based on the deployment of the UE energy saving technologies. This new service not only helps the devices to reduce communication power consumption and guarantee the service experience, but also provides the high-value service scenario for the operators. This new business model has the following two advantages:

- The operators could provide the new value added service and attract more subscribers by deploying the energy saving technologies;

- The devices could get better service experience with guaranteed QoS and reduced communication power consumption.

- Technical Impact

- End-to-end support on the UE energy efficiency (i.e., reduce UE communication power consumption and without QoS degradation), including UE, RAN and core network.

- Support for UE energy saving service, e.g., subscription, potential interaction between CN and RAN, energy related policy control at PCF.

Observation-2: End-to-end cooperation (including core network, RAN and UE) for network energy saving is beneficial for the design of the 6G network energy saving.

- Motivation: End-to-end cooperation (including core network, RAN and UE) for network energy saving

For this part, the descriptions about motivation, gap analysis, technical impacts are similar with S2-2506494. Please also refer to the related motivations and gap analysis for more details.

**** First Change (all new text) ****

# Annex A.X. WT#1.x Energy-related aspects

This WT will study:

- Whether and how to support energy-related features and services, including e.g. identification of the causes of energy consumption and potential actions to address these causes. This may include identifying any new energy-related features and/or evolving or reusing existing 5GS energy-related features.

- Whether and how to enhance UE and network interactions and coordination for the purpose of reducing end-to-end Energy Consumption (e.g. taking user preference into account, notifications to UE). The first release of 6G provides a unique opportunity to align UE population capabilities for energy specific interactions.

- How to optimize UE energy efficiency while satisfying the QoS requirements (e.g. large bandwidth, low latency), including UE power consumption reduction and QoS guarantee, UE energy saving service;

- Whether and how to address the mobility between 5G and 6G for any new energy features supported in 6G.

NOTE:	 it is expected this work will start after the baseline architecture and system assumptions have been agreed.

**** Second Change (all new text) ****

# 5.X. Key Issue #X: Energy-related aspects

This KI will study:

- Whether and how to support energy-related features and services, including e.g. identification of the causes of energy consumption and potential actions to address these causes. This may include identifying any new energy-related features and/or evolving or reusing existing 5GS energy-related features.

- Whether and how to enhance UE and network interactions and coordination for the purpose of reducing end-to-end Energy Consumption (e.g. taking user preference into account, notifications to UE). The first release of 6G provides a unique opportunity to align UE population capabilities for energy specific interactions.

- How to optimize UE energy efficiency while satisfying the QoS requirements (e.g. large bandwidth, low latency), including UE power consumption reduction and QoS guarantee, UE energy saving service;

- Whether and how to address the mobility between 5G and 6G for any new energy features supported in 6G.

NOTE: it is expected this work will start after the baseline architecture and system assumptions have been agreed.

**** End of Changes ****