3GPP TSG-WG SA2 Meeting #170	S2-2508028

25th – 29th August, 2025, Gothenburg, Sweden	(revision of S2-2507883, 7204)

Source:	Lenovo, NEC

Title:	[WT#1.2, Slicing] Proposal to study Network Slicing in 6G

Document for:	Discussions and Agreement

Agenda Item:	20.6.1.2

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution proposes to study Network Slicing for 6G.

# 1. Input contributions for the baseline paper on [WT#1.2 Slicing]

| Slicing |  |  |  |
| --- | --- | --- | --- |
| 20.6.1.2 | S2-2506419 | 23.801-01: [WT#1.2, Network Slicing], Enhancements to Network Slicing | InterDigital Pennsylvania |
| 20.6.1.2 | S2-2507442 | 23.801-01: [WT#1.2, Network Slicing] Network Slicing Evolution in 6G | Apple, Google, AT&T |
| 20.6.1.2 | S2-2506493 | 23.801-01: WT#1.2: Proposal to have a dedicated KI on network slicing | Nokia, T-Mobile USA |
| 20.6.1.2 | S2-2506578 | 23.801-01: [WT#1.2, network slicing] Scope and New Key Issue for network slicing | LG Electronics |
| 20.6.1.2 | S2-2506641 | 23.801-01: [WT#1.2, Network Slicing] Simplified Network Slicing | vivo |
| 20.6.1.2 | S2-2506674 | 23.801-01: Justification: on network slicing simplification | Huawei, HiSilicon |
| 20.6.1.2 | S2-2506777 | 23.801-01: [WT#1.2, Network Slicing] Discussion and proposal for Network Slicing | ZTE, Orange |
| 20.6.1.2 | S2-2506815 | 23.801-01: [WT#1.2, Network Slicing] Support of Network Slicing in 6G | Samsung |
| 20.6.1.2 | S2-2506932 | 23.801-01: [WT#1.2, Network slicing] WT and KI for Network Slicing | Ericsson |
| 20.6.1.2 | S2-2507016 | 23.801-01: [WT#1.2, Network Slicing] Key issue on enhancements to Network Slicing for 6G | Qualcomm Korea |
| 20.6.1.2 | S2-2507119 | 23.801-01: [WT#1.2, Slicing] Discussion on Network Slicing in 6G | MediaTek Inc. |
| 20.6.1.2 | S2-2507204 | 23.801-01: [WT#1.2, Slicing] Proposal to study Network Slicing in 6G | Lenovo |

# 2.	Summary of the input contributions

## 2.1	Summary in table form

| TDoc | Sources | WT/KI summary WT#1.2 on network slicing |
| --- | --- | --- |
| S2-2506419 | InterDigital | WT scope: Study whether and how to support and/or enhance Network Slicing aspects in 6G - Study how to reduce the complexity of system procedures that rely on Network Slicing, focusing first on elementary procedures, such as mobility management and session management - Identify which 5G network slicing features, which were added in releases 15 thru 20, should be supported in the first release of the 6G system. The motivation for including features in day 1 of 6G is to avoid backwards compatibility issues in future releases of 6G. - Identify which network slicing features can be combined into a single feature (e.g. partially allowed network slices and slice rejected partially in a Registration Area). - Study whether and how new architectural enhancements may further reduce the complexity of current 5G Network Slice procedures, while preserving the advantages that existing 5G network slicing functionality provide. [The Key Issue studies is the same as the WT proposal] |
| S2-2507442 | Apple, Google, AT&T | WT scope: Study whether and how to support and/or enhance network slicing in 6G - Study how to simplify slice management and slicing-related control plane signalling between UE and network and make network slicing solution more agnostic to UE behaviour. Study the implications on UE procedures to map application traffic to network slices without impacting user privacy. - Determine what are the essential network slicing features from 5GS (based on lack of deployment in 5GS) that need to be supported in 6GS and study how to support those. NOTE X: This WT will also cover how to achieve backward compatibility and interworking with 5GS as far as required. This Key Issue aims to study how to support network slicing in 6G as per service requirements specified in TS 22.101 [x1], TS 22.261 [x2], TS 22.173 [x3]. Solutions for this key issue will describe: - How to modify network slicing functionality by reducing complexity in control plane signalling and slice management aspects and make network slicing solution more UE agnostic. - What are the implications on the UE procedures to map application traffic to network slices without impacting user privacy. - Which slicing features from 5GS are essential for 6GS and how to achieve backwards compatibility and interworking with 5GS as far as required. |
| S2-2506493 | Nokia, T-Mobile USA | WT scope: this work task aims at: a) Identifying the set of network slicing features specified in TS 23.501 and other 5GS specifications in SA2 remit to be supported in the initial release of 6G specifications. b) Identifying any possible enhancements of the network slicing features identified in point a) above and any new network slicing features in the first release of 6G e.g. to improve monetization and operations by e.g. service/connectivity type selection by the user, service activation notification/advice, or to improve operational aspects of network slicing in 6G. c) Identifying which of the network slicing features from the optional 5G network slicing features ported to 6G and the newly defined 6G network slicing features are mandatory or optional to be supported in the UE and which are mandatory or optional to be supported in the network. d) Studying any required changes to slicing procedures and features needed due to e.g. changes in MM and SM procedures in 6G and/or due to changes in the architecture (e.g. due to new NFs). e) Studying the impact on migration and interworking with 5GS for any modified or newly introduced network slicing features. Key Issue #X: same as the WT scope. |
| S2-2506578 | LG Electronics | WT scope: Study whether and how to simplify network slicing in 6GS. This Key Issue studies: - Whether and how to simplify network slicing based on following: - identify network slicing features needs to be supported by mandatory and optional in the UE and network - remove multiple options for the same purpose e.g. multiple NSAC architecture options - reduce signalling overhead and configuration of UE and network in non-roaming and roaming scenario - support slice interworking with 5GS |
| S2-2506641 | vivo | WT scope: Study 6G Network slicing, in order to efficiently, simply and with low complexity implement the network slice in UE side. KI: Study how to simplify the network slicing in UE side: - Option#1: study how to support that UE is not aware of the Network slicing, including: - Study how to determine the network slices to be used for a given service; - Study how to establish PDU Session for a given service; - Option#2: study how to simplify the Network slicing handling logic in UE side, including. - study how the network ensures the RSD is validate from the network slice aspect to UE. - study whether and how to determine the Requested NSSAI. |
| S2-2506674 | Huawei, HiSilicon | WT scope: Define the overall 6G network slicing with solutions that addresses the deployment issues of 5G network slicing considering the following aspects: - Try to avoid dependency on UE logic. - avoid complex configuration among operators to support roaming for network slicing. Key Issue #X: Network Slicing simplification Slicing is one of the most prominent features introduced in 5G. However, the deployment of slicing in production networks has not been as successful as expected due to its complexity and lack support from UE side. The value of slicing in enabling flexibility in the network configuration, optimisation of the connectivity and resources utilisation remains undeniable, therefore it is advantageous to pursue this value in 6G with solutions that addresses the deployment issues of 5G network slicing considering the following aspects: Simplify the mechanism employed by a UE to request the use of a slice in such a way as to: - minimize or avoid the impact on the UE. - ensure the slice information can be used efficiently. - ensure efficient selection/reselection mechanisms Solutions for this KI should also devise a way to simplify how slicing is configured and presented to the UE when roaming. |
| S2-2506777 | ZTE, Orange, Oppo | WT scope: No update in the Working task description. No need for sub-WT. Key Issue #X: Support network slicing in 6G Study whether and how to support network slicing with minimizing the complexity on functionalities and procedures at both UE side and NW side. The following aspects should be considered: - Whether to support network slicing during UE registration. - Whether and how to support network slicing in the SM signalling, including how the network slice can be selected by the network if the S-NSSAI is not provided by the UE. - Whether and how to support application triggered network slice replacement without relying on URSP rule. - Whether and how to simplify the network slice overall architecture, e.g. merging the network slicing related NFs in network side. |
| S2-2506815 | Samsung | WT scope: Study how to support network slicing for 6G - Identify a set of network slicing features to be re-used, enhanced, or re-designed for simplification (e.g., less dependency on UE, reducing options) - How to support a introduction of additional network slicing features without impacting UE Key Issue #X: same as the WT scope. |
| S2-2506932 | Ericsson | WT scope: The study of network slicing for 6G systems covers the following aspects - Investigate which 3GPP slice solution baseline/features used for 5G shall be used and evolved to support 6G. - Investigate what simplifications (e.g. simplification of the non-homogeneous slice support in the system) can be done to increase the traction in Core and in UE. - Investigate how to decrease dependency towards UE e.g., NAS upgrades, URSP to support an evolution, new slice related features. - Investigate impact on 5G functionality e.g., IRAT mobility and roaming, that any suggested simplification and/or evolution may create. Key Issue #X: same as the WT scope. |
| S2-2507016 | Qualcomm | WT scope: Study whether and how to support and/or enhance the following aspects in 6G: […] network slicing, […]: a.: For Network Slicing, study whether and how to support the following enhancements a.1: Optimization of signalling overhead due to Network Slicing for different scenarios, use cases and verticals, e.g. when only a single data session is used, a.2: Enhancements to facilitate the binding of applications to network slices when appID is not available to the 3GPP system, e.g. via enabling awareness of network slice capabilities at the UE. Key Issue #X: This key issue objectives are: - Study whether and how to support Network Slicing features in 6G, including configuration, authorization, policies, etc. - Study enhancements to Network Slicing including: - Optimization of signalling overhead due to Network Slicing for different scenarios, use cases and verticals, e.g. when only a single data session is used, - Enhancements to facilitate the binding of applications to Network Slices when appID is not available to the 3GPP system, e.g. via enabling awareness of network slice capabilities at the UE. |
| S2-2507119 | MediaTek Inc. | WT scope: This sub-WT focuses on studying whether and how to support and/or enhance network slicing. Below aspects are studied: X.1 Based on 5G SA network slicing introduced since Rel-15, to identify: - what network slicing functionality should be introduced into 6G, whether as is or modified and how; and - if modified, to characterise the issue being addressed and the related solutions - what network slicing functionality should not be introduced into 6G - interworking between network slicing in 5G SA and network slicing in 6G will be considered. X.2 Depending on X.1, to identify whether and how to support network slicing with new "beyond-communication" services including Compute, AI, Data, Sensing in view of network slicing with communication services. Key Issue #X: same as the WT scope. |
| S2-2507204 | Lenovo | WT scope: The technical scope for WT#6 should include: - Study the overall design of the 6G network slicing feature in order to improve: - how to minimize the network slice related configuration of the UE. - Study how to simplify the UE Route Selection Policy (URSP) for traffic routing to avoid i) dependency on UE implementation and ii) monitoring by the network. - Identifying the network slicing features from 5GS which do not need to be supported in 6G (at least in the first release) and identifying new the network slicing features to support flexible selection of connectivity (i.e. network slice and PDU Session) for a service traffic. Key Issue #X: same as the WT scope. |

## 2.2	Summary of the proposals from clause 2.1

The following topics are proposed to be studied:

1.	The overall design of the 6G network slicing feature, including the impacts on the registration, session management and authorization procedures, as well as considering scenarios like roaming and non-homogeneous slice support in the system. The overall network slicing design aims at:

a.	Identify possible enhancements to minimize the impact on the UE side (including introduction of additional network slicing features without impacting UE); and

b.	Identify possible enhancements to minimize the network slice complexity on the network side.

Supported by: InterDigital, Apple, Google, AT&T, Nokia, T-Mobile USA (“e.g. changes in MM and SM procedures in 6G and/or due to changes in the architecture”), LG, Vivo, Huawei (“Try to avoid dependency on UE logic”, “avoid complex configuration among operators to support roaming for network slicing”), ZTE, Orange, Oppo (“Whether and how to simplify the network slice overall architecture, e.g. merging the network slicing related NFs in network side”), Samsung, Ericsson (“simplification of the non-homogeneous slice support in the system”), QC (“for different scenarios, use cases and verticals, e.g. when only a single data session is used”), Lenovo.

2.	Which of the 5G network slicing functionality should be introduced in the first release of 6G specifications and which new 6G network slicing functionality need to be introduced (e.g. .g. to improve monetization and operations by e.g. service/connectivity type selection by the user, service activation notification/advice, or to improve operational aspects of network slicing in 6G; or based on the support of new 6G service as from WT#3 - WT#8).

a.	Define which of the 5G functionality and the new 6G functionality are mandatory to support in the UE and in the network.

Supported by: InterDigital, Apple, Google, AT&T, Nokia, T-Mobile USA, LG, Samsung, Ericsson, MediaTek, Lenovo.

3.	Whether and how 6G network slicing functionality impose any requirements on the URSP design for 6G (e.g. how application traffic maps to network slices).

Supported by: Apple, Google, AT&T (“What are the implications on the UE procedures to map application traffic to network slices without impacting user privacy”), LG, Vivo, Ericsson, QC (“biding of applications to network slices when appID is not available”), Lenovo (“simplify URSP for traffic routing”).

4.	Interworking and migration between network slicing in 5G and network slicing in 6G.

Supported by: Apple, Google, AT&T, Nokia, T-Mobile USA, LG, MediaTek.

# 3. Proposal for the TR

It is proposed to include the following text in the TR 23.801-01.

**** First Change ****

# Annex A.X. WT#1.2 Scope on Network Slicing

The scope of the Network Slicing work task includes:

1.	Study the overall design of the 6G network slicing feature, including the impacts on the registration, session management and authorization procedures (e.g. considering the impact on both UE and network side), as well as considering scenarios like roaming support in the system. The overall network slicing design will also consider:

a.	Identifying how existing and/or new application traffic description is used to map application traffic to a data connections while preserving user privacy.

2.	Which of the 5G network slicing functionality should be introduced in the first release of 6G specifications.

3.	Whether and which new 6G network slicing functionality need to be introduced.

NOTE:	For bullets 2. and 3., it will be studied which of the 5G functionality and potentially new 6G functionality are mandatory to support in the UE and in the network.

4.	Study impacts of the 6G network slicing on the interworking and migration with 5G.

**** Second Change ****

# 5.X. Key Issue for WT#1.2 on Network Slicing

This key issue proposes to study how to design the network slicing feature in 6G with the goal to ease the network slicing deployment.

The key issue will investigate the following aspects:

1.	Study the overall design of the 6G network slicing feature, including the impacts on the registration, session management and authorization procedures, as well as considering scenarios like roaming support in the system. The overall network slicing design aims at:

a.	Identifying possible minimization of dependencies on the UE side;

b.	Identifying possible minimization of the network slice complexity on the network side; and

c.	Identifying whether and how application traffic maps to network slices.

2.	Which of the 5G network slicing functionality should be introduced in the first release of 6G specifications.

3.	Which new 6G network slicing functionality need to be introduced (e.g. to improve monetization and operations by e.g. service/connectivity type selection by the user, service activation notification/advice, or to improve operational aspects of network slicing in 6G; or based on the support of new 6G service as from WT#3 - WT#8).

NOTE:	For bullets 2. and 3. study which of the 5G functionality and new 6G functionality are mandatory to support in the UE and in the network.

4.	Interworking and migration between network slicing in 5G and network slicing in 6G.

**** End of Changes ****