SA WG2 Meeting #170	S2-2508116

# Goteborg, Sweden, August 25 – 29rd 2025              a revision of S2 2504998, 6886, 7955, 8007

Title:	LS for a new measurement related to the maintainability of a QoS flow

Response to:

Release:	Rel-19

Work Item:	eNA_Ph3, TEI19

Source:	SA2

To:	SA5

Cc:

Contact Person:

#### Name:	 Adrian Buckley

E-mail Address:	 adrian dot buckley at verizon dot com

Uri Baniel

E-mail Address:	 uri dot baniel at oracle dot com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	N/A

1.Overall Description:

In TS 23.288 clause 6.9, QoS Sustainability Analytics can provide analytics information regarding the potential QoS change statistics for an Analytics target period in the past in a certain area or the likelihood of a QoS change for an Analytics target period in the future in a certain area.   The consumer of this analytics provides reporting threshold(s) to indicate the conditions on the analytic level for triggering the reporting. In other words, when the analytics result crosses the threshold of the expected QoS KPI, a report is triggered.

As indicated in TS 23.288 clause 6.9.1, for a GBR flow, the reporting threshold(s) refers to the QoS flow Retainability KPI. In other words, the sustainability of the flow is determined by a threshold related to how often the end user abnormally loses the QoS flow during the time the QoS flow is used. The QoS flow Retainability KPI is defined in TS 28.554 clause 6.5.1.1

The QoS flow Retainability KPI is not sufficient to determine the ability of the wireless system to maintain a flow with some level of acceptable degradation.

If the OAM system has the ability to monitor and capture statistics of downgraded QoS flows by for example counting the QoS degradation events per a period of time, then in addition to QoS flow Retainability, a QoS flow Maintainability should be added providing a measurement that reflects the probability to experience a QoS degradation. The measurement will serve as a measure to determine the ability of the wireless system to maintain the flow.

2. Actions:

To SA5:

ACTION:

- SA2 would like to understand the present or future ability of SA5 to monitor and capture statistics of downgraded QoS flows.

- If the answer to question a is positive, SA2 kindly asks SA5 to consider adding a QoS Maintainability measurement on the basis of QoS flow level. The measurement will reflect the probability to experience a QoS degradation of any kind during the time when the QoS flow is used.

3. Date of Next TSG-SA WG2 Meetings:

|  | DATES | LOCATION | CTRY |
| --- | --- | --- | --- |
| SA2#171 | October, 25th-29th | Wuhan | China |
| SA2#172 | November, 17th – 21st | Dallas | Texas |