3GPP TSG-WG2 Meeting #170	S2-2506346

August 25 – 29, 2025, Göteborg, Sweden	(revision of S2-250xxxx)

Source:	Ericsson

Title:	KI#2, New Solution: A consumer-requested sensing service authorization

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract of the contribution:

This contribution proposes a new solution for KI#2 that aims at providing a baseline for a consumer-requested sensing service authorization performed by the core network.

# 1. Discussion

This document proposes a new solution for KI#2 that aims at providing a baseline for a consumer-requested sensing service authorization performed by the core network.

# 2. Text Proposal

It is proposed to agree and include the following solution into 3GPP TR 23.700-14.

>>>> Start of Changes <<<<

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Solutions | #1 | #2 | #3 | #4 | #5 | #6 |
| #1 | X |  | X | X | X |  |
| #2 |  | X |  |  | X |  |
| #3 | X | X | X | X | X | X |
| #4 | X |  |  |  |  |  |
| #5 | X |  |  | X |  |  |
| #6 | X | X | X |  |  |  |
| #7 | X |  |  |  |  |  |
| #8 |  | X |  |  |  |  |
| #9 |  | X |  |  |  |  |
| #10 |  | X |  |  |  |  |
| #11 |  | X |  |  |  |  |
| #12 |  | X |  |  |  |  |
| #13 |  |  | X |  |  |  |
| #14 |  |  | X |  |  |  |
| #15 |  |  | X |  |  | X |
| #16 |  |  | X |  |  |  |
| #17 |  |  | X |  |  |  |
| #18 |  |  |  | X |  |  |
| #19 | X | X |  |  | X |  |
| #20 |  |  |  |  |  | X |
| #XY |  | X |  |  |  |  |

>>>> Next Change (all new text)<<<<

## 6.XY	Solution #XY: A consumer-requested sensing service authorization

### 6.XY.1	High-level solution principles

The solution is based on following principles:

-	A sensing service consumer (e.g., AF) requests a sensing service from a NEF. The NEF performs authorization of the sensing service consumer, and, if the sensing service consumer is authorized, forwards the received request to an SnCF;

-	The SnCF performs sensing request specific authorization on the operation level (including the sensing service consumer requested parameters, sensing service consumer NF Type etc.). If authorized, the sensing service consumer may proceed with the requested sensing service.

### 6.XY.2	Impacts on the Key Issues

This solution impacts other key issues in the following way:

-	KI#1: A new core network function is considered – Sensing Core Function (SnCF) that handles sensing request in the core network. NEF can be used to expose the obtained sensing result to the sensing service consumer (e.g., AF that invoked the sensing service request),

-	KI#3: SnCF selection is performed by the NEF; the SnCF selects a NG-RAN node to perform sensing data collection.

### 6.XY.3.	Functional Description

Two network entities may be involved to perform authorization of the sensing service requested by a sensing service consumer (e.g., AF) from the network. After receiving a sensing service request, an NEF determines whether the sensing service consumer is authorized to invoke sensing APIs to the network. In case the sensing service consumer is authorized to do so, the NEF determines parameters for the request and then sends the sensing service request to an SnCF. Otherwise, the NEF reject the sensing service request due to authorization failure.

NOTE XY1:	How the NEF discovers and selects an SnCF to handle a specific sensing service request is outside the scope of this solution.

The SnCF performs a sensing request specific authorization in the implementation specific way, considering, for instance, sensing service consumer NF type, event reporting mode, sensing data exposure parameters, and other parameters from the sensing service request. In case of successful authorization, the SnCF proceeds with the requested sensing service. If the SnCF determines not to grant the authorization, the sensing service request is rejected and the sensing service consumer is informed.

### 6.XY.4	Procedures

Procedure for a consumer-requested sensing service authorization performed by the core network entities is shown in Figure 6.XY.4-1.

NG-RAN

SnCF

Sensing service consumer (e.g. AF)

NEF

1. Sensing service request

2. Authorization of the sensing service consumer (e.g., AF) requesting the sensing service

3. Sensing service response

4. Sensing service request to the SnCF

5. Sensing request authorization on the operation level (incl. the sensing service consumer requested parameters, sensing service consumer NF type, etc.)

6. SnCF responds to the requested sensing service

7. NEF responds to the requested sensing service

8. Proceeding with the requested sensing service if the authorization is successful

Figure 6.XY.4-1: Procedure of the requested sensing service authorization.

1.	A sensing service consumer, e.g. AF, invokes a sensing service request to a NEF. Inside the request the sensing service consumer includes the following mandatory and optional parameters:

-	mandatory: AF Identifier (in case a sensing service consumer is an AF), External Target Sensing Area information (e.g., geographical area information and, optionally, altitude information), Type of sensing to perform (e.g., SensingEventType = “Object Detection”, “Object Tracking”), sensing result detail level (e.g., an indication about the detected object, parameter determined, as well as time and/or spatial resolution of the sensing data);

-	optional: event reporting mode (e.g., one-time, event-triggered or periodic reporting), sensing data exposure parameters (e.g., type of streaming and transport mechanism to be applied in case the streaming of the obtained sensing data is being requested).

NOTE XY1:	Other parameters for a sensing service request can be considered and added during the later phases.

2.	The NEF determines whether the sensing service consumer (e.g., AF) is authorized to request sensing services from the network, i.e., to invoke sensing service-specific APIs.

3.	If the authorization is not granted, the NEF sends the response to the sensing service consumer (e.g., AF) with a Result value indicating that the authorization has failed, and all further steps are skipped.

4.	If the authorization is granted, the NEF sends the sensing service request to the SnCF and includes parameters from the sensing service consumer; if required, the NEF maps them to 3GPP internal parameters.

5.	The SnCF performs a sensing request specific authorization in the implementation specific way, considering, for instance, sensing service consumer NF type, event reporting mode, sensing data exposure parameters, and other parameters from the sensing service request.

NOTE XY2:	In case a sensing service request is invoked by an NF/AF within the operator’s trust domain, the existing NF discovery and authorization procedures apply.

6-7.	If the SnCF does not grant the authorization for the requested sensing service operation, the SnCF rejects the request and responds (via NEF) to the sensing service consumer with a Result value indicating that the authorization has failed, and all further steps are skipped.

8.	If the authorization is granted, the sensing service continues to provide the requested sensing result.

### 6.XY.5	Impacts on existing services, entities and interfaces

AF:

-	acts as a Sensing Service Consumer and sends a sensing service request to the NEF.

Network Exposure Function (NEF):

-	performs authorization of a sensing service consumer (e.g., AF) by determining whether the sensing service consumer (e.g., AF) is authorized to invoke the sensing-specific APIs to the network.

Sensing Core Function (SnCF):

-	performs a sensing request specific authorization.

>>>> End of Changes <<<<