3GPP TSG-SA2 Meeting #170	S2-2506488

# Gothenburg, Sweden, August 25-29 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 5528 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | On introducing NSSAA over EPC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, T-Mobile USA, Oracle, Orange |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | DUMMY (TBD, e.g. TEI20_NEPS) |  |  |  |  |  | Date: |  |  | 2025-07-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The 5G Standalone architecture is still being rolled out in many countries and some operators are still using 5G NSA. It has become clear that the situation where a UE moves and remains in EPS for quite some time or is powered up in EPS cannot be excluded at present time in many deployments worldwide. This makes the rollout of certain network slicing features almost impossible as the semantics of these features is broken when the UE is in EPS and needs to continue or initiate service in PDN connections associated to network slices affected by these features (e.g. NSSAA). This creates hurdles to adoption of the same features also in 5GS despite some MNOs see use cases. It is proposed to provide normative specification so that it is possible to perform NSSAA in EPS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specify the support NSSAA while the UE is served by the EPS by using an approach similar to what is documented in the normative Annex H on the "Support of EAP-based secondary authentication and authorization by DN-AAA over EPC". |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of adoption in PLMNs where EPS can still be used by a UE |  |  |  |  |  |  |  |  |
| Clauses affected: |  | (new) Annex Y |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

Annex Y (normative): Support of EAP-based NSSAA over EPC

# Y.1	Introduction

Network Slice Specific Authentication and Authorization (NSSAA) by a AAA Server for a network slice over 3GPP access to EPC, is supported based on following principles:

-	A SMF+PGW-C shall be used to serve S-NSSAI requiring NSSAA by a AAA Server (AAA-S).

-	If the UE has included the PDU Session ID in PCO, the UE may indicate in the PCO within the PDN connection establishment request its support for NSSAA over EPC. The UE may also include the S-NSSAI-specific identity in the PCO. The SMF+PGW-C may reject the PDN connection establishment if the UE does not support NSSAA over EPC while local policies indicate that NSSAA is mandatory to access the DN associated to a S-NSSAI subject to NSSAA. In 5GS, when a PDU Session is established, the UE may also indicate via 5GSM capability parameter that it supports secondary DN authentication and authorization over EPC so that the SMG+PGW-C knows the capability and can store it for the UE.

-	For an APN associated with a S-NSSAI subject to NSSAA, when the UE indicates support of NSSAA over EPC in PCO at PDN connection establishment, the SMF+PGW-C check whether there is already any PDU session registered at UDM associated with same S-NSSAI. If so, NSSAA can be skipped as it has already been executed previously to enable that PDU session to be established.

-	The interface towards the UE is different (usage of EPC NAS instead of 5GC NAS) between the EPC and 5GC cases.

-	The MME and SGW are not impacted by the procedure. Specific exchanges between the UE and the SMF+PGW-C for NSSAA are carried via PCO. This includes the support of EAP exchanges between the UE and the AAA Server.

-	As it is only possible to exchange PCO once between the UE and the PGW during PDN connection establishment, the PDN connection is established before EAP-based NSSAA takes place.

-	When NSSAA has successfully taken place, the SMF+PGW-C allows traffic exchange at the UPF and indicates to the UE that User plane traffic is now possible. The SMG+PGW-C also updates the UDM with the successful outcome of the NSSAA procedure by registering NSSAA status.

-	When NSSAA has failed, the SMF+PGW-C releases the PDN connection. The SMG+PGW-C also updates the UDM by deregistering the SMF+PGW-C if this was the only PDN connection at SMF+PGW-C, or by updating NSSAA status for the S-NSSAI at the entry for the SMF+PGW-C.

# Y.2	Procedures

## Y.2.1	NSSAA at PDN Connection Establishment

In the figure Y.2.1-1, the execution of the NSSAA over EPC is specified. The procedure assumes that:

-	The APN is associated with the selection of a SMF+PGW-C to serve APN(s) that require NSSAA at PDN connection establishment.

-	The SMF+PGW-C is configured with local policies indicating that the APN requires NSSAA at PDN connection establishment for supporting UEs.

-	A UE supporting the NSSAA feature is assumed to be configured with credentials and identities for NSSAA for each slice which is subject to NSSAA.

Figure Y.2.1-1: EAP-based NSSAA at PDN connection establishment

1.	As steps 1 - 13 of Figure 5.3.2.1-1 in TS 23.401 [13] (Attach Request) or as steps 1 to 3 of Figure 5.10.2 in TS 23.401 [13] (UE requested PDN connectivity) with following modifications: The UE may indicate in PCO its capability to support NSSAA over EPC if the UE included the PDU Session ID in PCO. The UE may also include the NSSAA-specific identity.

2.	The SMF+PGW-C gets subscription data from UDM/HSS as defined in step 4 of Figure 4.3.2.2.1-1. If configuration at SMF+PGW-C or subscription information for the S-NSSAI indicate the S-NSSAI is subject to NSSAA, the SMF+PGW-C then checks the UE supports NSSAA over EPC. If the UE does not support the NSSAA in EPS, the SMF+PGW-C rejects the establishment of the PDN connection (not shown in the figure Y.2.1-1). If the UE supports the NSSAA in EPS, the SMF+PGW-C queries the UDM/HSS via Nudm_UECM_Get operation to check whether the UE has any PDU session/PDN connection successfully established for the S-NSSAI at a SMF+PGW-C. If there is any SMF+PGW-C already registered for this S-NSSAI, the SMF+PGW-C proceeds to consider the user authorized for the S-NSSAI and then it would accept the PDN connection establishment and continues Steps 15-24 in Figure 5.3.2.1-1 of TS 23.401 [13] or steps 5-16 in Figure 5.10.2 of TS 23.401 [13]. Otherwise, it continues from step 3 and UPF selection and N4 session establishment is executed with the difference that the SMF+PGW-C configures the UPF+PGW-U to block any UE traffic over the PDN Connection (until the NSSAA over EPC has been done and is successful).

3.	[conditional] Steps 15-24 in Figure 5.3.2.1-1 of TS 23.401 [13] or steps 5-16 in Figure 5.10.2 of TS 23.401 [13].

During the Attach procedure, at step 15 in Figure 5.3.2.1-1 of TS 23.401 [13] or during UE requested PDN connectivity in step 5 in Figure 5.10.2 of TS 23.401 [13], the SMF+PGW-C includes in PCO, an Indication to the UE that "UpLink Data is NOT ALLOWED" on the PDN connection. The UE shall not send Uplink data to the network, until it receives an indication further from the network that "UpLink Data is ALLOWED". If the SMF+PGW-C detects that the NSSAA needs to be executed, the S-NSSAI is indicated as in Pending status to the UE and this means no further PDU sessions can be requested for the S-NSSAI in EPC until the outcome of NSSAA is obtained. The UE knows the sessions that relates to the same S-NSSAI based on the URSP information it may hold related to the APN/DNN in RSD.

NOTE:	How the Indication that Uplink data allowed/not allowed is carried in PCO is defined in TS 24.501 [25].

4.	[Conditional] If the SMF+PGW-C detect that the NSSAA needs to be executed, the PGW-C+SMF initiates EAP-based authentication. Multiple round-trip messages as required by the authentication method used by NSSAA may follow. The PCO including the authentication message from the NSSAA is transferred to the UE by the SMF+PGW-C in Update Bearer Request and then over S1 by Downlink NAS Transport (e.g. steps 4a-4b). The response from the UE is transferred to the SMF+PGW-C in an Uplink NAS Transport over S1 and Update Bearer Response (e.g. steps 4c-4d) over EPS.

5.	[Conditional] The SMF +PGW-C registers the PDU session/PDN connection in UDM.

6.	[Conditional] If the NSSAA failed, the SMF+PGW-C would send a PGW initiated Bearer Deactivation indicating all the bearers of the PDN connection associated with the S-NSSAI need to be deleted and the cause set to "NSSAA failure" in PCO. Step 7 is not executed. The SMG+PGW-C also ensures removal of context at UPF/PGW-U. The SMF deregisters the PDU session/PDN connection from the UDM.

7.	[Conditional] The SMF+PGW-C updates the UE by invoking the PDN GW initiated Bearer Modification without QoS update procedure (figure 5.4.3-1 of TS 23.401 [13]) initiated by sending an Update Bearer Request message to the SGW. The PCO includes an indication that "UpLink Data is ALLOWED" and a "NSSAA success" IE in the PCO, which also allows the UE to request other PDU sessions for the S-NSSAI. The UE confirms the update (see clause 5.4.3 of TS 23.401 [13]).

If the UE IP address is to be delivered to the UE over user plane (via Router advertisement or DHCP) then the UE IP address is only delivered to the UE after step 7.

If dynamic PCC is to be used for the PDU Session, the SMF+PGW-C performs an SM Policy Association Establishment procedure as defined in clause 4.16.4 In this step the SMF+PGW-C may retrieve the PDU Session related policy information and the PCC rule(s) from the PCF, e.g. the authorized Session AMBR. The SMF+PGW-C updates the N4 rules in the UPF+PGW-U to allow traffic over the PDN Connection.

## Y.2.2	NSSAA Re-authorization and Re-authentication over EPC

At any time, the AAA-S may trigger re-authorization and re-authentication of a user associated with a GPSI. If so the NSSAAF receives a request to re-authorize for a network Slice identified by a S-NSSAI a user associated with a GPSI by the AAA-S and determines to which node to send the re-authorization notification based on the S-NSSAI the re-authorization is for. To do so, the NSSAAF checks with UDM whether only SMF+PGW-C for the S-NSSAI or also an AMF for the S-NSSAI exist for the UE. If only SMF+PGW-Cs addresses/IDs for which a PDN connection for the slice is registered are returned, then the NSSAAF triggers NSSAA procedure as per Figure Y.2.2-1 via one of such SMF+PGW-C.

NOTE: If an AMF address was returned from UDM, the NSSAA re-authorization would occur via AMF.

If the re-authorization fails, including when the SMF+PGW-C receives an indication from the MME that the UE is unreachable, then the SMF+PGW-C informs the AAA-S that UE is not reachable for re-authentication. The AAA-S may reinitiate the re-authorization and re-authentication up to a configurable number of times, after which a revocation is initiated as per clause Y.2.3.

Figure Y.2.2-1: EAP-based NSSAA re-authentication and re-authorization  over EPC

- The AAA-S triggers a re-authentication and re-authorization for a user associated with a GPSI in a certain network slice associated with a S-NSSAI.

- The NSSAAF executes a Nudm_UECM_Get service operation to discover any SMF+PGW-C or AMF associated with the UE and S-NSSAI. If only SMF+PGW-Cs are returned, then the NSSAAF picks one SMF+PGW-C to execute the NSSAA re-authentication via the EPC. If an AMF for the UE is returned, then the NSSAA re-authentication and re-authorization procedure is executed via the AMF as in clause 4.2.9.3. If no SMF+PGW-C nor AMF, then see step 3.

- The NSSAAF provides an acknowledgement to the AAA protocol Re-Auth Request message. If no SMF+PGW-C nor AMF is registered in UDM the procedure is stopped here.

- The NSSAAF triggers the NSSAA for the S-NSSAI via the selected SMF+PGW-C for the UE identified by the GPSI.

- NSSAA is executed as in steps 4 of figure Y.2.1-1. If successful, then no action is taken by AAA-S. Otherwise the AAA-S may reattempt the procedure a configurable number of times according to own policy and then if failure persists a revocation as defined in clause Y.2.3 is executed.

## Y.2.3	NSSAA revocation over EPC

At any time, the AAA-S may trigger in a network slice identified by a S-NSSAI a revocation of authorization for a user associated with a GPSI. If so the NSSAAF receives this request by the AAA-S and checks to which node send the revocation notification request. To do so the NSSAAF checks whether any SMF+PGW-C or AMF is registered for the S-NSSAI for the UE. The UDM/HSS returns all the SMF+PGW-C addresses/IDs for which a PDN connection for the S-NSSAI is registered, and/or any AMF for the S-NSSAI. If only SMF+PGW-Cs are returned, then the NSSAAF notifies all these SMF+PGW-Cs that a revocation of service is required for the GPSI in the slice related to S-NSSAI a shown in figure Y.2.3-1. These SMF+PGW-Cs proceed then to remove all the PDN connections in the S-NSSAI for the GPSI by a PGW initiated Bearer Deactivation indicating all the bearers of the PDN connections need to be deleted for each PDN connection associated to the S-NSSAI and GPSI the SMF+PGW-C supports. If an AMF, then the NSSAAF executes the revocation via AMF as per clause 4.2.5.4.

Figure Y.2.3-1: EAP-based NSSAA revocation over EPC

- The AAA-S triggers a revocation for a user associated with a GPSI in a certain network slice associated with a S-NSSAI.

- The NSSAAF executes a Nudm_UECM_Get service operation to discover any SMF+PGW-C or AMF associated with the UE and S-NSSAI. If only SMF+PGW-Cs are returned, then the NSSAAF repeats the revocation towards all of these SMF+PGW-Cs at step 4. If an AMF for the UE is returned, then the NSSAA revocation procedure is executed via the AMF as in clause 4.2.9.4. If no SMF+PGW-C nor AMF is returned, then see step 3.

- The NSSAAF provides an acknowledgement to the AAA protocol RevocationRequest message. If no SMF+PGW-C for the UE and S-NSSAI nor AMF for the UE is registered in UDM the procedure is stopped here.

- The NSSAAF triggers the SMF+PGW-Cs for the S-NSSAI identified in step 2 to release all the PDN connections associated with the S-NSSAI for the UE identified by the GPSI by sending a Nnssaaf_NSSAA_RevocationNotification for the GPSI and S-NSSAI.

- Each SMF+PGW-C receiving the NSSAA revocation message from step 4 releases the PDN sessions with APN associated with the S-NSSAI by a PDN GW Initiated Bearer Deactivation with indication to remove all bearers in the PDN connection as defined in clause 5.4.4.1 of TS 23.401 [13] and a cause code in PCO "NSSAA revocation". The SMF+PGW-C deregisters PDU sessions for the UE and S-NSSAI at the UDM/HSS.

## Y.2.4	Modification of 5GS behaviour for NSSAA

The AMF may hold NSSAA results status "already authorized" once the UE has successfully passed NSSAA. However after a successful execution of NSSAA over EPS and mobility to 5GS, the AMF may not have such status. Therefore, the AMF, before executing NSSAA, for a UE that supports NSSAA over EPC and is inbound from the EPS, may check with the UDM if any PDU session is registered for the UE and S-NSSAI. If so, NSSAA can be skipped and the AMF stores NSSAA result status is "already authorized" for the UE.  This is shown in figure Y.2.4-1 here below.

Figure Y.2.4-1: Modification of 5GS behaviour for UEs supporting NSSAA over EPC

1) UE requests to register for a S-NSSAI subject to NSSAA, indicates support of NSSAA over EPC in UE MM Core Network capability in clause 5.4.4a of TS 23.501 [2].

2) The AMF checks the NSSAA result status stored in the UE context for the UE and S-NSSAI in the stored UE context (if any is available). If NSSAA result status is "already authorized" then all steps except Step 7 are skipped, otherwise the AMF continues with step 3 for the UE.

3) [Conditional] If the UE has indicated that it supports NSSAA over EPC, the AMF queries the UDM via Nudm_UECM_Get to obtain any SMF+PGW-C already registered for the UE and S-NSSAI in the UDM

4) [Conditional] The AMF check the result of the query at step 3. If the UE has already PDN connections at any SMF+PGW-C for the S-NSSAI Step 6 is executed, otherwise the AMF continues with step 5.

5) [Conditional] NSSAA is executed as per clause 4.2.9.2

6) if NSSAA is successful or at step 4 the UE has already PDN connections at any SMF+PGW-C for the S-NSSAI, the NSSAA result status is set to "already authorized".  If not successful, the S-NSSAI is included in rejected S_NSSAI for the failed or revoked NSSAA allowed NSSAI in step 7 and any NSSAA status at AMF is marked as "not authorized".

7) If the NSSAA was successful, the S-NSSAI from step 1 is included in the Allowed NSSAI, otherwise it is not included in the Allowed NSSAI and is rejected.

End of changes