SA WG2 Meeting #170	S2-2506610

25 - 29 August, 2025, Goteborg, Sweden

Source: 	vivo

Title: 	Interim agreements on KI#1: System Architecture to Support Sensing

Document for: 	Approval

Agenda Item: 	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract of the contribution: This contribution provides interim agreements on KI#1: System Architecture to Support Sensing.

1	Discussion

At June 2025 SA#108 meeting, R20 5GA SA2 ISAC (see approved SID SP-250833) was down-scoped into gNB-based sensing to aligned with Rel-20 5G-A RAN ISAC study scope. Based on this, we could have:

Observation#1: Only gNB can be performed as Sensing Entity (SE).

Up to May 2025 SA2#169 meeting, 7 Solutions was agreed for KI#1 and captured in TR 23.700-14 V0.2.0.

After analysing and evaluating the architecture proposals, new NF function definitions, existing NF impacts and end-to-end procedure definitions captured in agreed solutions for KI#1, following common aspects can be identified and could be captured as agreed principle for KI#1:

Observation#2: A new CN function Sensing Function (SF) is introduced to support Sensing service which provides following key functionalities:

- NF profile registration for the discovery of SF instances, e.g., by an NEF.

- Handling Sensing service request from Sensing service consumer.

- Sensing service request authorization and revocation.

- Sensing Entity selection and re-selection.

- Sensing data processing and Sensing result generation/exposure.

- Sensing configuration parameters provision.

Observation#3: The Sensing Entity (i.e. gNB) provides following key functionalities:

- Sensing service operations using the provisioned Sensing configuration parameters.

- Data tunnel establishment towards the SF for Sensing data transfer.

- Sensing data report to the SF.

Observation#4: The NEF needs to support following key functionalities:

- Handling Sensing service request from the AF.

- SF discovery and selection.

- Sensing service request authorization.

It proposes to capture above observations as interim agreements on KI#1: System Architecture to Support Sensing.

2 	Proposal

It is proposed to capture the following contents into TR 23.700-14 V0.2.0.

# * * * * First change* * * *

7	Interim agreements

7.1	Agreed Principles

7.1.1	Agreed Principles for KI#1

Editor's note:	This clause will include the principles that are agreed as work progresses for the specific KI#Y. This may be populated directly or e.g. also when a topic in clause 7.2.Y gets resolved and a principle is agreed.

The following interim agreements on KI#1: System Architecture to Support Sensing are made:

1.	A new core network function Sensing Function (SF) is introduced to support Sensing service. The SF performs the following functionality:

a.	The SF registers itself in the NRF with its NF profile, this is to enable the discovery of SF instances e.g. by an NEF. The NF profile at least includes the SF ID/address, NF type, information used to allow the NEF to discover SF instances. Other details of the NF profile, if needed, will be completed in normative phase.

b.	The SF receives a Sensing service request from a Sensing service consumer. If the AF is Sensing service Consumer and deployed in an untrusted third part, The Sensing service request will firstly send to the NEF and then send to the SF.

c.	The SF performs Sensing service request authorization to decide whether the requested Sensing service is allowed. Once the Sensing service was triggered, the SF can also perform Sensing service revocation if required, based on AF request or local decision. More details of Sensing service authorization and revocation are captured in the interim agreements on KI#2.

d.	The SF discovers and selects gNB as Sensing Entity (SE), e.g. based on the target Sensing service area in Sensing service request and triggers the gNB to perform Sensing service operations towards the environment and/or objects. More details of SE selection are captured in the interim agreements on KI#3.

e.	The SF receives Sensing data from the SE and performs Sensing result generation. The SF further exposes the Sensing results to the Sensing service Consumer. More details of Sensing data collection, Sensing result generation/exposure are captured in the interim agreements on KI#4 and KI#5.

f.	The SF provisions required Sensing configuration parameters to the SE for Sensing service operations in a separate provision procedure or piggybacked in the Sensing service request procedure. More details of Sensing configuration parameters provision are captured in the interim agreements on KI#6.

2.	Only gNB can be performed as Sensing Entity (SE). The SE performs the following functionality:

a.	The SE receives a Sensing service request from the SF and then performs Sensing service operations towards the environment and/or objects using the provisioned Sensing configuration parameters.

b.	The SE establishes a data tunnel towards the SF for Sensing data transfer, e.g. using the SF ID or SF IP address included in the provisioned Sensing configuration parameters.

c.	The SE reports the collected Sensing data to the SF over the established data tunnel. The frequency of Sensing data report is controlled by the SF, e.g. one time, periodic or event trigger indicated in the provisioned Sensing configuration parameters.

3.	The NEF needs to be enhanced to support the following functionality:

a.	In case of the AF is Sensing service Consumer and deployed in an untrusted third part, the NEF receives a Sensing service request from the AF.

b.	The NEF discovers and selects the SF for Serving Service request, e.g. based on the target Sensing service area in Sensing service request and then send the Sensing service request to the selected SF.

c.	The NEF performs Sensing service request authorization to decide whether the AF is allowed to request the Sensing service.

# * * * * End of change* * * *