3GPP SA WG2 Meeting #170	 S2-2506899

# Gothenburg, Sweden, 25-29 August 2025					             			    (was S2-250xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0560 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of parameters for ProSe additional parameters announcement request message |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph3 |  |  |  |  |  | Date: |  |  | 2025-08-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As indicated in CT1 LS on parameters for ProSe additional parameters announcement request message (S2-2506121/ C1-254010), the "Relay Service Code" and "Root Relay Info" are not required in the Additional Parameters Announcement Request message, as they can be derived by the Relay from the information of the layer-2 link the request message is received. Therefore, for protocol efficiency, the two elements should be removed from the request message. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the "Relay Service Code" and "Root Relay Info" from the Additional Parameters Announcement Request message description in clause 6.3.2.5.4. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect description that prevents correct development of stage 3 specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2.5.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# >>>>BEGINNING OF CHANGES<<<<

##### 6.3.2.5.4	Additional parameters announcement procedure over 5G ProSe Multi-hop UE-to-Network Relay

Additional parameters announcement procedure outlined in figure 6.3.2.5.4-1 is used by a 5G ProSe Remote UE to request a 5G ProSe UE-to-Network Relay to announce additional parameters (using Model A). The Additional parameters announcement procedure specified in clause 6.5.1.3 is used with the following differences described in the procedure below.

Figure 6.3.2.5.4-1: Procedure for additional parameters announcement for Multi-hop UE-to-Network relaying

1.	5G ProSe Remote UE has discovered a 5G ProSe UE-to-Network Relay for connectivity to the network as described in clause 6.3.2.5.2 and requires additional parameters.

2.	The 5G ProSe Remote UE sends to the 5G ProSe Intermediate UE-to-Network Relay an Additional Parameters Announcement Request to obtain additional parameters.

3-4.	The Additional Parameters Announcement Request is sent up to the 5G ProSe UE-to-Network Relay.

5.	The 5G ProSe UE-to-Network Relay acknowledges receipt of the request in step 4 with an Additional Parameters Announcement Response (Additional_Parameters_Announcement_Request_Refresh Timer).

6-7.	The Additional Parameters Announcement Response is sent to the 5G ProSe Remote UE, relayed via the 5G ProSe Intermediate UE-to-Network Relay.

8.	The 5G ProSe UE-to-Network Relay reuses the Relay Discovery Additional Information message as defined in clause 5.8.3 with the following additional information:

-	Hop-Count: the value is set to 1.

The Destination Layer-2 ID to send the Relay Discovery Additional Information message is selected based on the configuration as described in clause 5.1.4.1.

9.	The 5G ProSe Intermediate UE-to-Network Relay forwards the Relay Discovery Additional Information message. The 5G ProSe Intermediate UE-to-Network Relay updates the Hop-Count as below and may include additional information, Announcer Info and NCGI of 5G ProSe Intermediate UE-to-Network Relay if its NCGI is available:

-	Hop-Count: the value of the received Relay Discovery Additonal Information message is incremented by 1.

-	Announcer Info of 5G ProSe Intermediate UE-to-Network Relay: identity information (i.e. User Info ID) of the 5G ProSe Intermediate UE-to-Network Relay transmitting this message.

-	NCGI of 5G ProSe Intermediate UE-to-Network Relay: indicates the NCGI of the serving cell of the 5G ProSe Intermediate UE-to-Network Relay.

10.	The 5G ProSe Intermediate UE-to-Network Relay forwards the Relay Discovery Additional Information message, with the Hop-Count incremented. The 5G ProSe Intermediate UE-to-Network Relay shall not forward the Relay Discovery Additional Information message if the Hop-Count is bigger than the configured maximum number of hops.

11.	The 5G ProSe UE-to-Network Relay detects new or updated additional parameters.

12-14.	Same as steps 8-10.

If the 5G ProSe Intermediate UE-to-Network Relay connected to 5G ProSe UE-to-Network Relay detects new or updated additional parameters (e.g. due to change of its serving cell, moving into network coverage), it may send a Relay Discovery Additional Information message including the new or updated additional parameters. In this case, the 5G ProSe Intermediate UE-to-Network Relay acts as the 5G ProSe UE-to-Network Relay in figure 6.3.2.5.4-1, i.e. to send Relay Discovery Additional Information message. The 5G ProSe Intermediate UE-to-Network Relay also keeps a record of the additional parameters from the 5G ProSe UE-to-Network Relay.

NOTE:	The NCGI of the 5G ProSe Intermediate UE-to-Network Relay connected to 5G ProSe UE-to-Network Relay can be used as supplementary by the 5G ProSe Remote UE and not provided to the network side from the 5G ProSe Remote UE.

# >>>>END OF CHANGES<<<<