3GPP TSG-SA WG2 Meeting #170	S2-2506914

# Goteborg, Sweden, 25-29 August 2025	(was S2-2504591)

Source:	Qualcomm Incorporated

Title:	KI#2: Solution for IMS signaling performance optimization to support voice over GEO

Agenda item:	20.1.1

Document for:	Discussion/Approval

1. Introduction

SA#107 approved the study for the support of IMS voice over GEO via NB-IoT NTN connecting to EPC.

Under WT-1, SA2 agreed to study the support of IMS voice over GEO via NB-IoT NTN connecting to EPC. Specifically, WT-1.2 studies the IMS enhancement to utilize NB IoT as IP-CAN and potential optimization to comply with the relevant SA1 requirements. This contribution proposes a solution for optimization of IMS signalling performance on WT-1.2.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-19.

* * * First Change * * * *

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | #1 | #2 | #3 | #4 |
| #1 | X | X |  |  |
| #2 | X |  |  |  |
| #3 | X |  |  |  |
| #4 | X | X | X | X |
| #5 | X |  |  |  |
| #6 | X |  |  |  |
| #7 | X | X |  |  |
| #8 | X |  |  |  |
| #9 | X |  |  |  |
| #10 | X |  |  |  |
| #11 | X |  |  |  |
| #12 |  | X |  |  |
| X |  | X |  |  |

* * * Next Change * * * *

## 6.x	Solution X: IMS signalling performance optimization based on SIP message compression or I1 interface

### 6.X.1	High level principles

This solution addresses Key Issue 2 and supports IMS signalling performance optimization.

This solution is based on the usage of SigComp or I1 interface for IMS signaling message delivery between UE and IMS network.

If I1 interface is used in an IMS call flow, the procedures specified in TS 24.294 [z] are reused.

If SIP/SigComp is used in an IMS call flow, the following principles are proposed:

-	An IMS AS is applied as B2BUA to support the simplified IMS signalling for a UE connected via GEO and signalling exchange with the peer UE via legacy signalling procedures.

-	The IMS AS is applied to control proactive media transcoding for the potential low data rate audio codec for the UE connected via GEO.

-	A media function (e.g., MF, MRF) is applied to support transcoding of low data rate audio codec to other existing audio codec in an IMS session.

-	The serving P-CSCF of a UE connected via GEO may trigger QoS authorization and resource reservation when receives a SIP INVITE to/from the UE to reduce the signalling exchange.

-	UE and P-CSCF perform SIP message compression/decompression.

### 6.X.2	Description

#### 6.X.2.1	Requirements

To meet SA1 performance requirements for IMS voice call setup time using GEO satellite, the IMS signaling messages exchanged between UE and IMS network via GEO satellite need to be optimized.

The IMS signaling flow always requires high reliability. It’s a big challenge to support IMS signaling exchange between UE and IMS network in the scenarios of voice services via GEO satellite. The transmission data rate of NB-IoT via GEO satellite is very restricted, e.g., 1-2 kbps as described in TR 22.887 [y], which causes long transmission delay for the IMS signaling message delivery. The propagation delay via satellite is associated with these orbit ranges. The UE to ground max propagation delay is estimated to be 272ms, as specified in TS 22.261 [8]. In addition, the re-transmission of IMS singling via GEO satellite is more frequent than terrestrial access due to the radio resource condition,

Considering the above, the message size and number of exchanged signaling messages between UE and IMS network need to be reduced to meet SA1 requirements on IMS call setup time.

Potential new Ultra Low Bitrate Codecs (ULBC) for the voice services via GEO satellite are under study in SA4 to support the low data rate via GEO over NB-IoT, e.g., 1-2 kbps as described in TR 22.887 [y]. It is assumed that the potential new audio codec is supported by the legacy IMS entities and UEs. Media transcoding is always required for the interworking with legacy UEs.

#### 6.X.2.2	Uncompressed SIP message

Following message is an example of minimum required set of Initial SIP INVITE message with SDP for audio media. The size of the SIP INVITE message is counted to be 491 Bytes.

INVITE sip:bob@biloxi.com SIP/2.0

Via: SIP/2.0/UDP pc33.atlanta.com;branch=z9hG4bK776asdhds

Max-Forwards: 70

To: Bob <sip:bob@biloxi.com>

From: Alice <sip:alice@atlanta.com>;tag=1928301774

Call-ID: a84b4c76e66710@pc33.atlanta.com

CSeq: 314159 INVITE

Contact: <sip:alice@pc33.atlanta.com>

Content-Type: application/sdp

Content-Length: 142

v=0

o=mhandley 2890844526 2890842807 IN IP4 126.16.64.4

s=

c=IN IP4 224.2.17.12/127

t=0 0

m=audio 49170 RTP/AVP 99

a=rtpmap:99 ULBC-X

b=AS:2

ptime=40

#### 6.X.2.3	SIP Signaling Compression

SigComp (Signaling Compression) is a protocol designed to compress signaling messages, such as those used in SIP (Session Initiation Protocol), to reduce bandwidth usage and improve performance, especially over networks with limited capacity

As an example, the SIP INVITE message presented in clause x.y.z.2 can be compressed to 285 Bytes with SigComp (processed by toolkit of Doubangou + zlib 1.2.3).

#### 6.X.2.4	IMS signaling delivery via I1 interface

The I1 interface as specified in TS 24.294 [z], is used for communication between the IMS Centralized Services (ICS) User Equipment (UE) and the Service Centralization and Continuity (SCC) Application Server (AS).

The I1 protocol is used for session control procedures, including session origination and management. The I1 protocol is transport-independent, meaning it can operate over any transport-layer protocol.

Follow table illustrates the content of I1 INVITE message initiated from UE. The size of the I1 INVITE message mapping to the SIP INVITE message in clause x.y.z.2 is estimated 18 Bytes.

| Information element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- |
| Protocol Information | Protocol Information | M | V | 1 |
|  | 7.2.2.1.2 |  |  |  |
| Message Type | Request Message - INVITE | M | V | 2 |
|  | 7.2.2.2.1.2 |  |  |  |
| Call ID | Call-Id | M | V | 2 |
|  | 7.2.2.1.4 |  |  |  |
| Message Sequence Number | Sequence-Id | M | V | 1 |
|  | 7.2.2.1.5 |  |  |  |
| Timestamp | Timestamp | O | TLV | 1 |
|  | 7.3.2.8 |  |  |  |
| To-id | To-id | M | TLV | 3-X |
|  | 7.3.2.3 |  |  | Note 1 |
| From-id | From-id | M | TLV | 3-X |
|  | 7.3.2.4 |  |  | Note 1 |
| Accept Contact | Accept Contact | O | TLV | 5 |
|  | 7.3.2.5 |  |  |  |
| ERAccept Contact | ERAccept Contact | O | TLV | 3-X |
|  | 7.3.2.6 |  |  | Note 1 |
| Reject Contact | Reject Contact | O | TLV | 5 |
|  | 7.3.2.7 |  |  |  |
| Note 1 – The length of this information element in combination with the other information elements cannot exceed the length of the transport protocol information element to transport the I1 message. |  |  |  |  |

Table 6.X.2.4-1: I1 INVITE message specified in TS 24.294 [z]

#### 6.X.2.5	Comparison of uncompressed SIP, SigComp, I1 interface messages

Follow table provides the estimated message size before SIP compression, after signaling compression and I1 interface messages. The selected SIP messages are abstracted from the SIP call flow described in clause x.y.z.5.

| Message | Message Size | Message Size After Compression | I1 Interface Message |
| --- | --- | --- | --- |
| INVITE | 491B | 285B (SigComp: Doubango + zlib1.2.3) | 18B |
| 100 Trying F3 | 238B | 232B (LZ77) | 6B |
| 180 Ringing F8 | 280B | 256B (LZ77) | 6B |
| 200 OK F11 | 491B | 285B (SigComp: Doubango + zlib1.2.3) | 6B |
| ACK | 254B | 233B (LZ77) | 6B |

Table 6.X.2.5-1: Message size comparison

### 6.X.3	Procedures

#### 6.X.3.1	General

To reduce the IMS signaling messages exchanged between the UE and IMS network via GEO satellite access, the following SIP call setup flow specified in RFC 3261 [xx] can be considered as baseline for the reduction of IMS signaling messages exchanged via GEO satellite access.

|<-----NTN----->|<--------------TN--------------->|

atlanta.com  . . . biloxi.com

.      proxy              proxy     .

.                                       .

Alice's  . . . . . . . . . . . . . . . . . . . .  Bob's

softphone                                        SIP Phone

|                |                |                |

|    INVITE F1   |                |                |

|--------------->|    INVITE F2   |                |

|  100 Trying F3 |--------------->|    INVITE F4   |

|<---------------|  100 Trying F5 |--------------->|

|                |<-------------- | 180 Ringing F6 |

|                | 180 Ringing F7 |<---------------|

| 180 Ringing F8 |<---------------|     200 OK F9  |

|<---------------|    200 OK F10  |<---------------|

|    200 OK F11  |<---------------|                |

|<---------------|                |                |

|                       ACK F12                    |

|------------------------------------------------->|

|                   Media Session                  |

|<================================================>|

|                       BYE F13                    |

|<-------------------------------------------------|

|                     200 OK F14                   |

|------------------------------------------------->|

|                                                  |

Figure 6.X.3.1: Example of SIP call flow

Editor's note:	Whether supporting preconditions specified in RFC 3312 [yy] for resource reservation during SIP session establishment is FFS.

#### 6.X.3.2	Satellite UE originated session

Figure 6.3.2-1: Satellite UE originated session procedures

1. 	UE (A) sends an initial INVITE towards User (B). Based on the radio access type (NB-IoT via GEO satellite access), UE(A) decides to use low data-rate audio codec (codec-x) for the session and applies simplified SIP signalling, e.g., SIP precondition not needed or marked as met in the UE side.

2.	The P-CSCF replies 100 Try Response to UE(A).

3.	The P-CSCF detects that the UE is connected via NB-IoT (GEO). To reduce the call setup time, the P-CSCF triggers QoS authorization and resource reservation to EPS. An additional radio bearer may be established for the voice flow.

4.	The P-CSCF forwards the INVITE to the S-CSCF. The S-CSCF sends the INVITE to the IMS AS which serves as an B2BUA of the IMS session.

5.	The AS determines to trigger proactive transcoding for UE(A). The AS triggers the related media function (e.g., MF/MRF) to reserve media resource for the transcoding from codec-X to the other audio codec for the IMS session.

6.	The AS modifies the SIP INVITE with additional list of default audio codec (e.g., AMR).

7.	The AS and terminating network/UE(b) exchange SDP Off/Response for media codec negotiation.

8.	Codec-A is selected and confirmed for the media flow between MF/MRF and UE(B).

9. 	The AS updates the media resource in the MF/MRF for transcoding between codec-X and codec-A. The media endpoint to UE(B) is updated as well.

10.	 UE(B) starts ringing.

11.	When the user answers, UE(B) sends a SIP 200-OK final response to UE(A). UE starts the media flow(s) for this session.

12.	UE(B) send ACK to UE(A)

13.	The media flow between UE(A) and MF/MRF is in codec-X. The MF/MRF performs media transcoding.

14.	The media flow between MF/MRF and UE(B) is in codec-A.

#### 6.X.3.3	Satellite UE terminated session

Figure 6.3.3-1: Satellite UE terminated session procedures

1.	UE (B) sends an initial INVITE towards UE(A).  The INVITE is sent to the S-CSCF in the serving IMS of UE(A) and forwarded to the serving AS.

2-4.	The AS is aware that UE(A) is connected to IMS via NB-IoT (GEO) based on the UE(A)’s IMS registration information. The AS serves as a B2BUA for UE(A) to negotiate the media codec with UE(B). Codec-A is finally selected and confirmed between the AS and UE(B).

5.	 The AS determines to trigger proactive transcoding for UE(A) and initiates the media resource reservation in the MF/MRF for media transcoding between UE(A) and UE(B).

6.	The AS modifies the INVITE message and includes a list of audio codecs that can be transmitted via NB-IoT (GEO). Codec-X and Codec-Y are listed hereby as examples. The INVITE is sent to the S-CSCF and forwarded to the P-CSCF.

7.	The P-CSCF is aware that UE(A) is connected to IMS via NB-IoT (GEO) based on the UE(A)’s IMS registration information. And triggers QoS authorization and resource reservation to EPS. An additional radio bearer may be established for the voice flow.

8.	After the radio resource is reserved for UE(A), the P-CSCF sends the INVITE to UE(A) with codec-X and codec-Y.

9.	UE(A) starts ringing and confirms selection of codec-X.

10.	The ringing message is sent to the AS via the S-CSCF.

11.	The AS receives the confirmation of code-X and updates the media resource reservation in the MF/MRF. The AS instructs the MF/MRF to perform transcoding between codec-X and codec-A for the IMS session.

12.	The AS modifies the 180 Ringing message with confirmation of codec-A and sends to UE(B).

13. When the user answers, UE(A) sends a SIP 200-OK final response to UE(B). UE starts the media flow(s) for this session.

14.	UE(B) sends ACK to UE(A).

15.	The media flow between UE(A) and MF/MRF is transmitted in codec-X. The MF/MRF performs media transcoding.

16.	The media flow between MF/MRF and UE(B) is transmitted in codec-A.

### 6.X.4	Impacts to Services, Entities and Interfaces

UE:

-	Supports SIP message compression/decompression or I1 interface protocols.

-	Supports transmission of audio stream via Ultra Low Bitrate Codec.

-	Supports simplified SIP signalling procedures when connected to IMS via NB-IoT (GEO).

IMS AS:

-	Supports acting as B2BUA for the simplified SIP signalling exchange with UE connected to IMS via NB-IoT (GEO).

-	Supports the signalling exchange with the peer network/UE via legacy IMS signalling procedures.

-	Supports triggering of proactive media transcoding for the session from/to a UE connected to IMS via NB-IoT (GEO).

-	Supports I1 interface protocol in ICS AS (if I1 interface is applied).

MF/MRF:

-	Supports media transcoding for the low data rate audio codec from/to a UE connected to IMS via NB-IoT (GEO).

P-CSCF:

- Supports early triggering QoS authorization and resource reservation to EPS upon receiving INVITE from/to a UE connected to IMS via NB-IoT (GEO).

- Supports SIP message compression/decompression.

* * * Next Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G system, Stage 2".

[4]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System".

[5]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[6]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[7]	3GPP TS 23.167: "IP Multimedia Subsystem (IMS) emergency sessions".

[8] 	3GPP TS 22.261: "Service requirements for the 5G system".

[9] 	3GPP TS 22.101: "Service aspects; Service principles".

[y]	3GPP TR 22.887: “Study on satellite access - Phase 4”.

[z]	3GPP TS 24.294: “IP Multimedia Subsystem (IMS) Centralized Services (ICS) protocol via I1 interface”.

[xx]	IETF RFC 3261: "SIP: Session Initiation Protocol".

[yy]	IETF RFC 3312: "Integration of Resource Management and Session Initiation Protocol (SIP)”.

* * * End of Changes * * * *