3GPP TSG-SA WG2#171	S2-2508294

Wuhan, China, 13 – 17 October, 2025	(revison of S2-2506291)

Source:	OPPO

Title:	Update to solution #18 on sensing data and the associated information transport

Document for:	Approval

Agenda Item: 	20.2.1

Work Item / Release:	FS_Sensing_ARC/Rel-20

Abstract of the contribution: This paper proposes a solution update to solution #18 on sensing data and the associated information transport for the FS_Sensing_ARC TR 23.700-14.

1	Discussion

This paper proposes a solution update to solution #18 on sensing data and the associated information transport. “UE as sensing entity” approach is removed according to the agreement in SA plenary meeting.

Sensing data could be collected and stored for multiple sensing services if the sensing requirements can be met. For example, a Sensing Entity might satisfy multiple separate sensing requests for mobility tracking, interference mapping, and QoS validation with a single dataset. So it is proposed to allow Sensing Function collects sensing data from the Data Collection Entity which can collect data from data sources (e.g. OAM system, Sensing Entity) or work as sensing data storage function, in order to reuse the sensing data more efficiently.

2	Proposal

It is proposed to include the following changes in TR 23.700-14 V1.0.0.

* * * * Start of Changes * * * *

## 6.18	Solution #18: Sensing data and associated information transport via data connection

### 6.18.0	High-level solution Principles

The solution is based on the following general principles to support sensing service:

1)	The sensing data and associated information are transmitted between the Sensing Entity (i.e. gNB) and the Sensing Function via data connection.

2)	The data connection establishment procedure is carried out on control plane signalling interaction.

3)	The Sensing Function provides its connection information (e.g. IP address) for data connection and binding information (e.g. Sensing Entity ID, connection ID or specific binding ID) to the Sensing Entity (i.e. gNB).

4)	The Sensing Entity sends binding information via established data connection to sensing function for connection binding.

5)	gNB is configured by the Sensing Function for sensing data and associated information collection over the data connection

6)	For any matching sensing service request, sensing data and associated information can be directly collected from the Data Collection Entity instead of Sensing Entity (i.e. gNB).

### 6.18.1	Description

This solution is to address KI#4 on sensing data and the associated information collection and transport.

### 6.18.2	Procedures

### 6.18.2.1	Data connection Establishment between Sensing Function and Sensing Entity

Figure 6.18.2.1-1: Procedure to establish data connection between Sensing Function and Sensing Entity

1.	The Sensing Function determines to establish data connection for sensing data and associated information transport.

2a1.	[Conditional] The Sensing Function sends the connection information to AMF in a N2 container to indicate the selected sensing entity (i.e. NG-RAN node) to utilize data connection for sensing. The connection information includes the address or end point information of the Sensing Function. The binding information (e.g. Sensing Entity ID, connection ID or specific binding ID) associated with the data connection is also included in the connection information.

2a2.	[Conditional] AMF sends the connection information to the Sensing Entity via N2 message.

2b.	[Conditional] If there is direct interface between the Sensing Function and Sensing Entity, the Sensing Function can send the connection information to the Sensing Entity directly.

3.	After the Sensing Entity receives the connection information, the Sensing Entity establish data connection with the Sensing Function according to the address of the Sensing Function. The sensing entity establishes direct data connection with the Sensing Function. After the data connection has been established successfully, the Sensing Entity sends the binding information received in step 3 to Sensing Function via the established data connection to enable the Sensing Function to perform the correlation of the Sensing Entity or Connection ID with the data connection.

Editor's note:	Details for Sensing Entity as a gNB to establish the data connection is FFS.

4a1.	The Sensing Entity sends an acknowledgement to Sensing Function to indicate a success of data connection establishment for sensing service.

4a2.	[Conditional] AMF sends the acknowledgement received in step 4a1 to the Sensing Function.

4b.	[Conditional] If there is direct interface between the Sensing Function and Sensing Entity, the Sensing Entity can send the acknowledgement to the Sensing Function directly.

5.	If Sensing Function or Sensing Entity determines to utilize the data connection for sensing data and the associated information collection and transfer and the data connection is established, messages are transferred between Sensing Entity and Sensing Function over the established data connection.

6.18.2.2	Data collection from sensing entity or data collection entity

Figure 6.18.2.2-1: Procedure to collect data from sensing entity or data collection entity

- Sensing data is collected by the Data Collection Entity from the data source(s) (e.g. OAM system, Sensing Entity). The Data Collection Entity can also work as sensing data storage function. The collected sensing data may come from other service requests (e.g. other sensing services) and collected/stored by the Data Collection Entity.

- Sensing Function receives a sensing service request and successfully selects Sensing Entities.

- Sensing Function sends a sensing data collection message to the Data Collection Entity to request/subscribe sensing data. The message may include the following information:

- data collection session ID

- Sensing Entity ID(s) obtained at Step 2

- object sensing service area obtained from the sensing service request

- Time period obtained from the sensing service request

- QoS requirement obtained from the sensing service request

- Time interval obtained from the sensing service request if it a periodic request

- Event(s) obtained from the sensing service request if it is event-based request

- Sensing result type

- Data Collection Entity checks if any session matches with the sensing data service request. If any matched session is available, the Data Collection Entity responds to Sensing Function with the following information:

- data collection session ID

- data collection entity Address

- Sensing data format

- Sensing data volume

- data collection entity capability to support data transport

If Sensing Function receives information indicating available matched session, e.g. data collection session ID, data collection entity Address, Sensing data format, Sensing data volume, Sensing data, Sensing Function collects data from the Data Collection Entity for the sensing service request; otherwise Sensing Function collects data from Sensing Entity, i.e. step 5-7 are skipped.

- Sensing Function determines whether the sensing data is transferred over CP or data connection based on sensing data format, sensing data volume, data collection entity capability received from step 4, service type received from the sensing service request and local configuration.

- If Sensing Function decides to transfer data over data connection, it checks if data connection is already established with Data Collection Entity, if yes, then this connection is used for sensing data collection; otherwise, Sensing Function establishes a data connection to the Data Collection Entity based on its address obtained from step 4 or preconfigured address.

- Sensing data collection is conducted between Data Collection Entity and Sensing Function. For any further matching sensing service request, Sensing Function collects data over this connection.

- Sensing Function can determine to collect data from Sensing Entity directly based on the sensing request or information received from Data Collection Enity indicating no available matched session.

- If Sensing Function decides to transfer data over data connection, it checks if data connection is already established with the sensing entity, if yes, then this connection is used for sensing data collection; otherwise, Sensing Function can establish data collection via the procedure defined in clause 6.18.2.1.

- Sensing Function configures the Sensing Entity for collection of sensing data and the associated information.

- If there’s no on-going sensing procedure matching the sensing request, the Sensing Entity performs sensing procedure to collect sensing data.

- Sensing data collection is conducted between the Sensing Entity and Sensing Function. For any further matching sensing data collection request, Sensing Function collects data over this connection.

### 6.18.3	Impacts on services, entities and interfaces

The solution has impacts on the following entities:

AMF:

-	Support to send the connection information received from Sensing Function to Sensing Entity via N2 message.

Sensing Function:

-	Support to provide connection information to Sensing Entity directly or via AMF in N2 container.

-	Support to receive binding information from the Sensing Entity via data connection for connection binding.

-	Support to transfer sensing data and the associated information with the Sensing Entity via data connection.

Sensing Entity:

-	Support to establish data connection with the Sensing Function according to the connection information received from Sensing Function.

-	Support to send binding information to the Sensing Function via established data connection for connection binding.

-	Support to transfer sensing data and the associated information with the Sensing Function via data connection.

Data Collection Entity:

-	Support to collect data from data sources (e.g. OAM system, Sensing Entity) or work as sensing data storage function.

-	Support to establish data connection with the Sensing Function.

-	Support to transfer sensing data and the associated information with the Sensing Function via data connection.

* * * * End of Changes * * * *