SA WG2 Meeting #171	S2-2508719

13 – 17 Oct, 2025, Wuhan, China

Source:	Samsung

Title:	Solution for KI#2 to Support Asset Tracking

Document for:	Approval

Agenda Item:	20.5.1

Work Item / Release:	FS_AmbientIoT_Ph2_ARC/Rel-20

Abstract of the contribution: This contribution proposes a solution to support asset tracking regarding Key Issue #2.

1. Discussion

Key Issue 2 is like the following:

This key issue will study the system architecture to support DO-A capable Ambient IoT Devices in Topology 1 and Topology 2.

The following aspects will be studied:

-	How the AIoT Device informs the network of its presence autonomously (e.g., an AIoT Device initiated registration-like procedure) and what are the triggers for the DO-A capable device to inform the network of its presence.

-	Whether and how to consider power consumption of DO-A Capable AIoT Devices.

-	How an AIoT Device sends data to the AIOTF autonomously.

-	Support for routing the data received by AIOTF from an AIoT Device to an AF.

-	Whether and how to enhance the Inventory and Command procedures defined in TS 23.369 [3] to support DO-A capable AIoT Devices.

-	Naiotf, Namf and Nnef interface enhancements to support DO-A capable AIoT Device.

NOTE 1:	The conclusions from Key Issue #1 are the basis for supporting DO-A capable AIoT Devices in topology 2 in this key issue.

NOTE 2:	Coordination with RAN WGs is required.

Ambient IoT Service Use cases in TS 22.269 includes Asset Tracking. With asset tracking, the main purpose is to determine the location of goods. Ambient IoT devices attached to these goods report an identifier associated with the good. This can then be combined with location information.

2. Proposal

It is proposed to agree the following changes to 3GPP TS 23.700-30.

* * * Start of Change (all new text) * * * *

6.X	Solution #X: Support of Asset Tracking

6.X.1	Description

This solution addresses KI#2. The solution includes Service Reqeust from AF to 5GS, Parameter Configuration for the Service, Data Transfer for the Service.

The solution is based on the following assumptions:

-	The Asset Tracking Service utilizes Periodic DO-A Asset Tracking Service from the Device including AF, AIOTF, and AIoT Device. It uses Inventory Request type with a Period.

-	The Asset Tracking Service utilizes Periodic DO-A Data Transfer (Device, AIoT Reader, AIOTF)

-	Asset Tracking Service Paramters (e.g., Tracking Period, StartTime, Event-Triggered) can be pre-configured on AIoT Devices or configured by AF.

-	AIoT Reader maintains (Device ID and Correlation ID) context for every periodic behavior or event-triggered behavor.

6.X.2	Procedures

The procedure is shown in Figure 6.X.2-1. It includes Service Reqeust from AF to 5GS, Parameter Configuration for the Service, Data Transfer for Asset Tracking Service.

Figure 6.X.2-1: Asset Tracking Procedure

1.	AF sends AIoT Inventory Request to NEF. It includes Device(s) Info, Loc(ext.) which is 3GPP-external location information, AF Transaction ID, Period, and optimially Tracking Parameters which include [Tracking Period Offset, period], [StartTime Offset, Now/time/Off], [Event-Triggered Offset, On/Off].

2.	NEF sends AIoT Inventory Request to AIOTF. It includes Device(s) Info, Loc(Int.) which is internal location info like RAN ID or Cell ID mapped to Loc(ext), Transaction ID which is mapped to AF Transaction ID, Period, and optionally Tracking Parameters which include [Tracking Period Offset, period], [StartTime Offset, Now/time/Off], [Event-Triggered Offset, On/Off].

3.	AIOTF generates Inventory Request based on Device(s) Information included in step 1 in cooperation with ADM.

4.	AIOTF sends AIoT Inventory Response to NEF. It includes Status (Success/Failure) and Transaction ID.

5.	NEF sends AIoT Inventory Response to AF. It includes Status (Success/Failure) and AF Transaction ID.

6.	Steps 7 to 10 of the Procedure for Inventory in clause 6.2.2 of TS 23.369.

7.	AIOTF retrieves Device Profile Data Including DO-A Capability. If the device does not support DO-A Capability, step  15 to step 19 or step 20 to setp 24 are skipped.

8a~12a, 8b~12b, 8c~12c.	Parameter set with Write Command can be done before steps 8~12.

8.	AIOTF sends AIoT Inventory Request to AIoT Reader. This includes Device ID (temporary), (Reader ID), Correlation ID, Period, and Tracking Parameters ([Tracking Period Offset, period,] [StartTime Offset, Now/time/Off] [Event-Triggered Offset, On/Off]). AIoT Reader stores Correlation ID with AIoT Device ID as context data.

9.	AIoT Reader sends AIoT Inventory Request to AIoT Device. This includes Device ID (temporary), (Reader ID), Correlation ID, Period, and Tracking Parameters ([Tracking Period Offset, period], [StartTime Offset, Now/time/Off], [Event-Triggered Offset, On/Off]).

10.	AIoT Device set parameters, including Tracking Period (a specific Period), StartTime (Now, a specific time, or Off) and Event-Triggered (On or Off). If StartTime is set to Now, the device sends Inventory Report immediately and every Period. If StartTime is set to a specific time, then the device sends Inventory Report at that time and every Period thereafter. If StartTime is set to Off, the devce sends Inventory Report once, then stops periodic reporting. If Event-Triggered is set to On, the device sends Inventory Report when it detects a change in Reader ID. If Event-Triggered is set to Off, the device stops sending  Inventory Report when the device detects a change in Reader ID. Depending on the size of the report, the radio resource allocation procedure can be different.

11.	AIoT Device sends AIoT Inventory Response to AIoT Reader. This includes Device ID (temporary), Reader ID.

12.	AIoT Reader sends AIoT Inventory Response to AIOTF. This includes Device ID (temporary), Reader ID, and Correlation ID. AIoT Reader retrieves Correlation ID with AIoT Device ID from the stored context.

13.	AIOTF sends AIoT Inventory Notify to NEF. This includes Device ID (permanent), Reader ID, and Transaction ID. AIOTF retrieves Transaction ID with Correlation ID from the stored context.

14.	NEF sends AIoT Inventory Notify to AF. This includes Device ID (permanent), Reader ID, and AF Transaction ID. NEF retrieves AF Transaction ID with Transaction ID from the stored context.

15.	Every Period, the device sends Inventory Report. Depending on the size of the report, the radio resource allocation procedure may vary.

16.	AIoT Device sends AIoT Inventory Response to AIoT Reader. This includes Device ID (temporary) and Reader ID.

17.	AIoT Reader sends AIoT Inventory Response to AIOTF. This includes Device ID (temporary), Reader ID, and Correlation ID. AIoT Reader retrieves Correlation ID with AIoT Device ID from the stored context.

18.	AIOTF sends AIoT Inventory Notify to NEF. This includes Device ID (permanent), Reader ID, and Transaction ID. AIOTF retrieves Transaction ID with Correlation ID from the stored context.

19.	NEF sends AIoT Inventory Notify to AF. This includes Device ID (permanent), Reader ID, and AF Transaction ID. NEF retrieves AF Transaction ID with Transaction ID from the stored context.

20.	On Event (When the device detects the Reader ID has changed), the device sends Inventory Report. Depending on the size of the report, the radio resource allocation procedure may vary.

21.	AIoT Device sends AIoT Inventory Response to AIoT Reader. This includes Device ID (temporary) and Reader ID.

22.	AIoT Reader sends AIoT Inventory Response to AIOTF. This includes Device ID (temporary), Reader ID, and Correlation ID. AIoT Reader retrieves Correlation ID with AIoT Device ID from the stored context.

23.	AIOTF sends AIoT Inventory Notify to NEF. This includes Device ID (permanent), Reader ID, and Transaction ID. AIOTF retrieves Transaction ID with Correlation ID from the stored context.

24.	NEF sends AIoT Inventory Notify to AF. This includes Device ID (permanent), Reader ID, and AF Transaction ID. NEF retrieves AF Transaction ID with Transaction ID from the stored context.

6.X.3	Impacts on services, entities and interfaces

Impacts on existing entities:

AIOTF:

-	Receives Inventory Request with a Period from AF through the NEF.

-	Performs Inventory procedure and then then sends Inventory request with Period and Tracking Parameters.

-	Performs Inventory procedure and then then sends Write Command request with Tracking Parameters.

-	Retrieves Device Profile Data Including DO-A Capability. If the device does not support DO-A Capability, AIOTF skips DO-A related steps.

-	Receives AIoT Inventory Report from Device(s) through AIoT Reader(s) and deliveres it to the the AF

-	Sends AIoT Inventory Notify with Reaer ID as 3GPP-Internal Location information, which is converted into 3GPP-External Location information at NEF, to AF.

ADM:

-	Supports the AIoT Device profile including DO-A Capability.

AIoT Reader (RAN Reader / UE Reader):

-	Supports the AIoT TempID handling including Device ID and Corrleation ID mapping, and maintains the mapping context.

-	Supports DO-A procedure for AIoT Inventory Report from AIoT Device.

AIoT device:

-	Supports DO-A procedure for AIoT Inventory Report from AIoT Device.

-	Sensing AIoT Inventory Report to AIoT Reader periodically via the DO-A procedure.

-	Sets Tracking parameters. It sets Sensing Period to Period (a specific period). It sets StartTime to Now, a specific time, or Off. If StartTime is set to Now, the device sends Inventory Report immediately and every Period. If StartTime is set to a specific time, the device sends Inventory Report at that time and every Period thereafter. If StartTime is to Off, the device sends Inventory Report once, and turns off the periodic reporting. It sets Event-Triggered to On or Off. If Event-Triggered is set to On, the device sends Inventory Report when it detects a change in Reader ID. If Event-Triggered is set to Off, the device stops sending  Inventory Report when the device detects a change in Reader ID.

-	Performs different the radio resource allocation procedure depending on the size of the report.

* * * End of Change * * * *